3GPP TSG-RAN WG4 Meeting # 108bis		R4-2316581

# Xiamen, China, October 9 – October 13, 2023

Agenda item:	5.14.4

Source:	Apple

Title:	On remaining issues for spectrum less than 5MHz

Document for:	Discussion

# Introduction

In last RAN4 #108 meeting, the WF of spectrum less than 5MHz was agreed in [1]. In this contribution, we continue discussion about the RRM impacts for the spectrum less than 5MHz, mainly focusing on the remaining issues from last meeting.

# Radio Link Monitoring

One of the remaining issues in last meeting is:

Issue 1-17: Use one table for 3MHz in-sync transmission parameters? Attribute; Value for BLER Configuration #0. 3MHz (12 PRBs); 3MHz (15 PRBs). DCI format; 1-0. Number of control OFDM symbols; [2]; [3]. Aggregation level (CCE); [2]; [4]. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; [0]dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; [0]dB. Bandwidth (PRBs); 12; 15. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. Note: it is FFS if Aggregation level (CCE) 2 can be used.

RAN1 agreed that,

- For 3MHz channel bandwidth in all bands (max channel utilization 15 PRBs as already agreed in RAN1/RAN4):

  - PBCH transmission bandwidth is 12 PRBs

  - For CORESET#0 transmission bandwidth, both 12 PRBs and 15 PRBs are supported

| CBW | AL (CCE num) | Symbol num | PDCCH RBs on each symbol |
| --- | --- | --- | --- |
| 3MHz (PDCCH on 12PRBs) | 8 | 2 | 24 PRBs |
|  | 8 | 3 | 16 PRBs |
|  | 4 | 2 | 12PRBs |
|  | 4 | 3 | 8PRBs |
| 5MHz (PDCCH on 24PRBs) | 8 | 2 | 24 PRBs |

RAN4 agreed that,

PDCCH transmission parameters for RLM and BFD requirement:

  - Aggregation level (CCE) for RLM OOS and BFD

    - AL = 4 for 12 PRBs hypothetical PDCCH transmission BW

    - AL = 8 for 15 PRBs hypothetical PDCCH transmission BW

      - Note: AL 8 will be considered if it is supported by RAN1 design

  - Aggregation level (CCE) for RLM IS

    - AL = [2] for 12 PRBs hypothetical PDCCH transmission BW

    - AL = [4] for 15 PRBs hypothetical PDCCH transmission BW

  - Number of control OFDM symbols

    - 2 OFDM symbols for 12 PRBs hypothetical PDCCH transmission BW

    - 3 OFDM symbols for 15 PRBs hypothetical PDCCH transmission BW

The DCI format 1-0 could be used for:

- scheduling of PDSCH in one cell

- schedule RAR (Msg2) in RACH Procedure

- schedule Contention Resolution (Msg4) in RACH Procedure

- schedule SIB1 and other SIB messages

- schedule Paging messages

However, in TS38.213 section 10.1, depending on different use cases of DCI format 1-0, it’s specified that,

If we use the AL of Type0-PDCCH as a baseline for RLM IS evaluation design, then the AL of RLM evaluation can be defined no less than 4. However, Type3-PDCCH CSS and USS can also be used for DCI 1-0, which can support AL as low as AL=1. RLM IS is an evaluation of “how good the channel is when UE is able to recover the link from OOS”, so we think coverage might not be a factor to limit the usage of AL=2 for IS evaluation (AL=4 is assumed in OOS for coverage). Moreover, it’s also important to guarantee sufficient SINR gap between OOS and IS to avoid mis-judging OOS/IS at UE. Thus, we still think AL=2 shall be assumed for 12 PRBs hypothetical PDCCH transmission BW for RLM IS evaluation.

Proposal 1:  keep AL = 2 for 12 PRBs hypothetical PDCCH transmission BW for RLM IS evaluation.

# Side Conditions

The agreements in last meeting were:

Issue 1-34: Use following side condition for NR target cell detection for RRC connection re-establishment and RRC connection release with re-direction? - FFS - Intra-frequency NR target cell - Option 1: target cell Es/Iot≥-6 dB - Option 2: target cell Es/Iot≥-4 dB - Inter-frequency NR target cell - Option 1: target cell Es/Iot≥-4 dB - Option 2: Other

The side condition for RRC connection re-establishment and release in the existing spec are:

UE Re-establishment delay requirement

RRC Connection Release with Redirection

To summarize, in current spec, the SINR side condition for intra-frequency re-establishment requirement is: -6dB, while for inter-frequency re-establishment requirement is: -4dB. The SINR side condition for RRC Connection Release requirement is: -4dB. We think the same side condition shall be reused for less than 5MHz case.

Proposal 2: Reuse existing side condition Es/Iot for target NR cell detection for RRC connection re-establishment and RRC connection release with re-direction, i.e.,

-	Es/Iot≥-4 dB for target NR cell detection for inter-frequency RRC connection re-establishment and RRC connection release with re-direction

-	Es/Iot≥-6 dB for target NR cell detection for intra-frequency RRC connection re-establishment

# Conclusion

In this contribution, we discuss the RRM impacts for the spectrum less than 5MHz.

Proposal 1:  keep AL = 2 for 12 PRBs hypothetical PDCCH transmission BW for RLM IS evaluation.

Proposal 2: Reuse existing side condition Es/Iot for target NR cell detection for RRC connection re-establishment and RRC connection release with re-direction, i.e.,

-	Es/Iot≥-4 dB for target NR cell detection for inter-frequency RRC connection re-establishment and RRC connection release with re-direction

-	Es/Iot≥-6 dB for target NR cell detection for intra-frequency RRC connection re-establishment

# References

[1] R4-2314273, WF on RRM requirements for NR support for dedicated spectrum less than 5MHz for FR1, Nokia, RAN4#108