3GPP TSG-RAN WG4 Meeting # 108bis														R4-2316595

Xiamen, China, 09 – 12 October 2023

Agenda item:			5.21.2

Source:	Samsung

Title:	Views on use case of AI/ML

Document for:	Discussion

1		Introduction

A Rel-18 Study Item on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface has been discussed over the past meetings. So far, RAN4 made solid progress on general issues, use case specific issues, and interoperability/testing with solid progress. However, in RAN4#108, the WF tried to capture the updated agreement and status could not be approved unfortunately [1], while the relevant agreements are captured in the ad-hoc minutes and chair notes [2].

In this contribution, we would like to continue providing our view on the remaining issues of the six sub use cases under the three categorized target enhancements, i.e., CSI feedback enhancement, Beam management, and Positioning accuracy enhancements for different scenarios, based on discussion status in both RAN4 and other WGs.

2		Discussion

Since the lastest WF could not be made  in RAN4#108, the issues in the moderator’s summary have been pasted below to think about remaining topics that should discussed in this meeting. Following issues are listed up for the discussion in narrowing down to the use case specific issues for AI/ML [1]:

Issue 2-1: Metrics/KPIs for CSI requirements/tests Issue 2-2: Metrics/KPIs for Beam prediction requirements/tests Issue 2-3: Metrics/KPIs for positioning requirements/tests Issue 2-4: Model delivery/update/transfer requirements Issue 2-5: Feasibility Intermediate KPIs for CSI requirements or LCM Issue 2-6: Accuracy requirements for measurement data or label data Issue 2-7: Requirements for network side

In short, the issues 2-1 to 2-3 and 2-5 are to discuss metric/KPI for each use case idenfied in RAN1 from the beginning. Issue 2-4, 2-6 and 2-7 are about the need of other requirements than the KPI for the use cases. Based on such classfication, we have tried to share our views on the KPI and other related requirements for each use case.

2.1	CSI feedback enhancement

The test metric of model inference for CSI feedback enhancements, as discussed, it has been considered that the throughput is a promising test metric for both CSI compression and CSI prediction, which has been used for the legacy metric of PMI reporting.

Regarding the intermediate KPI such as SGCS and/or NMSE, CSI compression for example, RAN1 has agreed that SGCS between reconstructed CSI and target CSI would serve as one of the basic KPIs for model inference accuracy, which also means that direct measuring of SGCS could be a baseline for the performance monitoring method. The NMSE is for evaluating AI/ML output of CSI performance monitoring as an additional performance metric for RAN1 evaluation.

In our view, those intermediate KPIs can be considered for performance monitoring LCM. However, the only reason that RAN1 has the intermediate KPI would be to simplify their evaluation since the AI/ML study has so many different kinds of the performance related evaluation for the simulation check. In that light, the throughput would be too heavy to see if the two models are compatible from their evaluation purpose. Therefore, from the actual testing perspective, RAN4 is recommended to focus only on the throughput rather than those intermediate KPIs at least for the model inference discussion.

Observation 1: Throughput is considered as a promising test metric for both CSI compression and CSI prediction.

Observation 2: Only reason for RAN1 to have the intermediate KPIs is to simplify their evaluation without actual testing.

Proposal 1: RAN4 is recommended to focus only on the throughput rather than those intermediate KPIs for model inference discussion.

In addition, according to the performance benefit of the two sub use cases in RAN1, although the performance can be sensitive to UE speed for CSI prediction case, the problem would be alleviated by LCM assistance such as data collection, model/functionality management and performance monitoring. In that sense, the sub use case of time domain CSI prediction would be good to apply AI/ML framework to the UE-side model. However, for the CSI compression, the performance is limited as shown with a marginal gain in terms of the throughput. Also, as the two-sided models can be impacted by training assumption, it requires the complex training procedure that may necessitate offline multi-vendor collaboration.

Observation 3: It has been shown that the CSI compression as the two-sided model can be much impacted by training assumption, and the cost that may necessitate offline multi-vendor collaboration for the limited gain.

2.2	Beam management

At the last meeting, RAN4 discussed following options for the evaluation metric of the beam prediction requirements and tests:

  - Option 1: RSRP accuracy

  - Option 2: beam prediction accuracy: Top-1(%), Top-K(%)

  - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

    - Related measurement accuracy can be considered to determine x

  - Option 4: overhead/latency reduction

  - Option 5: combinations of above options

RSRP accuracy is based on the ideal RSRP of the Top-1 predicted beam, and the ideal RSRP of the Top-1 genie-aid beam, which are the most important metrics for the beam prediction. According to the RAN1 agreement, the difference between those ideal RSRPs could be the RSRP accuracy, which is different with the solid RSRP prediction ruled out already. RAN4 could evaluate the performance with the RSRP accuracy and further discuss how to define the accuracy requirement considering the legacy ones.

Regarding the beam prediction accuracy, in our view, Top-1 (%) should be the baseline, which is the percentage of “the Top-1 strongest beam is Top-1 predicted beam”. According to the simulation results provided in RAN1, since the accuracy of Top-1 (%) and Top-K (%) provides similar information of the prediction accuracy, the others such as Top-K/1 and Top-1/K could be considered as additional metrics for the beam prediction accuracy to make simplify the testing efforts.

The new option 3, which is based on the successful rate with the RSRP gap between predicted beam and strongest beam in dB scale, is considered as the mix of previous options above that would need further study of the benefit at the requirement phase. Therefore, in our view, all the options for the beam management inference performance would be good metrics/KPIs to have for further performance verification.

Observation 4: Most options for the beam management inference performance would be good metrics/KPIs to have for further performance verification.

In terms of the beam prediction performance in RAN1, the decent beam prediction accuracy has been shown with less measurements or RS overhead for spatial domain beam prediction for downlink transmission at least. In terms of generalization, there would be no issue for gNB side downlink transmission beam prediction, but for UE side prediction may need to generalize to different gNB antenna configuration. Based on the analysis, RAN4 should study at least the network side model for DL Tx beam prediction for both spatial domain and temporal domain beam management cases without the procedure of model delivery/update/transfer.

Proposal 2: RAN4 should study at least the network side model for DL Tx beam prediction for both spatial domain and temporal domain beam management cases without the procedure of model delivery/update/transfer.

2.3	Positioning accuracy enhancements for different scenarios

For the metric for positioning related use cases, most proposed candidates for positioning on the table can be considered for the KPIs/metrics to be studied in RAN4. However, for the direct AI/ML positioning, since the output of the model is UE position, which is to verify the final positioning performance, the positioning accuracy with ground truth would be only option applicable for the direct positioning. However, as RAN4 has never defined requirements for positioning accuracy based on the true location, it is not possible to setup test environment to simulate a real scenario at this stage from the testing perspective. Nevertheless, as it does not make sense, or would be too premature not to define or study the test metric in RAN4, it needs to have further discussions on alternative ways to evaluate this AI/ML scenario with legacy features or products for the reference, e.g., reference positioning entity.

For AI/ML assisted positioning, as discussed so far, reusing the legacy accuracy requirements should be the baseline as its model inference output is related to existing timing/power measurements. Among the options on the table, the LOS/NLOS indicator or PRS RSRP is preferred for the AI/ML assisted positioning as these metrics are commonly used in the RAN1 study.

Observation 5: For the direct AI/ML positioning, it would be too premature not to define or study the test metric in RAN4. RAN4 can have further discussions on alternative ways to evaluate this AI/ML scenario with legacy features or products for the reference, e.g., reference positioning entity.

Observation 6: LOS/NLOS indicator or PRS RSRP is preferred for the AI/ML assisted positioning as these metrics are commonly used in the RAN1 study.

Regarding the performance benefit carried out in RAN1, both direct AI/ML positioning and AI/ML assisted positioning have been shown that they can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods. As the use cases are the one-sided model, the discussion can be focused on AI/ML model indication, data collection for training/inference/fine tuning/monitoring and model monitoring in terms of LCM. Therefore, both direct AI/ML position and AI/ML assisted position should be kept considering NW and UE side model without model transfer for further discussion.

Observation 7: Both direct AI/ML positioning and AI/ML assisted positioning have been shown that they can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods.

Proposal 3: Both sub use cases should be kept considering NW and UE side model without model transfer for further discussion.

3		Conclusion

This contribution provides our view on the remaining issues of the six sub use cases under the three categorized target enhancements from RAN4 perspectives. Based on the discussion status in RAN4 and other WGs, following summary could be derived.

Observation 1: Throughput is considered as a promising test metric for both CSI compression and CSI prediction.

Observation 2: Only reason for RAN1 to have the intermediate KPIs is to simplify their evaluation without actual testing.

Proposal 1: RAN4 is recommended to focus only on the throughput rather than those intermediate KPIs for model inference discussion.

Observation 3: It has been shown that the CSI compression as the two-sided model can be much impacted by training assumption, and the cost that may necessitate offline multi-vendor collaboration for the limited gain.

Observation 4: Most options for the beam management inference performance would be good metrics/KPIs to have for further performance verification.

Proposal 2: RAN4 should study at least the network side model for DL Tx beam prediction for both spatial domain and temporal domain beam management cases without the procedure of model delivery/update/transfer.

Observation 5: For the direct AI/ML positioning, it would be too premature not to define or study the test metric in RAN4. RAN4 can have further discussions on alternative ways to evaluate this AI/ML scenario with legacy features or products for the reference, e.g., reference positioning entity.

Observation 6: LOS/NLOS indicator or PRS RSRP is preferred for the AI/ML assisted positioning as these metrics are commonly used in the RAN1 study.

Observation 7: Both direct AI/ML positioning and AI/ML assisted positioning have been shown that they can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods.

Proposal 3: Both sub use cases should be kept considering NW and UE side model without model transfer for further discussion.

Reference

[1] R4-2314740	WF on FS_NR_AIML_air					Qualcomm

[2] R4-2314907	Ad hoc minutes for FS_NR_AIML_air		Qualcomm