3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316613

Xiamen, China, October 09 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.133 on Improvement on SCell/SCG setup delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Supporting the agreements made for on Improvement on SCell/SCG setup delay |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN4 #108 Agreements: Measurement reporting <Agreement> - For UE which is not capable of EMR, when the measurement results during idle/inactive mode are available and valid, UE can report these results to the network during or after RRC setup or resume procedures. The reporting procedure is up to RAN2. <Agreement> - It is up to UE implementation how to select a band to report unless NW specify a band to report. Starting point of the enhanced measurement <Agreement> - For MT originating call, UE starts to perform additional measurement after paging reception. - And for MO call, UE starts to perform additional measurement after first RACH preamble transmission, i.e. Msg1. - Note: the respective agreement can be used to derive requirements once feasibility of enhanced solution is confirmed Ending point of the enhanced measurement <Agreement> - The ending point of enhanced measurement at the reception of the RRC CONNECTED mode measurement configuration (the 1st RRC_reconfiguration message) - Note: the respective agreement can be used to derive requirements once feasibility of enhanced solution is confirmed RAN4 #107 Agreements: RAN4 #106bis-e Agreements: Agreements: - Candidate criteria for measurements validity definition - A) the measurement are performed within the last [X] seconds before it is reported - B) the reported measurement results satisfy measurement accuracy - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB - FFS whether a single or several criteria should be used for measurements validity definition. Agreements: - FFS whether to introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports. RAN4 #106bis-e Agreements: Agreement - RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios. Agreement - UE is allowed to reuse existing measurement, including legacy measurement for cell re-selection and EMR. - UE is allowed to perform addition measurement starting from RRC connection setup/resume procedure. - RAN4 can continue discussion on the feasibility of doing additional measurement starting from RRC setup/resume, and requirements shall be defined if feasible solution is agreed. RAN4 #105 <Agreement>: - Enhancements on UE behavior in IDLE/INACTIVE mode are not in the scope - The following aspects can be further studied - Using measurement results obtained during IDLE/INACTIVE mode for measurements during RRC connection procedure - Note: enhancements on IDLE/INACTIVE mode measurement are not in the scope RAN4 #104-e <Agreement >: - Reduce the number of EMR carriers to be measured for improved measurement. - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable. <Agreement> - Agreement: DRX is not in use during RRC connection setup/resume procedure for enhanced measurement. Editor’s note: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_mob_enh2 WID objective 7 is not supported, hence incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change 1>>

## 3.3	Abbreviations

EMR	Early measurement Reporting

eEMR	Enhanced Early Measurement Reporting

<< End of Change 1>>

<< Start of Change 2>>

## 6.x	Measurements for fast CA/DC setup

### 6.x.1 	Introduction

This clause contains requirements for UE capable of supporting [AvailabilityIndication-r18] and / or [fastCADCmeasurements-r18]. The UE supporting [AvailabilityIndication-r18] shall support indication of the carriers on which UE has detected or measured cells.

Note: UE supporting [fastCADCmeasurements-r18] support measurements at least starting from MSG-1 and Paging message when transitioning from RRC_IDLE or RRC_INACTIVE to RRC_CONNECTED mode.

Editor’s note: update the capabilities based on RAN2 definition.

### 6.x.2	Requirements applicability

The requirements in clause 6.x shall apply provided the

- UE has received a measurement configuration for EMR and/or eEMR measurements and reporting.

- The detected cell is among the list of cells or on a carrier frequency provided for EMR and/or eEMR, and

- The detected cell and SSBs remain detectable until UE reconnects to the network and transmits a measurement report, and

- The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combinations of the UE.

### 6.x.3	Measurement Capability

Measurement availability indication [AvailabilityIndication-r18] capability and [fastCADCmeasurements-r18] measurement capabilities are indicated by the UE according to TS 37.355.

#### 6.x.4	Measurement Reporting Requirements for fast CA/DC setup

UE which has performed measurements during the RRC_IDLE or RRC_INACTIVE mode and, at the time of RRCSetupComplete, the measurements are fulfilling the [validity criteria], and accuracy requirements defined in 10.1.2B and 10.1.5b respectively, shall indicate available measurement information for eEMR according to TS 38.331.

Note: Reporting before the RRCSetupComplete or RRCResumeComplete shall be according to idle-mode DC/CA accuracy requirements for both [AvailabilityIndication-r18] capability and [fastCADCmeasurements-r18] capability.

Note: If UE has performed measurement starting from MSG-1 or Paging message, the UE shall report measurement results based on connected mode accuracy requirements if the reporting occurs in the RRC_CONNECTED mode.

Editor’s note: RAN4 to agree on which accuracy requirements are used during the performance part.

Editor’s note: Finalise once RAN2 agrees the details

UE supporting [fastCADCmeasurements-r18] has yet to finish the measurements during the state transition from RRC_INACTIVE to RRC_CONNECTED, the UE shall indicate the carrier frequencies in RRCResumeComplete or RRCSetupComplete message, and report according to [connected mode requirements].

#### 6.x.6	Measurement Requirements

Editor’s note: FFS which part will be captured in the section 8 or 9.

#### 6.x.6.1	Measurement period requirements for fast CA/DC setup

Editor’s note: FFS, issue open in RAN4

<< End of Change 2>>

<< Start of Change 3>>

# 8	Signalling characteristics

# Editor’s note: FFS if changes to section 8 are needed.

<< End of Change 3>>

<< Start of Change 4>>

# 9	Measurement Procedure

Editor’s note: FFS if changes to section 9 are needed.

<< End of Change 4>>

<< Start of Change 5>>

# 10	Measurement Performance requirements

Editor’s note: Performance part agreements needed.

<< End of Change 5>>