3GPP TSG-RAN WG4 Meeting #108bis 	R4-2316633

Xiamen, China, October 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Carrier Aggregation

Agenda item:	5.12.4.2

Document for:	Discussion

Introduction

In RAN4#108 meeting, the discussions on HST FR2 Enhanced UE Demodulation performance requirements were held, in which the way forward (WF) is documented in [1]. Specifically, for UE Demodulation PDSCH requirements with Carrier Aggregation (CA), the related WF, which will be further discussed in this paper, is summarized below:

Issue 2-1-3: Release independent for CA PDSCH demodulation requirements requirement Agreement - From demodulation point of view, it is feasible to define Rel-18 FR2 HST PDSCH CA requirements which are release independent from Rel-17 according to the UE capability with PC6 UE - The RRM conclusion on release independence of Rel-18 FR2 HST CA requirements shall be taken into account

Apart from the above-mentioned issue captured in WF [1], there is another outcome from RAN4#108 meeting related to HST FR2 PDSCH with CA, namely, the updated simulation assumptions and parameters documented in [4]. With such updates of parameters, consequently, companies need to provide new simulation results according to [4].

Discussion

In the following, we provide our views on remaining issues related to HST FR2 PDSCH with CA. We start with simulation on CA with and without impairments, followed by the release independency of HST FR2 PDSCH with CA.

Simulation on CA with and without Impairments

In RAN#107, some companies already provided simulation results on HST FR2 with CA without impairments [2]. In that meeting, it was further discussed to provide simulation with impairments in RAN4#108, as captured in the WF of RAN4#107 [5].

In RAN4#108, there were no dedicated discussions on the collected simulation results, although the summary of the simulation results is available in [8]. Hence, few related residual issues from RAN4#107 below are still worth to be revisited:

Issues from RAN4#107 [5] not specifically discussed during RAN4#108: Issue 2-1-3: RRH and Channel model assumption for CA Agreement: - For FR2 HST CA Demod requirements, RAN4 will define requirements for the agreed BW carrier configuration based on collected impairment results provided by companies. Issue 2-1-4/5: Additional noise margin for CA requirement based on single carrier Agreement: - No additional noise and imperfection model modelling based on single carrier link level simulation for PDSCH CA alignment result is considered. - The impact of additional noise in CA can be considered into the impairment results up to companies’ implementation Issue 2-1-6: Simulation results alignment and requirement derivation Agreement: - Companies are encouraged to provide the simulation result in the next meeting for further alignment according to the simulation assumption R4-2305893 - The requirement derivation and whether to add additional margin will be discussed based on the collected simulation results.

For discussions in RAN4#108bis meeting, we again provide our simulation results on HST FR2 PDSCH with CA both with and without impairments based on the simulation assumptions in [4]. They are documented in our simulation contribution as reference in [6].

Test cases and simulation parameters for HST FR2 PDSCH with CA have been discussed and agreed in RAN4#108.

Simulation results have been collected since RAN4#107.

RAN4 to collect simulation results in RAN4#108bis for alignment, and define requirements for HST FR2 PDSCH with CA.

Release Independency

There is one issue captured in the WF in [1], namely,

Issue 2-1-3: Release independent for CA PDSCH demodulation requirements requirement Agreement - From demodulation point of view, it is feasible to define Rel-18 FR2 HST PDSCH CA requirements which are release independent from Rel-17 according to the UE capability with PC6 UE - The RRM conclusion on release independence of Rel-18 FR2 HST CA requirements shall be taken into account

The WF of the discussions in RRM during RAN4#108 is documented in [7], in which agreements on issues related to the introduction of new network indication or UE capabilities to support CA enhancements for Rel-18 HST FR2 are shown below:

Issue 2-1-1: Whether need to support separate CA and inter-frequency enhancements for Rel-18 FR2 HST (UE capability) - Agreement: - A single per-UE capability is introduced for FR2 HST CA and inter-frequency measurement enhancement: - [measurementEnhancementCAInterFreqFR2-r18] indicates whether the FR2 PC6 UE supports the enhanced RRM requirements for carrier aggregation as specified in TS 38.133 and the UE supports the enhanced RRM requirements for inter-frequency measurements in connected and idle mode as specified in TS 38.133. Issue 2-1-1: Whether need to support separate CA and inter-frequency enhancements for Rel-18 FR2 HST (network flag signaling) The following agreement was achieved in Tuesday ad-hoc session: - Agreement: - Reuse Rel-17 signaling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in Connected mode. - Reuse Rel-17 signaling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for intra-frequency measurement on SCC for FR2 HST in Connected mode. Issue 2-1-2: Whether need to introduce signalling for idle mode inter-frequency measurement requirements - Agreement: - Reuse Rel-17 IE highSpeedMeasFlagFR2-r17 in SIB to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in idle mode.

RRM has decided to reuse Rel-17 network signaling; hence, no new network indication is introduced.

RRM has introduced a single per-UE capability for FR2 HST CA and inter-frequency measurement enhancement UE capability for Rel-18 HST FR2 CA.

Conclusion

In this paper, we provide our views and responses to the agreements and WF on HST FR2 PDSCH with CA. They are summarized in the following Observations and Proposals:

Observation 1: Test cases and simulation parameters for HST FR2 PDSCH with CA have been discussed and agreed in RAN4#108.

Observation 2: Simulation results have been collected since RAN4#107.

Proposal 1: RAN4 to collect simulation results in RAN4#108bis for alignment, and define requirements for HST FR2 PDSCH with CA.

Observation 3: RRM has decided to reuse Rel-17 network signaling; hence, no new network indication is introduced.

Observation 4: RRM has introduced a single per-UE capability for FR2 HST CA and inter-frequency measurement enhancement UE capability for Rel-18 HST FR2 CA.

- References

- R4-2313919, WF for FR2 HST demodulation requirements, Samsung, RAN4#108, Toulouse, France, August 21 - 25, 2023.

- R4-2308834 Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4#107, Incheon, South Korea, May 22 - 26, 2023.

- R4-2312798, Simulation Results on HST FR2 Enhanced with Carrier Aggregation, Nokia, Nokia Shanghai Bell, RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- R4-2313920, Simulation assumption for FR2 HST demodulation, Samsung, RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- R4-2309825, WF for FR2 HST demodulation requirements, Samsung, RAN4#107, Incheon, South Korea, May 22 - 26, 2023.

- R4-2316634, Simulation Results on HST FR2 Enhanced with Carrier Aggregation, Nokia, Nokia Shanghai Bell, RAN4 #108, Xiamen, China, October 09 – 13, 2023.

- R4-2314297, WF on FR2 HST RRM requirements (part 1), Samsung, RAN4#108, Toulouse, France, August 21st – 25th, 2023

- R4-2312211, Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4 #108, Toulouse, France, August 21 – 25, 2023.