3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316639

Xiamen, China, October 09 – October 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancement of NR Dynamic Spectrum Sharing: General and Work Plan

Agenda item:	5.36.1

Document for:	Discussion

Introduction

In [1], the Work Item Description (WID) of Enhancement of NR Dynamic Spectrum Sharing (DSS) is documented. The objective of Performance part WI is described as below:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. Specify, if any, necessary UE performance requirements for above core objectives: - Specify necessary UE demodulation performance requirements (RAN4)

As RAN4#108bis is the first meeting for performance part of NR enhanced-DSS (eDSS), it is expected that companies need to discuss the general aspects and work plan for this work item (WI). In this contribution, we provide our initial views on the general aspects and work plan for performance part of NR eDSS.

Discussion

As described in the objective of performance part WI, RAN4 needs to specify necessary UE performance requirements for the given core objectives as described in [1] and summarized below:

### 4.1 Objective of SI or Core part WI or Testing part WI The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

From the core objectives above, we can deduce that the work in core part was mainly on the impact on NR PDCCH reception in symbols with LTE CRS REs as well as allowing a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP. Concerning two overlapping CRS rate matching, the justification is to mitigate the interference from neighboring LTE cells: “Transmission of LTE CRS from neighboring LTE cells can cause interference to NR cells. In order to mitigate this interference, it would be useful to have multiple LTE CRS rate matching patterns. However, currently two overlapping LTE rate matching patterns are only supported together with support and configuration of multi-TRP transmission. [1]”

Concerning NR PDCCH reception in symbols with LTE CRS REs, in [2] some studies have been conducted on the BLER performance of PDCCH with puncturing versus superpositioning, which show tradeoffs in performance depending on the aggregation level and the power ratio of CRS and PDCCH. In [3], possible UE Channel Estimation (CE) schemes and possible gNB Tx schemes are discussed. The works in [2] and [3] have shown possible factors which are impacting the demodulation performance of PDCCH, and it is expected that such factors should be considered in RAN4 to define UE demodulation performance requirements of NR eDSS.

With time limitation in mind, RAN4 may need to have an efficient work plan to be able to have UE demodulation requirements for NR eDSS in Rel-18 38.101-4 specification.

RAN4 to limit the scope of UE demodulation performance part to only cover PDCCH.

RAN4 to discuss and select UE channel estimation scheme and gNB Tx scheme to be used in defining the requirements.

RAN4 to discuss whether to have requirements for either puncturing or superpositioning, or both, under the selected UE channel estimation scheme and gNB Tx scheme.

RAN4 to prioritize defining requirements with an assumption of no interference from neighboring LTE cells, hence, no consideration on two overlapping CRS rate matching.

Conclusion

In this contribution, we provide our views on general aspects and work plan of NR eDSS demodulation performance requirements. The following Proposals were made:

Proposal 1: RAN4 to limit the scope of UE demodulation performance part to only cover PDCCH.

Proposal 2: RAN4 to discuss and select UE channel estimation scheme and gNB Tx scheme to be used in defining the requirements.

Proposal 3: RAN4 to discuss whether to have requirements for either puncturing or superpositioning, or both, under the selected UE channel estimation scheme and gNB Tx scheme.

Proposal 4: RAN4 to prioritize defining requirements with an assumption of no interference from neighboring LTE cells, hence, no consideration on two overlapping CRS rate matching.

- References

- RP-221622, Revised WID on Enhancement of NR Dynamic spectrum sharing (DSS), Ericsson, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6-9, 2022

- R1-2204823, DSS – NR PDCCH overlapping with LTE CRS, Nokia, Nokia Shanghai Bell, RAN1#109, e-Meeting, 9 – 20 May, 2022

- R1-2211286, [eDSS] RRC configuration and UE capability for PDCCH on CRS, RAN1#111, Toulouse, France, November 14th – 18th, 2022