3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316640

Xiamen, China, October 09 – October 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements

Agenda item:	5.36.2

Document for:	Discussion

Introduction

In [1], the Work Item Description (WID) of Enhancement of NR Dynamic Spectrum Sharing (DSS) is documented. The objective of Performance part WI is described as below:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. Specify, if any, necessary UE performance requirements for above core objectives: - Specify necessary UE demodulation performance requirements (RAN4)

As RAN4#108bis is the first meeting for performance part of NR enhanced-DSS (eDSS), it is expected that companies need to discuss the general aspects and work plan for this work item (WI). We provide our views on the general aspects and work plan in [4].

In this contribution, we provide our initial views on UE demodulation performance requirements of NR eDSS. The objective of performance part WI stated that the UE performance requirements need to be specified for the given core objective as given below [1]:

### 4.1 Objective of SI or Core part WI or Testing part WI The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

Discussion

Referring to the objective of core part WI, the UE demodulation performance requirements of NR eDSS will mainly be defined for PDCCH. Hence, different to most of other WIs for their demodulation performance requirements, RAN4 do not need to consider requirements for other physical channels and CSI requirements.

For NR eDSS, RAN4 do not need to define UE demodulation requirements for PDSCH and PBCH, and CSI requirements.

In [2], a realistic scenario to assess performance of NR eDSS PDCCH was proposed:

- 15 kHz SCS

- 4-port LTE CRS

- LTE PCFICH/PDCCH occupies symbol #0

- 2-symbol CORESET with SS start in symbol #1.

Realistic scenario is needed to evaluate PDCCH demodulation performance in NR eDSS.

RAN4 to define realistic scenario for evaluation with 4-port LTE CRS, 2-symbol CORESET and SS start in symbol#1 can be considered as a baseline scenario.

Conclusion

In this contribution, we provide our initial views on UE demodulation performance requirements of NR eDSS. The following Observations and Proposals were made:

Proposal 1: For NR eDSS, RAN4 do not need to define UE demodulation requirements for PDSCH and PBCH, and CSI requirements.

Observation 1: Realistic scenario is needed to evaluate PDCCH demodulation performance in NR eDSS.

Proposal 2: RAN4 to define realistic scenario for evaluation with 4-port LTE CRS, 2-symbol CORESET and SS start in symbol#1 can be considered as a baseline scenario.

- References

- RP-221622, Revised WID on Enhancement of NR Dynamic spectrum sharing (DSS), Ericsson, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6-9, 2022

- R1-2204823, DSS – NR PDCCH overlapping with LTE CRS, Nokia, Nokia Shanghai Bell, RAN1#109, e-Meeting, 9 – 20 May, 2022

- R4-2316639, Enhancement of NR dynamic spectrum sharing: General and Work Plan, RAN4#108bis, Xiamen, China, October 09 – 13, 2023