3GPP TSG-RAN WG4 Meeting #108bis                                        	                   R4-2316642

# Xiamen, China, October 9 – October 13, 2023

Agenda item:	5.3.3.1.3

Source:	Apple Inc.

Title:	Final Discussion on SDR Requirements for 8Rx in TDD, FDD and CA

Release:	Rel-18

Document for:	Discussion

# Introduction

Overall, this WI has been focusing on:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices

  - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx

    - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers

  - Specify the SDR requirements with 8 MIMO layers

In addition, as per the revised WID RP-231490 in RAN#100, we wait for RAN4 RF to specify UE RF requirements for both single carrier and CA/DC for 8Rx. About specific band combos and configurations, these are FFS.

  - Specify the UE RF requirements to support 8Rx for both single carrier and CA/DC

    - Example band combos and configurations need to be defined

The main aim of this paper is to summarize the status and still open discussions related to PDSCH demodulation requirements for 8Rx UEs.

# Discussion

The purpose of the SDR requirements is to verify that Layers 1 and 2 correctly process in a sustained manner the received transport blocks corresponding to the maximum data rates indicated by UE capabilities. From what has been previously discussed and agreed [1], we have the following:

Maximum MCS and MIMO layers for 64QAM

- 2 and 4 MIMO layers resus the existing MCS value defined in Table 5.5A-5 of TS 38.101-4

- 8 layers: MCS26 (64QAM Table)

Maximum MCS and MIMO layers for 256QAM

- 2 and 4 layers: reuse the requirements defined in Table 5.5A-5 of TS 38.101-4

- 8 layers: MCS24 (256QAM Table)

Maximum MCS and MIMO layers for 1024QAM

- 2 layers: MCS23 (1024QAM Table)

In congruence with the above, MCS look-up tables have been agreed [1].

With respect to TS 38.101-4, the requirements are applicable to the FR1 single carrier case. For single carrier operation, we resort to the requirements and procedure defined in section 5.5A.1, using the operating band instead of CA configuration, and the bandwidth of the carrier instead of bandwidth combination.

Paraphrasing TS 38.101-4, the general procedure is performed by selecting one CA bandwidth combination among all supported CA configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate in accordance with clause 4.1.2 of TS 38.306. Moreover, as per same section 5.5A.1, when there are multiple sets of CA bandwidth combinations and UE capabilities (channel bandwidth, subcarrier spacing, number of MIMO layer, modulation format, scaling factor) with same largest data rate, select one among sets with the smallest aggregated channel bandwidth.

Proposal #1: Reuse the existing methodology of Re-15 SDR CA test by selecting one CA bandwidth combination among all supported CA configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate in accordance with clause 4.1.2 of TS 38.306

# Conclusion

In this contribution, we provide our final views on 8Rx UE SDR requirements. More specifically:

Proposal #1: Reuse the existing methodology of Re-15 SDR CA test by selecting one CA bandwidth combination among all supported CA configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate in accordance with clause 4.1.2 of TS 38.306.

# References

- R4-2313877, “WF on 8Rx UE performance requirements”, Huawei, HiSilicon, RAN4#108