3GPP TSG-RAN WG4 Meeting # 108bis                                	                        R4-2316643

# Xiamen, China, October 9 – October 13, 2023

Agenda item:	5.3.3.1.4

Source:	Apple Inc.

Title:	Final discussion on CQI Requirements

Release:	Rel-18

Document for:	Discussion

# Introduction

Overall, this WI has been focusing on:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices

  - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx

    - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers

  - Specify the SDR requirements with 8 MIMO layers

In addition, as per the revised WID RP-231490 in RAN#100, we wait for RAN4 RF to specify UE RF requirements for both single carrier and CA/DC for 8Rx. About specific band combos and configurations, these are FFS.

  - Specify the UE RF requirements to support 8Rx for both single carrier and CA/DC

    - Example band combos and configurations need to be defined

The main aim of this paper is to summarize the status and still open discussions related to PDSCH demodulation requirements for 8Rx UEs.

# Discussion

During RAN4#108 all but one of the issues remains. For completeness, we present the set of items discussed during the previous meeting as stated in the WF [1], in addition to our proposal.

TDD Requirements

Issue 6-1-1: Report quantity

Keep the previous agreements [4]

- two-one-TypeI-SinglePanel-Restriction = 00000001

- RI Restriction = 00001000

- TE schedules fixed PMI matrix with i1,1=i1,2= i2=i1,3= 0

Issue 6-1-2: SNR points

[4, 5]dB and [10, 11]dB

FDD Requirements

Issue 6-2-1: Test parameter

- 10 MHz/15 kHz, 52 RBs.

- CQI index for up to 64QAM.

- Rank 4.

Issue 6-2-2: Report quantity

Same as TDD requirements

- two-one-TypeI-SinglePanel-Restriction = 00000001

- RI Restriction = 00001000

- TE schedules fixed PMI matrix with i1,1=i1,2= i2=i1,3= 0

Issue 6-2-3: SNR points

[4, 5]dB and [10,11]dB

Finally, our proposal for this paper,

Proposal#1: Do not consider CQI reporting test for 8Rx CA case.

# Conclusion

In this contribution we provided our view on 8Rx UE CQI reporting requirements. We use this contribution to clarify our stance with respect to

Proposal#1: Do not consider CQI reporting test for 8Rx CA case.

# References

- R4-2313877, “WF on 8Rx UE performance requirements”, Huawei, HiSilicon, RAN4#107