3GPP TSG-RAN4 Meeting #108bis	       R4-2316695 Xiamen, China, 09 – 13 August 2023

Source: 	Qualcomm Inc.

Title: 	TP to TR 38.741: A-MPR for LS-band

Agenda Item:	4.31.3

Document for:	Approval

# Introduction

In this contribution A-MPR simulation results are provided to quantify required A-MPR to meet additional emission requirements in LS-band. The results are included in a text proposal to TR 38.741 to capture the A-MPR simulation data as background for the final A-MPR values.

# Discussion

A number of additional emission requirements have been considered for LS-band, and these additional requirements may results in need for A-MPR to meet the emission limits. A simulation study was done to evaluate the magnitude of A-MPR. The simulations assumptions were:

- PA calibration point: ACLR of 30 dBc is met with DFT-s-OFDM waveform with StartRB = 0, L_CRB = 100

- IQ-image: -28 dBc

- LO-leakage: -28 dBc

- CIM3: -60 dBc

- Requirements considered: NR ACLR, SEM, Spurious emissions, EVM, Additional out of band bands emissions from ETSI EN 301 441 (see table 1 and table 2)

Table 1: Additional out of band emission requirements considered in the simulations

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 2: Maximum unwanted emissions of MES carriers within the operational band of CDMA carriers

| Frequency offset (kHz) (Note 1) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) (Note 2) | Measurement bandwidth | Measurement method |
| 0 to 70 | -6 to -20 | 30 kHz | Average |
| 70 to 600 | -20 to -28 | 30 kHz | Average |
| 600 to 2000 | -28 to -45 | 30 kHz | Average |
| 2000 to 5000 | -45 to -69 | 30 kHz | Average |
| 5000 to 16500 | -69 | 30 kHz | Average |
| NOTE 1: Frequency offset is determined from edge of nominated bandwidth. NOTE 2: Linearly interpolated in dBW vs. frequency offset |  |  |  |

The simulation results based on these assumptions have been captured in the text proposal to avoid duplication of multiple figures inside the Tdoc. Following proposals are made

Proposal 1: Capture the A-MPR simulations results in the technical report 38.741

Proposal 2: Take the simulation results into account when defining the A-MPR specification.

# Conclusions

In this contribution A-MPR simulation study for LS-band was presented. Following proposals are made

Proposal 1: Capture the A-MPR simulations results in the technical report 38.741

Proposal 2: Take the simulation results into account when defining the A-MPR specification.

# References

# Text proposal to TR 38.741 v.0.1.0

#### 6.2.1.3	Emission requirements and NS values

##### 6.2.1.3.X	A-MPR simulations

A simulation study was done to evaluate the magnitude of A-MPR to meet additional emission requirements in LS-band. The simulations assumptions were:

- PA calibration point: ACLR of 30 dBc is met with DFT-s-OFDM waveform with StartRB = 0, L_CRB = 100

- IQ-image: -28 dBc

- LO-leakage: -28 dBc

- CIM3: -60 dBc

- Requirements considered: NR ACLR, SEM, Spurious emissions, EVM, Additional out of band bands emissions from ETSI EN 301 441 (see Table 6.2.1.3.X-1 and Table 6.2.1.3.X-2)

Table 6.2.1.3.X-1: Additional out of band emission requirements considered in the simulations

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.2.1.3.X-2: Maximum unwanted emissions of MES carriers within the operational band of CDMA carriers

| Frequency offset (kHz) (Note 1) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) (Note 2) | Measurement bandwidth | Measurement method |
| 0 to 70 | -6 to -20 | 30 kHz | Average |
| 70 to 600 | -20 to -28 | 30 kHz | Average |
| 600 to 2000 | -28 to -45 | 30 kHz | Average |
| 2000 to 5000 | -45 to -69 | 30 kHz | Average |
| 5000 to 16500 | -69 | 30 kHz | Average |
| NOTE 1: Frequency offset is determined from edge of nominated bandwidth. NOTE 2: Linearly interpolated in dBW vs. frequency offset |  |  |  |

Simulation results are captured in Figure 6.2.1.3.X-1 to Figure 6.2.1.3.X-12. No need for A-MPR was observed for DFT-s-OFDM QPSK and CP-OFDM QPSK with 5 MHz ChBW. Center frequencies used for 5 MHz simulations were 1612.5 MHz, 1615.5 MHz, 1620.5 MHz and 1624 MHz.

Figure 6.2.1.3.X-1: DFT-s-OFDM BPSK, 10 MHz,  Fc = 1615 MHz

Figure 6.2.1.3.X-2: DFT-s-OFDM BPSK, 10 MHz, Fc = 1621.5 MHz

Figure 6.2.1.3.X-3: DFT-s-OFDM QPSK, 10 MHz, Fc = 1615 MHz

Figure 6.2.1.3.X-4: DFT-s-OFDM QPSK, 10 MHz, Fc = 1621.5 MHz

Figure 6.2.1.3.X-5: CP-OFDM QPSK, 10 MHz, Fc = 1615 MHz

Figure 6.2.1.3.X-6: CP-OFDM QPSK, 10 MHz, Fc = 1621.5 MHz

Figure 6.2.1.3.X-7: DFT-s-OFDM BPSK, 15 MHz,  Fc = 1617.5 MHz

Figure 6.2.1.3.X-8: DFT-s-OFDM BPSK, 15 MHz, Fc = 1619 MHz

Figure 6.2.1.3.X-9: DFT-s-OFDM QPSK, 15 MHz, Fc = 1617.5 MHz

Figure 6.2.1.3.X-10: DFT-s-OFDM QPSK, 15 MHz, Fc = 1619 MHz

Figure 6.2.1.3.X-11: CP-OFDM QPSK, 15 MHz, Fc = 1617.5 MHz

Figure 6.2.1.3.X-12: CP-OFDM QPSK, 15 MHz, Fc = 1619 MHz

#### 6.2.1.4	UE co-existence requirements