3GPP TSG-RAN4 Meeting #108bis	       R4-2316997 Xiamen, China, 09 – 13 August 2023

Source: 	Qualcomm Inc.

Title: 	TP to TR 38.869: Low-power wake-up receiver RF aspects

Agenda Item:	5.20.2

Document for:	Approval

# Introduction

Study item description for low-power Wake-up Signal and Receiver for NR can be found in [1]. In this contribution we discuss the UE RF aspects.

# Discussion

In this contribution we concentrate on architectures where detection happens at baseband, either in digital domain or by analog envelope detector. In RAN4#106 we presented results showing that RF design can be relaxed by narrowband WUS design and limiting the WUS placement in the middle of the channel [3]. This analysis was expanded on in RAN4#107 [4] based on the evaluation scenarios agreed in RAN4#106bis [5]. These evaluation scenarios are reproduded below in Table 1

Table 1: LP-WUS evaluation scenarios

| NR RF channel BW | 5MHz for 1.4MHz WUS; 20MHz/100MHz for 5MHz WUS |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1 |
| WUS BW within NR channel | 1.44MHz, 5.04 MHz |
| WUS RB allocation (Note 1) | [6] RB in 1.44 MHz, total 8 RBs, or other number of RBs [24] RB in 5.04 MHz, total 28 RBs, or other number of RBs |
| WUS placement within NR channel | 3 cases: - case 1: Center; - case 2: edge; - case 3: between center and edge of NR channel, 5 MHz offset from channel edge used |
| Guard RB size of LP-WUS | - 0 RB, 1RB at each side, 2RBs at each side, or other number of RBs. - Asymmetric guard RB can also be considered |
| ACS interferer | According to RF CBW |
| Filter characteristic | 2nd to 5th order Butterworth Both analog and digital filter can be considered |
| Filter passband BW | At least WUS bandwidth (number of RBs), depends on guard RB size |
| LO frequency | Case 1: In the middle of WUS (modeling fixed WUS position) Case 2: In the middle of RF channel (modeling flexible WUS location) |
| Target ACS | TBD |
| Target ASCS | TBD |
| Target WUS SNR | TBD |
| RF impairment | FFS |
| Note 1: the maximum number of allocated WUS RBs, depends on how many Guard RBs are needed. 5MHz WUS within 5MHz NR CBW is not considered currently. |  |

In the WF from RAN4#107 [6] it was further agreed to conclude the relationship before guard RB and adjacent channel selectivity before considering target ACS value. It was also confirmed that there is no need to restrict to symmetric guard band and to further discuss whether guardRB for WUS ACS could be used for NR.

Based on these agreements we did simulations to evaluate the impact of adjacent channel and adjacent subcarrier interference. The simulation configuration was created closely match with existing RAN4 minimum requirement for adjacent channel selectivity. Minor adaptations needed to be done as 30 kHz SCS was used. The details of the simulation configuration are provided in Table 2. The main updates to configuration in RAN4#108 has been highlighted

Table 2: Simulation configuration

| NR system BW | 20 MHz |
| --- | --- |
| Subcarrier spacing | 30 kHz |
| Guardband of NR channel | 810 kHz |
| WUS BW within NR channel | 12 RB = 4.32 MHz |
| WUS signal | OOK-4, see [7] |
| Guard RB size of LP-WUS | 0 or 1 RB. GB is placed outside of the 12 RB carrying WUS |
| WUS placement within NR channel | - Immediately adjacent to system BW edge, NR channel GB is respected - Additional 3RB = 1.08 MHz offset from system BW edge compared to case 1) - Additional 6RB = 2.16 MHz offset from system BW edge compared to case 1) NOTE: Together with 1 RB GB, total offset from system BW edge to first WUS RB is reported offset + 360 kHz |
| ACS interferer signal | 12 RB = 4.32 MHz, center of interferer 2.52 MHz offset from system BW channel edge |
| ACS interferer signal level | Relative difference to signals within system BW aligned with Table 7.5-3 in TS 38.101-1 for 20 MHz channel bandwidth |
| Adjacent subcarrier interference | Except for the Guard RB size of LP-WUS of 0 or 1 RB, all subcarriers within system BW carry OFDM QPSK data |
| Filter characteristic | 2nd to 5th order Butterworth |
| Filter passband BW (-3 dB) | - 4.32 MHz - 5 MHz - 6 MHz |
| LO frequency | In the middle of WUS |
| Frequency error | 0 ppm, 50 ppm, 100ppm |
| Phase noise | Not modelled |
| Non-linearities | Not modelled |
| Channel model | TDL-C 300 ns |

It should be noted that in the simulation assumptions result in more relaxed simulation scenario than set by minimum requirements in TS 38.101-1: NR channel GB is slightly wider and interferer signal is slightly further away. Especially comparing to minimum requirements with 15 kHz SCS, there is approximately 380 kHz difference in NR channel GB.

Simulation results are also optimistic as no frequency error, phase noise or non-linearities were modelled. For filter passband BW 3 options were used. With the narrowest passband BW the filter already cuts into WUS with 3 dB attenuation at WUS edges. Wider bandwidths are used to model practical implementation imperfections as well as possibility of missing or simplified calibration for filter corner frequency.

Compared to results presented in RAN#108, these results consider impact of frequency error and also cover performance at wider GB of 6 RB from system BW edge. Also impact of ACSI only is highlighted.

The results in Figures 1 show impact of ASCI without any ACI

Figure 1:

Left: BER vs. SNR with ASCI only, 3rd order filter with 4.32 MHz filter BW, LP-WUS 3 RB offset from system BW edge. Comparison between 0 and 1 RB GB.

Right: BER vs. SNR with ASCI only, 3rd order filter with 4.32 MHz filter BW, LP-WUS 6 RB offset from system BW edge. Comparison between 0 and 1 RB GB.

From Figure 1 it can be seen that applying 360 kHz GB adjacent to WUS does not impact BER. Therefore guard band is unlikely to be needed against ASCS. Furthermore, ASCS has been present in number of RAN1 evaluations, where LP-WUS has been placed in the middle of wider carrier, and no issues have been observed.

Proposal 1: No guard band is needed against adjacent subcarrier interference

In Figure 2 we present results showing the impact of frequency error

Figure 2:

Left: SINR vs. SNR with ASCI and ACI, 3rd order filter with 4.32 MHz filter BW, LP-WUS 3 RB offset from system BW edge with 1 RB GB towards ASCS.

Right: SINR vs. SNR with ASCI and ACI, 3rd order filter with 4.32 MHz filter BW, LP-WUS 6 RB offset from system BW edge with 1 RB GB towards ASCS.

In Figure 2 it can be seen that frequency error has a clear negative impact to SINR, and e.g. in case of 6+1 RB offset from adjacent channel, having frequency error in place can result in need of up to 7 dB higher input SNR to reach the same SINR when compared to against case with no frequency error. With 3+1RB offset to band edge in left sub-figure, impact of frequency error makes it very difficult to reach SINR range where successful operation would be possible. It should be noted that while LP-WUS receiver is expected to periodically synchronize itself, there will also be long sleep periods during which frequency can drift. Therefore, considering large frequency offsets is not unreasonable.

Observation 1: Impact of frequency error increases the guard band need

In Figure 3 and 4, BER vs, SNR results are presented for case where LP-WUS is placed 6 RB offset from system BW edge, and when additional 1 RB GB is applied on top of this.

Figure 3:

Left: BER vs. SNR with ASCI and ACI, 4.32 MHz filter BW, LP-WUS 6 RB offset from system BW edge with no RB GB towards ASCS.

Middle: BER vs. SNR with ASCI and ACI, 5 MHz filter BW, LP-WUS 6 RB offset from system BW edge with no RB GB towards ASCS.

Right: BER vs. SNR with ASCI and ACI, 6 MHz filter BW, LP-WUS 6 RB offset from system BW edge with no RB GB towards ASCS.

Figure 4:

Left: BER vs. SNR with ASCI and ACI, 4.32 MHz filter BW, LP-WUS 6+1 RB offset from system BW edge with no RB GB towards ASCS.

Middle: BER vs. SNR with ASCI and ACI, 5 MHz filter BW, LP-WUS 6+1 RB offset from system BW edge with no RB GB towards ASCS.

Right: BER vs. SNR with ASCI and ACI, 6 MHz filter BW, LP-WUS 6+1 RB offset from system BW edge with no RB GB towards ASCS.

The results in Figures 3 and 4 show that performance is still far from ideal with lower order filters even with 6 RB offset. When looking at the data, there are some things to consider: while frequency error, phase noise and non-linearities will make the performance even more worse, RAN4 allows a significant relaxation to wanted signal level in ACS test. Therefore, the correct SNR range to be looked at would be rather towards the right side of the plots.

Finally Figure 5 shows performance with 3rd filter with different offsets from system BW edge

Figure 5: Performance with 3rd order filter at different RB offsets from system BW edge with 4.32 MHz filter BW

If full flexibility for WUS location within NR channel bandwidth is needed, the opportunities for low-power implementation are severely limited for envelope detection based RF implementations. Analog filtering is not sufficient if WUS is placed on the RBs closest to channel edge. Even with 6RB, i.e. 2.16 MHz offset with 30 kHz SCS, performance challenges remain unless complex high order filtering with accurate filter BW is used. For 15 kHz, the performance is expected to be worse due to narrower NR channel GB.

To enable digital filtering, analog-to-digital conversion needs to have sufficient dynamic range to digitize also the interfering signal. Increase in ADC complexity results in higher power consumption.

Based on the provided data and the observations made from it, we suggest to inform RAN1 that analog envelope detection architectures will perform poorly or require a complex implementation if WUS is placed immediately adjacent to channel edge. To improve the likelihood of successful operation and allowing possibilities for low-power implementation, greater than 1.44 MHz offset from outermost RB edge at channel edge is needed, but exact value needs further study. For architectures using digital detection, placing WUS away from channel edge can enable use of simpler RF HW and power savings.

Observation 2: Analog envelope detection architectures will perform poorly or require a complex implementation if WUS is placed immediately adjacent or at too small offset from channel edge. To improve the likelihood of successful operation and allowing possibilities for low-power implementation, at least 6 RB offset from outermost RB edge at channel edge is preferred. For architectures using digital detection, placing WUS away from channel edge can enable use of simpler RF HW and power savings.

Proposal 2: Capture the results in TR 38.869 as in the attached text proposal

## Noise Figure and Power consumption considerations

Generally noise figure and power consumption are competing specifications – it is very challenging to achieve both low noise figure and low power consumption at the same time and power consumption typically increases when sensitivity is improved. Some trends based on scientific publications can be found e.g. in [8] and [9] with Figure 8 below being reproduced from [8].

Figure 8: Power consumption vs. sensitivity [8]

While this work targets towards low power receiver, the noise figure cannot be allowed increase without boundary, as that can start to negatively impact performance at cell edge.

Observation 3: Low power consumption needs to be balanced with negative impacts to performance.

Overall selection of the NF is a multi-dimensional issue, which depends also on what is the target SNR where the wake-up signal will be designed to be detectable. If the target SNR is low enough, NF can be higher and power consumption lower without negative impacts to the system operation.

The typical NF assumption used in RAN4 is 9 dB noise figure, together with the reference sensitivity signal being detectable at approximately -1 dB SNR. This is based on 2Rx UE. Instead of debating noise figures in RAN4, we should guide RAN1 to design the wake-up signal taking advantage of the specified reference point and considering power consumption in the targe SNR.

Once RAN1 provides more information on wake-up signal RAN4 can then agree on applicable noise figure which enables the system to work as intended. Finally, it should be noted that such low noise figures as specified for typical NR UE cannot be reached with RF envelope detection.

Observation 4: Required NF can be concluded based on coverage target, which is expected to full coverage of the cell, and SNR where wake-up signal can be successfully detected. For reference, 9 dB NF and -1 dB SNR is used for typical NR UE in reference sensitivity test case, but typical NR UE also has 2 receivers. RAN1 should take into account in wake-up signal design that lower SNR will enable higher NF and therefore also lower power consumption. 9 dB noise figure would not be possible to reach at least with RF envelope detection.

# Conclusions

In this contribution RF aspects for low-power wake-up receiver were discussed. Following observations and proposals were made:

Proposal 1: No guard band is needed against adjacent subcarrier interference

Observation 1: Impact of frequency error increases the guard band need

Observation 2: Analog envelope detection architectures will perform poorly or require a complex implementation if WUS is placed immediately adjacent or at too small offset from channel edge. To improve the likelihood of successful operation and allowing possibilities for low-power implementation, at least 6 RB offset from outermost RB edge at channel edge is preferred. For architectures using digital detection, placing WUS away from channel edge can enable use of simpler RF HW and power savings.

Proposal 2: Capture the results in TR 38.869 as in the attached text proposal

Observation 3: Low power consumption needs to be balanced with negative impacts to performance.

Observation 4: Required NF can be concluded based on coverage target, which is expected to full coverage of the cell, and SNR where wake-up signal can be successfully detected. For reference, 9 dB NF and -1 dB SNR is used for typical NR UE in reference sensitivity test case, but typical NR UE also has 2 receivers. RAN1 should take into account in wake-up signal design that lower SNR will enable higher NF and therefore also lower power consumption. 9 dB noise figure would not be possible to reach at least with RF envelope detection.

# References

[1]	RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, Vivo

[2]	R1-2212999, LS to RAN4 on low-power wake-up receiver architectures, RAN1

[3]	R4-2302276 ,Low-power wake-up receiver RF aspects, Qualcomm Inc,

[4]	R4-2307460 ,Low-power wake-up receiver RF aspects, Qualcomm Inc,

[5]	R4-2306617, WF on LP-WUS, Vivo

[6]	R4-2310299, WF on LP-WUS SI

[7]	R1-2302003, Summary of discussion on L1 signal design and procedure for low power WUS, Moderator (Nordic)

[8]	Power Consumption of Integrated Low-Power Receivers, IEEE Journal on emerging and selected topics in circuits and systems, Vol 4, No. 3, September 2014, Emil Nilsson and Christer Svensson

[9]	David D. Wentzloff, “Low Power Radio Survey,” [Online]. www.eecs.umich.edu/wics/low_power_radio_survey.html

The TP is based on R4-2314726: all changes were accepted and new changes are showed with rev marks

# Text proposal to TR 38.829

#### 7.1.2.1	General

This Clause captures the study outcome of LP-WUR RF impacts. RAN4 focus on FR1 frequency ranges as first priority, 2.6GHz is selected as an example band for evaluation purpose.

For evaluation purpose, RAN4 define a new term named as guard RB for LP-WUS, which is Granularity of RB. Meanwhile, the traditional guardband for NR channel bandwidth defined in Clause 5.3, TS 38.101-1 is unchanged.

-	Depends on the WUS location, the LP-WUS guard RB could be the number RBs between LP-WUS and NR signals (edge of WUS RB location to nearest edge of eMBB RB), or the number RBs between LP-WUS and nearest edge of guardband (edge of WUS RB location to lowest/highest NR transmission bandwidth configuration in spec TS 38.101-1)

RAN4 agrees there is no need to restrict symmetric guard RBs for interference rejection of WUS.

RAN4 suggests overall bandwidth of the wake-up signal in the NR channel i.e., desired signal along with all the required guard RBs shall fit in the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 of TS 38.101-1.

Figure 7.1.2.1-1: Definition of the guardband of NR channel and guard RB for LP-WUS

#### 7.1.2.2	ACS evaluation

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

In general, NR ACS requirement is derived based on co-existence study to meet max 5% throughput loss metric. While for LP-WUS, the purpose in the study of ACS is not for justification of co-existence between contained LP-WUS and adjacent NR carrier, but to evaluate the required guard RBs in-between the different signals. Given the WUS waveform would be different from NR signal, the traditional throughput loss metric can not be reused. Therefore, RAN4 agrees to focus on the evaluation of required guard RB rather than ACS requirements with co-existence simulation.

Framework in RAN4 to evaluate ACS and ASCS vs guard RBs:

- ACS and ASCS guard RB can be evaluated based on the following aspects:

  - Typical filter characteristic, e.g., filter order, pass BW, cut-off frequency

  - Guard RB size

  - RF impairment can also be considered

- RAN4 adopt two approaches for guard RB evaluation, i.e., filter suppression level analysis and link level simulation

- Averaged power attenuation at ACS or ASCS frequency range

- FFS whether SINR of the wanted signal at detector input is needed

- FFS whether use ICS to instead ASCS

- FFS Coexistence-simulation-based framework can also be considered

  - FFS on details of coexistence study (if needed) of LP-WUS

  - Coverage should be considered

RAN4 Consider a limited set of WUS scenarios in table below for study purpose in RAN4, as shown in Table 7.1.2.2-1.

Table 7.1.2.2-1: Common LP-WUS evaluation parameters for ACS

| NR RF channel BW | 20MHz/100MHz for 5MHz WUS |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1. 810kHz for wanted cell1 20MHz (5MHz WUS at edge), and 510kHz for interference cell2 5MHz. |
| WUS BW within NR channel | Max 5.04 MHz |
| WUS RB allocation (Note 1) | 24 RB (15kHz SCS) or 12 RB (30kHz SCS) within 5.04 MHz; |
| WUS placement within NR channel | Edge of channel, i.e., close to guardband |
| Guard RB size of LP-WUS | 0 RB, 0.5RB,1RB, 2RB, 3RB |
| ACS interferer (cell 2) | 5MHz with guardband |
| Filter characteristic | 2nd to 5th order Butterworth lowpass@ baseband bandpass@100 MHz bandpass@2.6 GHz |
| Filter passband BW | Same as WUS bandwidth (number of RBs) |
| Target ACS | TBD; reuse NR value as starting point |
| Target WUS SNR | TBD |
| WUS SNR condition | TBD |
| NR power level | TBD |
| RF impairment | Max. Center frequency offset based on companies report |
| Note 1: For easy evaluation, the number of allocated WUS RBs is fixed. The number of Guard RBs is changed. |  |

The frequency response of 2nd to 5th order Butterworth filters are shown as below:

Figure 7.1.2.2-1: Frequency response of Butterworth filters

For ACS guard RB evaluation, RAN4 focus on the WUS at the edge of NR channel case, as shown in Figure 7.1.2.2-2.

Figure 7.1.2.2-2: WUS at the edge of NR channel

Filter suppression level analysis:

For filter suppression level calculation, the suppression level is calculated with averaged power attenuation at ACS frequency range.

Company 1 (R4-2314665)

The parameter assumption is aligned with that in Table 7.1.2.2-1. The required guard RB is provided in the Table 7.1.2.2-1 below. CFO from 0ppm~200ppm is considered as RF impairment impacts.

Table 7.1.2.2-2: Required number of Guard RBs for 5MHz ACS

| Filter order |  |  | ACS, BWinterference = 5MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Guard RB CBW | Frequency Offset | 0RB | 0.5RB | 1RB | 2RB | 3RB |
| 5th | 20MHz, SCS = 15KHz | 0 ppm | -28.20 | -29.51 | -30.37 | -32.43 | -34.40 |
|  |  | 5 ppm | -27.95 | -29.26 | -30.11 | -32.18 | -34.14 |
|  |  | 10 ppm | -27.69 | -29.01 | -29.86 | -31.92 | -33.88 |
|  |  | 50 ppm | -25.73 | -27.02 | -27.87 | -29.92 | -31.87 |
|  |  | 200 ppm | -19.35 | -20.52 | -21.31 | -23.24 | -25.13 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -34.51 | -36.33 | -38.07 | -41.33 | -44.33 |
|  |  | 5 ppm | -34.25 | -36.07 | -37.81 | -41.07 | -44.07 |
|  |  | 10 ppm | -33.99 | -35.81 | -37.55 | -40.81 | -43.81 |
|  |  | 50 ppm | -31.97 | -35.53 | -33.79 | -38.79 | -41.79 |
|  |  | 200 ppm | -25.21 | -27.00 | -28.72 | -31.97 | -34.97 |
| 4th | 20MHz, SCS = 15KHz | 0 ppm | -23.71 | -24.70 | -25.35 | -26.92 | -28.42 |
|  |  | 5 ppm | -23.51 | -24.50 | -25.15 | -26.72 | -28.22 |
|  |  | 10 ppm | -23.31 | -24.30 | -24.94 | -26.51 | -28.01 |
|  |  | 50 ppm | -21.75 | -22.73 | -23.37 | -24.92 | -26.41 |
|  |  | 200 ppm | -16.73 | -17.61 | -18.20 | -19.65 | -21.08 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -28.35 | -29.76 | -31.11 | -33.64 | -35.97 |
|  |  | 5 ppm | -28.14 | -29.55 | -30.90 | -33.43 | -35.77 |
|  |  | 10 ppm | -27.94 | -29.34 | -30.69 | -33.22 | -35.56 |
|  |  | 50 ppm | -26.34 | -27.74 | -29.08 | -31.61 | -33.95 |
|  |  | 200 ppm | -20.98 | -22.34 | -23.67 | -26.17 | -28.50 |
| 3rd | 20MHz, SCS = 15KHz | 0 ppm | -18.80 | -19.49 | -19.94 | -21.04 | -22.10 |
|  |  | 5 ppm | -18.65 | -19.34 | -19.79 | -20.89 | -21.94 |
|  |  | 10 ppm | -18.50 | -19.19 | -19.64 | -20.74 | -21.79 |
|  |  | 50 ppm | -17.36 | -18.04 | -18.48 | -19.56 | -20.61 |
|  |  | 200 ppm | -13.68 | -14.29 | -14.69 | -15.70 | -16.69 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -21.90 | -22.90 | -23.87 | -25.70 | -27.39 |
|  |  | 5 ppm | -21.75 | -22.75 | -23.72 | -25.54 | -27.24 |
|  |  | 10 ppm | -21.59 | -22.60 | -23.56 | -25.39 | -27.09 |
|  |  | 50 ppm | -20.41 | -21.41 | -22.37 | -24.19 | -25.88 |
|  |  | 200 ppm | -16.46 | -17.43 | -18.36 | -20.14 | -21.82 |
| 2nd | 20MHz, SCS = 15KHz | 0 ppm | -13.43 | -13.84 | -14.11 | -14.77 | -15.41 |
|  |  | 5 ppm | -13.34 | -13.75 | -14.02 | -14.68 | -15.31 |
|  |  | 10 ppm | -13.25 | -13.65 | -13.92 | -14.58 | -15.22 |
|  |  | 50 ppm | -12.52 | -12.92 | -13.18 | -13.83 | -14.45 |
|  |  | 200 ppm | -10.16 | -10.53 | -10.77 | -11.36 | -11.95 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -15.18 | -15.80 | -16.40 | -17.54 | -18.62 |
|  |  | 5 ppm | -15.08 | -15.70 | -16.30 | -17.44 | -18.52 |
|  |  | 10 ppm | -14.98 | -15.60 | -16.20 | -17.34 | -18.41 |
|  |  | 50 ppm | -14.22 | -14.83 | -15.42 | -16.56 | -17.62 |
|  |  | 200 ppm | -11.70 | -12.28 | -12.85 | -13.94 | -14.98 |

Company 5 (R4-2311502)

The evaluation parameters are listed in Table 7.1.2.2-3 below.

Table 7.1.2.2-3 Evaluation assumptions.

| WUS signal bandwidth | 4.32 MHz |
| --- | --- |
| Interferer power level | 0 dBc |
| F0 | 2.6 GHz |
| Filter Type | Butterworth |

Table 7.1.2.2-4 Guard required in KHz for a target interference level of -10 dBc.

| CFO | Filter order |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| ± 0 ppm | 4320.0 | 1598.4 | 972.0 | 691.2 | 540 |
| ± 5 ppm | 4346.0 | 1608.0 | 977.9 | 695.4 | 543.3 |
| ± 50 ppm | 4580.0 | 1694.6 | 1030.5 | 732.8 | 572.5 |
| ± 100 ppm | 4840.0 | 1790.8 | 1089.0 | 774.4 | 605.0 |
| ± 200 ppm | 5360.0 | 1983.2 | 1206.0 | 857.6 | 670.0 |

Based on the results in Table 7.1.2.2-4, we suggest that for 5th order filter, the guard RB number for LP-WUS can be 2RBs for 30KHz SCS, or 3RBs ~ 4RBs for 15KHz SCS.

Link-level simulation analysis:

Company 1 (R4-2314665)

The detailed parameters are listed in Table 7.1.2.2-5.

Table 7.1.2.2-5: LP-WUS ACS and ASCS simulation parameters

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Carrier Frequency | 2.6GHz |  |  |
| Case name | OOK-1 waveform | OOK-2 waveform | OOK-4 waveform |
| Channel structure | data: 6 bits CRC: 8 bits | data: 20 bits CRC: 8 bits | data: 20bits CRC: 8 bits |
| Chip rate | M=1 | M=2 | M=2 |
| WUS duration | 28 symbols |  |  |
| Waveform | OOK -1/OOK-2/OOK-4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| residual Frequency error | 0/10/20/50/22700 ppm(for ASCS evaluation) |  |  |
| SCS | 30kHz |  |  |
| gNB Channel BW | 20MHz (50 RB) |  |  |
| WUS BW | 12RB ~4.32MHz |  |  |
| Guardband of NR channel (ACS) | 27RE~810kHz for wanted cell1 20MHz (5MHz WUS at edge), and 510kHz for interference cell2 5MHz. |  |  |
| Guard RB | Outer GB:2/1/0.5RB on each side of LP-WUS bandwidth (2 RB for ACS) Inner GB: 4RE between segments |  |  |
| Filter | 3th/5th Order lowpass Butterworth with 4.32MHz bandwidth (2.1 MHz bandwidth for OOK-2 M=2) |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| ACS | PDSCH mapped on RBs(14RB~5MHz), one side; EPRE of PDSCH /EPRE of LP-WUS = 31.5 dB 31.5dB higher |  |  |
| Sampling Rate | 7.68MHz |  |  |
| ADC bit width | 4 bits ADC for ASCS, 8 bits ADC for ACS; |  |  |
| Phase noise | unable |  |  |
| Power boosting | 0dB for OOK-1/4 3dB for OOK-2 |  |  |
| Channel Model | TDL-C 300 |  |  |
| Performance metric | {FAR, MDR}: {1%, 1%} |  |  |

The simulation results are in Figure 7.1.2.2-3 for ACS.

Figure 7.1.2.2-3. ACS guard RB evaluation

It can be observed that 1 RB of 30kHz SCS with 5th order filtering is sufficient for OOK-1, OOK-2, and OOK-4 LP-WUS.

Company 2 (R4-2312248)

The 5MHz interferer is 31.5dB higher than the wanted signal for ACS simulation. Simulations for the waveforms in Figure 6 have been performed.

Figure 7.1.2.2-4: Some of the LP-WUS waveform options

For waveform options of OOK-2, FSK-1 and FSK-2, since segmented RB allocations are considered, several narrower filters would be needed to filter out the interference in contrast to a single filter used for OOK-1 and OOK-4.

Figure 7.1.2.2-5 shows the simulation results for the aforementioned waveforms.

a) Guard RB evaluation of OOK-1									 b) Guard RB evaluation of OOK-2

c) Guard RB evaluation of OOK-4					  				d) Guard RB evaluation of FSK-1

e) Guard RB evaluation of FSK-2

Figure 7.1.2.2-5: ACS guard RB evaluation for several waveforms

For the evaluated waveform options of OOK-1, OOK-2, OOK-4, FSK-1 and FSK-2, guard RB with the size of one 30kHz SCS RB could provide necessary protection of LP-WUS from interference of adjacent NR carrier. For 5th order Butterworth filter assumption, it is proposed to reserve 1 RB for 30kHz SCS or 2RBs for 15kHz SCS for protection of LP-WUS from interference of the adjacent NR carrier.

Company 3 (R4-2311902, R4-2316697)

Simulations were done to evaluate the impact of adjacent channel and adjacent subcarrier interference. The simulation configuration was created closely match with existing RAN4 minimum requirement for adjacent channel selectivity. Minor adaptations needed to be done as 30 kHz SCS was used. The details of the simulation configuration are provided in Table 7.1.2.2-6.

Table 7.1.2.2-6: Simulation configuration

| NR system BW | 20 MHz |
| --- | --- |
| Subcarrier spacing | 30 kHz |
| Guardband of NR channel | 810 kHz |
| WUS BW within NR channel | 12 RB = 4.32 MHz |
| WUS signal | OOK-4 |
| Guard RB size of LP-WUS | 0 or 1 RB. GB is placed outside of the 12 RB carrying WUS |
| WUS placement within NR channel | - Immediately adjacent to system BW edge, NR channel GB is respected - Additional 3RB = 1.08 MHz offset from system BW edge compared to case 1) - Additional 6 RB = 2.16 MHz offset from system BW edge compared to case 1 - Additional 12 RB = 4.32 offset from system BW edge compared to case 1 NOTE: Together with 1 RB GB, total offset from system BW edge to first WUS RB is reported offset + 360 kHz |
| ACS interferer signal | 12 RB = 4.32 MHz, center of interferer 2.52 MHz offset from system BW channel edge |
| ACS interferer signal level | Relative difference to signals within system BW aligned with Table 7.5-3 in TS 38.101-1 for 20 MHz channel bandwidth |
| Adjacent subcarrier interference | Except for the Guard RB size of LP-WUS of 0 or 1 RB, all subcarriers within system BW carry OFDM QPSK data |
| Filter characteristic | 2nd to 5th order Butterworth |
| Filter passband BW (-3 dB) | 4.32 MHz 5 MHz 6 MHz |
| LO frequency | In the middle of WUS |
| Frequency error | 0, 50, 100 ppm |
| Phase noise | Not modelled |
| Non-linearities | Not modelled |
| Channel model | TDL-C 300 ns |

Simulation results are also optimistic as no frequency error, phase noise or non-linearities were modelled. For filter passband BW 3 options were used. With the narrowest passband BW the filter already cuts into WUS with 3 dB attenuation at WUS edges. Wider bandwidths are used to model practical implementation imperfections as well as possibility of missing or simplified calibration for filter corner frequency.

First an example of ACI impact and GB impact are presented in Figures 7.1.2.2-6 and 7.1.2.2-7.

Figure 7.1.2.2-6: Impact of ACI, 3rd order filter, 4.32 MHz filter BW, WUS at system BW edge with GB = 0 RB

Figure 7.1.2.2-7: Impact of GB, 3rd order filter, 4.32 MHz filter BW, WUS at system BW edge with GBoff meaning GB = 0RB and with GBon meaning 1 RB GB. ACI is on for both curves.

It can be seen that for the chosen example scenario, adding ACI results in extremely poor BER. With no ACI present, there is clear relationship between SNR and BER. Whether guard band of 1 RB = 360 kHz is present or not does not change the results meaningfully. When looking at the SINR vs. SNR, the explanation is clear: when ACI is present, SINR never increase above -7 dB when SNR increases to 10 dB.

Next results are presented for different filter bandwidths and filter orders when ACI is present in Figures 7.1.2.2-8 and 7.1.2.2-9.

Figure 7.1.2.2-8: Left: BER vs. SNR, 4.32 MHz filter BW, WUS at system BW edge, GB = 1 RB; Right: BER vs. SNR, 5 MHz filter BW, WUS at system BW edge, GB = 1 RB

Figure 7.1.2.2-9: Left: BER vs. SNR, 4.32 MHz filter BW, WUS at system BW edge, GB = 0 RB; Right: BER vs. SNR, 5 MHz filter BW, WUS at system BW edge, GB = 0 RB

It can be seen that with 1 RB GB, BER is starting to be borderline acceptable with higher SNR levels only with 5th order filter which for which the -3 dB bandwidth is exactly aligned with WUS BW. This is when no phase noise, non-linearities, frequency error, or practical filter implementation aspects are considered. Also with 15 kHz SCS, the NR channel GB would be narrower and performance with the same 360 kHz GB would be worse. Therefore, we conclude that 360 kHz GB at channel edge does not enable low-power low-cost RF implementation of wake-up receiver.

To estimate the required GB we simulated how the performance changes if 3 additional RBs, i.e. additional 1.08 MHz, are used for guard band at channel edge. These RBs carry NR OFDM signal. Together with the WUS GB of 0 or 1 RB, the total separation from system BW edge is up to 1.44 MHz, and this is on top of the NR channel GB of 810 kHz. Results are shown in Figures 7.1.2.2-10 to 7.1.2.2-11.

Figure 7.1.2.2-10: Left: BER vs. SNR, 4.32 MHz filter BW, WUS 3 RB offset from system BW edge, GB = 0 RB; Middle: BER vs. SNR, 5 MHz filter BW, WUS 3 RB offset from system BW edge, GB = 0 RB; Right: BER vs. SNR, 6 MHz filter BW, WUS 3 RB offset from system BW edge, GB = 0 RB

Figure 7.1.2.2-11: Left: BER vs. SNR, 4.32 MHz filter BW, WUS 3 RB offset from system BW edge, GB = 1 RB; Middle: BER vs. SNR, 5 MHz filter BW, WUS 3 RB offset from system BW edge, GB = 1 RB; Right: BER vs. SNR, 6 MHz filter BW, WUS 3 RB offset from system BW edge, GB = 1 RB

In Figure 7.1.2.2-12 and 7.1.2.2-13, BER vs, SNR results are presented for case where LP-WUS is placed 6 RB offset from system BW edge, and when additional 1 RB GB is applied on top of this.

Figure 7.1.2.2-12:

Left: BER vs. SNR with ASCI and ACI, 4.32 MHz filter BW, LP-WUS 6 RB offset from system BW edge with no RB GB towards ASCS.

Middle: BER vs. SNR with ASCI and ACI, 5 MHz filter BW, LP-WUS 6 RB offset from system BW edge with no RB GB towards ASCS.

Right: BER vs. SNR with ASCI and ACI, 6 MHz filter BW, LP-WUS 6 RB offset from system BW edge with no RB GB towards ASCS.

Figure 7.1.2.2-13:

Left: BER vs. SNR with ASCI and ACI, 4.32 MHz filter BW, LP-WUS 6+1 RB offset from system BW edge with no RB GB towards ASCS.

Middle: BER vs. SNR with ASCI and ACI, 5 MHz filter BW, LP-WUS 6+1 RB offset from system BW edge with no RB GB towards ASCS.

Right: BER vs. SNR with ASCI and ACI, 6 MHz filter BW, LP-WUS 6+1 RB offset from system BW edge with no RB GB towards ASCS.

The results in Figures 3 and 4 show that performance is still far from ideal with lower order filters even with 6 RB offset. When looking at the data, there are some things to consider: while frequency error, phase noise and non-linearities will make the performance even more worse, RAN4 allows a significant relaxation to wanted signal level in ACS test. Therefore, the correct SNR range to be looked at would be rather towards the right side of the plots.

Finally Figure 7.1.2.2-14 shows performance with 3rd filter with different offsets from system BW edge

Figure 7.1.2.2-14: Performance with 3rd order filter at different RB offsets from system BW edge with 4.32 MHz filter BW

The results in Figures 7.1.2.2-6 to Figure 7.1.2.2-11 show that even with total of 1.44 MHz separation from system BW edge, on top of the 810 kHz NR channel GB the SINR with 3rd order filter will not rise above 0 dB. This is with the filter BW matched to WUS BW, no nonlinearities, no frequency error and no phase noise present. If some practical filter implementation aspects are considered and therefore 5 MHz filter BW is considered, only 5th order filter reaches acceptable performance. With 15 kHz SCS the NR channel GB would be narrower, and performance would be worse.

The results in Figures 7.1.2.2-12 to Figure 7.1.2.2-14 further show that more than 2.16 MHz offset from system BW edge, on top of the NR channel GB, is preferred for improved system performance with lower complexity implementation.

When frequency error is consider in addition, there is additional negative impact to performance, as exemplified in Figure 7.1.2.2-15

Figure 7.1.2.2-15:

Left: SINR vs. SNR with ASCI and ACI, 3rd order filter with 4.32 MHz filter BW, LP-WUS 3 RB offset from system BW edge with 1 RB GB towards ASCS.

Right: SINR vs. SNR with ASCI and ACI, 3rd order filter with 4.32 MHz filter BW, LP-WUS 6 RB offset from system BW edge with 1 RB GB towards ASCS.

In Figure 7.1.2.2-15 it can be seen that frequency error has a clear negative impact to SINR, and e.g. in case of 6+1 RB offset from adjacent channel, having frequency error in place can result in need of up to 7 dB higher input SNR to reach the same SINR when compared to against case with no frequency error. With 3+1RB offset to band edge in left sub-figure, impact of frequency error makes it very difficult to reach SINR range where successful operation would be possible. It should be noted that while LP-WUS receiver is expected to periodically synchronize itself, there will also be long sleep periods during which frequency can drift. Therefore, considering large frequency offsets is not unreasonable.

If full flexibility for WUS location within NR channel bandwidth is needed, the opportunities for low-power implementation are severely limited for envelope detection based RF implementations. Analog filtering is not sufficient if WUS is placed on the RBs closest to channel edge. Even with 4RB, i.e. 1.44 MHz offset with 30 kHz SCS, performance challenges remain unless complex high order filtering with accurate filter BW is used. For 15 kHz, the performance is expected to be worse due to narrower NR channel GB.

To enable digital filtering, analog-to-digital conversion needs to have sufficient dynamic range to digitize also the interfering signal. Increase in ADC complexity results in higher power consumption.

Analog envelope detection architectures will perform poorly or require a complex implementation if WUS is placed immediately adjacent to channel edge. To improve the likelihood of successful operation and allowing possibilities for low-power implementation, greater than 2.16 MHz offset from outermost RB edge at channel edge is preferred, but exact value needs further study. For architectures using digital detection, placing WUS away from channel edge can enable use of simpler RF HW and power savings.

In case only adjacent subcarrier interference is considered, situation is greatly improved. Figure 7.1.2.2-16 shows impact to performance with 3rd order filter (4.32 MHz filter BW) with and without 1 RB GB between WUS and other NR subcarriers:

Figure 7.1.2.2-16:

Left: BER vs. SNR with ASCI only, 3rd order filter with 4.32 MHz filter BW, LP-WUS 3 RB offset from system BW edge. Comparison between 0 and 1 RB GB.

Right: BER vs. SNR with ASCI only, 3rd order filter with 4.32 MHz filter BW, LP-WUS 6 RB offset from system BW edge. Comparison between 0 and 1 RB GB.

Based on the results, guard band does not impact the performance and no guard band is required between WUS and other NR subcarriers.

Company 4 (R4-2313476)

The simulation parameters are listed in Table 7.1.2.2-7 below.

Table 7.1.2.2-7: LLS parameters

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| Channel | TDL-C |
| SCS | 30 kHz |
| Delay spread | 300 ns |
| UE speed | 3 km/h |
| UE Rx antennas | 1 for WUR |
| WUR sampling rate | 10 Msps |
| WUS bandwidth | For OOK WUS: 5 MHz (for 30 kHz SCS: 12 PRBs+ ~ 1 PRB guard band each side of WUS) |
| Number of ADC bits | 8 |
| Receiver filter | 3rd order Butterworth, filter BW is the same as WUS BW |
| Non-WUS transmissions | Adjacent channel interference from other NR transmissions is considered. |
| Max. false alarm due to noise or random symbols | 0.1% |
| Frequency offset (ppm) | For OOK WUS: 50 ppm maximum frequency error |
| Rx approach | Payload based: based on Manchester decoding. |
| RF impairments | Phase noise profile Phase noise (dBc/Hz). Offset; NF 9 dB; NF 12 dB; NF 15 dB. @5MHz; -99.4; -99.4; -99.4. @10MHz; -108.1; -105; -102.1. @15MHz; -120; -117; -114. |

It can be observed in Figure 7.1.2.2-12 that for SNR@10% BLER the SNR gain is 2 dB when increasing the offset between WUS to ACS interferer from 5MHz to 10MHz for NF of 15 dB. For SNR@1% which is the same with PDCCH, the 5MHz offset result in error floor. The SNR gain is even higher for smaller NF (12 dB or 9 dB). The trend is similar for 1bit OOK and multi-bit OOK, though the SNR gain is more for multi OOK bit case than for single OOK bit per case.  The simulation does not take account the leakage power from ACS interferer. As for the reciprocal mixing, this is the impairment when a high-power level interferer signal present and filter attenuation does not help when the phase noise of the LO is mixed with interferer into the wanted signal bandwidth.

Figure 7.1.2.2-12: Link performance when ACS interferer is present.

Summary for ACS guard RB analysis:

Based on the analysis above, RAN4 suggested that for 5th order filter, the guard RB number for LP-WUS ACS is in the range of 1RB ~ 3RBs for 30KHz SCS, or 2RBs ~6RBs for 15KHz SCS.

- 7.1.2.3	ASCS evaluation

To evaluate the in-channel interference, RAN4 has defined a new metric as Adjacent Subcarrier Selectivity (ASCS) for evaluation purpose, which is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent sub-carrier(s).

The ACSC BWinterference is set as 5MHz, for both ~5MHz and ~1.4MHz WUS cases, as a starting point for ASCS evaluation. The same PSD of WUS and NR signal is assumed within the channel.

As shown in Figure 7.1.2.3-1, WUS at the center of NR channel would be the worst case of ASCS.

Figure 7.1.2.3-1：WUS at the centre of NR channel, only ASCS needs to be considered.

Table 7.1.2.3-1: LP-WUS evaluation parameters for WUS at centre

| NR RF channel BW | 20MHz/100MHz for 5MHz WUS |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1 |
| WUS BW within NR channel | Max 5.04 MHz |
| WUS RB allocation (Note 1) | 24 RB (15kHz SCS) or 12 RB (30kHz SCS) in 5.04 MHz; |
| WUS placement within NR channel | case 1: Center; |
| Guard RB size of LP-WUS | 0 RB, 0.5RB,1RB, 2RB, 3RB at each side Symmetric guard RB |
| ASCS interferer | 5MHz |
| Filter characteristic | 2nd to 5th order Butterworth lowpass@ baseband bandpass@100MHz bandpass@2.6GHz |
| Filter passband BW | Same as WUS bandwidth (number of RBs) |
| Target ACS | TBD |
| Target ASCS | TBD |
| Target WUS SNR | TBD |
| NR power level | Same PSD with WUS signal |
| RF impairment | Max. Center frequency offset = 5, 10, 50, 200 ppm. Initial search |
| Note 1: For easy evaluation, the number of allocated WUS RBs is fixed. The number of Guard RBs is changed. |  |

Filter suppression level analysis:

Company 1 (R4-2314665)

The parameter assumption is aligned with that in Table 7.1.2.2-1. The required guard RB is provided in the Table 7.1.2.3-1 below.

Table 7.1.2.3-2: 5MHz LP-WUS ASCS results for WUS at centre

| Filter order |  |  | ASCS, BWinterference = 5MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Guard RB CBW | Frequency Offset | 0RB | 0.5RB | 1RB | 2RB | 3RB |
| 5th | 20MHz, SCS = 15KHz | 0 ppm | -19.25 | -20.67 | -21.63 | -24.04 | -26.39 |
|  |  | 5 ppm | -19.04 | -20.45 | -21.40 | -23.79 | -26.13 |
|  |  | 10 ppm | -18.83 | -20.22 | -21.17 | -23.55 | -25.89 |
|  |  | 50 ppm | -17.26 | -18.54 | -19.42 | -21.68 | -23.95 |
|  |  | 200 ppm | -12.73 | -13.63 | -14.26 | -15.96 | -17.81 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -18.24 | -20.63 | -22.97 | -27.59 | -31.80 |
|  |  | 5 ppm | -18.03 | -20.40 | -22.72 | -27.34 | -31.54 |
|  |  | 10 ppm | -17.82 | -20.17 | -22.48 | -27.08 | -31.28 |
|  |  | 50 ppm | -16.25 | -18.42 | -20.61 | -25.11 | -29.28 |
|  |  | 200 ppm | -11.74 | -13.27 | -14.90 | -18.70 | -22.60 |
| 4th | 20MHz, SCS = 15KHz | 0 ppm | -16.99 | -18.06 | -18.78 | -20.58 | -22.34 |
|  |  | 5 ppm | -16.82 | -17.88 | -18.60 | -20.39 | -22.14 |
|  |  | 10 ppm | -16.56 | -17.71 | -18.42 | -20.20 | -21.95 |
|  |  | 50 ppm | -15.37 | -16.36 | -17.03 | -18.73 | -20.42 |
|  |  | 200 ppm | -11.54 | -12.28 | -12.80 | -14.15 | -15.57 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -16.03 | -17.84 | -19.57 | -23.06 | -26.27 |
|  |  | 5 ppm | -15.87 | -17.65 | -19.38 | -22.86 | -26.06 |
|  |  | 10 ppm | -15.70 | -17.47 | -19.19 | -22.65 | -25.86 |
|  |  | 50 ppm | -14.42 | -16.08 | -17.72 | -21.10 | -24.27 |
|  |  | 200 ppm | -10.61 | -11.87 | -13.15 | -16.05 | -18.99 |
| 3rd | 20MHz, SCS = 15KHz | 0 ppm | -14.18 | -14.92 | -15.42 | -16.65 | -17.86 |
|  |  | 5 ppm | -14.05 | -14.79 | -15.28 | -16.51 | -17.71 |
|  |  | 10 ppm | -13.92 | -14.66 | -15.15 | -16.37 | -17.57 |
|  |  | 50 ppm | -12.95 | -13.64 | -14.11 | -15.28 | -16.45 |
|  |  | 200 ppm | -9.95 | -10.51 | -10.89 | -11.87 | -12.87 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -13.33 | -14.58 | -15.76 | -18.17 | -20.42 |
|  |  | 5 ppm | -13.20 | -14.44 | -15.62 | -18.02 | -20.27 |
|  |  | 10 ppm | -13.07 | -14.31 | -15.48 | -17.87 | -20.12 |
|  |  | 50 ppm | -12.10 | -13.28 | -14.40 | -16.73 | -18.95 |
|  |  | 200 ppm | -9.13 | -10.07 | -10.10 | -13.04 | -15.08 |
| 2nd | 20MHz, SCS = 15KHz | 0 ppm | -10.71 | -11.15 | -11.44 | -12.17 | -12.88 |
|  |  | 5 ppm | -10.62 | -11.06 | -11.35 | -12.08 | -12.78 |
|  |  | 10 ppm | -10.54 | -10.98 | -11.27 | -11.99 | -12.69 |
|  |  | 50 ppm | -9.89 | -10.31 | -10.59 | -11.29 | -11.98 |
|  |  | 200 ppm | -7.86 | -8.22 | -8.46 | -9.06 | -9.67 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -10.05 | -10.79 | -11.49 | -12.92 | -14.27 |
|  |  | 5 ppm | -9.97 | -10.71 | --11.40 | -12.82 | -14.18 |
|  |  | 10 ppm | -9.89 | -10.62 | -11.31 | -12.73 | -14.08 |
|  |  | 50 ppm | -9.25 | -9.96 | -10.62 | -12.00 | -13.33 |
|  |  | 200 ppm | -7.24 | -7.85 | -8.42 | -9.65 | -10.87 |

Company 4 (R4-2311502)

The evaluation parameters are listed in Table 7.1.2.3-3 below.

Table 7.1.2.3-3 Evaluation assumptions.

| WUS signal bandwidth | 4.32 MHz |
| --- | --- |
| Interferer power level | 0 dBc |
| F0 | 2.6 GHz |
| Filter Type | Butterworth |

Table 7.1.2.3-4 Guard required in KHz for a target interference level of -10 dBc.

| CFO | Filter order |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| ± 0 ppm | 4320.0 | 1598.4 | 972.0 | 691.2 | 540 |
| ± 5 ppm | 4346.0 | 1608.0 | 977.9 | 695.4 | 543.3 |
| ± 50 ppm | 4580.0 | 1694.6 | 1030.5 | 732.8 | 572.5 |
| ± 100 ppm | 4840.0 | 1790.8 | 1089.0 | 774.4 | 605.0 |
| ± 200 ppm | 5360.0 | 1983.2 | 1206.0 | 857.6 | 670.0 |

Based on the above table, we suggest that for 5th order filter, the guard RB number for LP-WUS can be 2RBs for 30KHz SCS, or 3RBs ~ 4RBs for 15KHz SCS.

Link-level simulation analysis:

Company 1 (R4-2314665)

The detailed parameters are listed in Table 7.1.2.2-3. The simulation results are in Figure 7.1.2.3-2.

Figure 7.1.2.3-2: ASCS guard RB evaluation

Company 2 (R4-2312248)

Different from ACS, for evaluation of ICS we consider the PSD for all RBs including both LP-WUS and NR signal is flat and no power boosting is considered for LP-WUS signal. Figure 7.1.2.3-3 shows the ICS guard band evaluation for several different waveforms, all the simulations are based on 5th order Butterworth filter.

a) ICS guard RB evaluation for OOK-1

b) ICS guard RB evaluation for OOK-2

c) ICS guard RB evaluation for OOK-4

d) ICS guard RB evaluation for FSK-2

e) ICS guard RB evaluation for FSK-4

Figure 7.1.2.3-3: ICS guard RB evaluation

Less guard RB is required for ICS compared to ACS between LP-WUS and NR signals. Considering frequency offset impact, guard RB with the size of one 15kHz SCS RB could provide necessary protection of LP-WUS from interference of adjacent NR sub-carriers.

Company 3 (R4-2313476)

For ASCS, it can be observed that 1st order filter performs worst while and even with adding 6 RB, the link performance only slightly improves., This means that guard RB does not considerably help if filter order is so low in WUR implementation. For higher then 2nd and 3nd order, the needed Gard RB is not obvious. In simulated link performance vs # RB for 3nd order filter in Figure 7.1.2.3-4, the SNR improvement for 2 RB is 0.2 dB.

Figure 7.1.2.3-4: Link performance for ASCS selectivity with different filter order

If the filter order is higher than and equal to 2, smaller guard RB (1 to 2 RB) is sufficient between adjacent subcarrier of eMBB signal and a WUS signal. If the filter order is too low, the guard RB does not help to improve the ASCS selectivity.

Summary for ASCS guard RB analysis:

Based on the analysis above, RAN4 suggested that for 5th order filter, the guard RB number for LP-WUS ASCS is in the range of 0.5RB ~ 2RBs for 30KHz SCS, or 1RBs ~4RBs for 15KHz SCS.

#### 7.1.2.4	Phase noise

Editor’s note: reserved

#### 7.1.2.5	gNodeB RF impacts

Editor’s note: reserved

#### 7.1.2.6	Multi-band capability

RF envelop detection architecture is more appropriate for single-band operation. IF envelop detection and BB envelop detection are more appropriate for multi-bands operation. Multi-band here still means that only one band at a time is being received. Multi-band capability for other architectures is FFS.

## 7.2	LP-WUS design and L1 procedure