3GPP TSG-RAN WG4 Meeting#108bis					R4-2316701

9th Oct, 2023 – 13th Oct, 2023

Source: 	Qualcomm Incorporated

Title: 	NR MIMO: Views on UE demodulation performance and CSI requirements

Agenda item:	 5.29.4.1

Document for:	Discussion

# 1 Introduction

In this paper, our views on NR MIMO evolution [1] are discussed. Specifically, we discuss scope of performance requirements for the topic - increased number of DMRS ports.

# 2 General aspects

In the broad scope of MIMO evolution for DL and DL, Type-II doppler codebook for high velocity UEs, and Type-II codebook for CJT were studied in RAN1 along with increased number of DMRS ports. We are open to discuss on the Type-II codebook enhancements but are skeptical on realizable gains in field. Therefore, we prefer to prioritize discussion on increased number of DMRS orthogonal ports.

Proposal1: Prioritize discussion on increased number of DMRS orthogonal ports.

# 3 Increased DMRS Orthogonal Ports

## 3.1 PDSCH demodulation performance requirements

The number of REs that can be multiplexed in frequency domain is increased to 4 (FD-OCC) [1], and consequently 8 Tx ports can be supported with a single symbol.

Obsevation1: With higher FD-OCC, the PDSCH performance is more sensitive to the channel variations in frequency domain.

Proposal2: RAN4 to consider low delay spread channels such as TDLA30 for PDSCH demodulation performance requirements.

Single symbol front loaded Type 1 DMRS pattern is sufficient to capture the performance impact with 4-FDD OCC and has lower overhead to capture throughput gains. Moreover, Type1 has been considered in legacy spec. for PDSCH demodulation performance.

## Proposal3: RAN4 to consider single symbol front loaded Type 1 DMRS configuration for PDSCH demodulation performance requirements.

## 3.2 CSF requirements

Since the feature only impacts the DMRS pattern, and has no change in CSI-RS, new CSF requirements are not required.

Proposal4: RAN4 shall not have new CSF requirements.

# 5. Conclusions

Following is the summary of the proposals:

Proposal1: Prioritize discussion on increased number of DMRS orthogonal ports.

Proposal2: RAN4 to consider low delay spread channels such as TDLA30 for PDSCH demodulation performance requirements.

Proposal3: RAN4 to consider single symbol front loaded Type 1 DMRS configuration for PDSCH demodulation performance requirements.

Proposal4: RAN4 shall not have new CSF requirements.

# References

- 3GPP TSG RAN Meeting #94e, RP-213598 “New WID: MIMO Evolution for Downlink and Uplink”, Samsung, Electronic Meeting, Dec. 6 - 17, 2021.