3GPP TSG-RAN WG4 Meeting #108bis	R4-2316724

# Xiamen, China, October 9 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial Simulation Results for RedCap Positioning with Frequency Hopping

Agenda item:	5.22.2.4

Document for:	Information

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including:

- Support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning.

NR RedCap positioning has been studied in the FS_NR_pos_enh2 study item by RAN1 with company contributions and outcome of the study being captured in TR 38.859 [2]. In RAN4#108 [3], the link simulation assumptions for RedCap positioning with FH are agreed. The agreed parameters for link level simulations are captured in Table 1 and Table 2 in [4]. Also, the agreed performance metrics for RSTD and UE Rx-Tx measurements are provided in Section 3 [4].

In this contribution we list the simulation assumptions and provide the initial simulation results for RedCap positioning with FH. It is noted that these results do not contain any RF/BB impairments, and thus PRB overlapping is not considered between FHs.

Simulation Assumptions

Simulation assumptions for RedCap positioning with FH captured in [4] are considered. The PRS configurations as listed in Table 1 were used in the simulation study.

| Carrier frequency | SCS (kHz) | PRS BW in RB | sampling rate (Tc) | Num Symbols | Comb Size | Repetition Factor (Rep) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 GHz | 15 | 272 | 32 | 4 | 4 | 6 |
| 4 GHz | 30 | 272 | 16 | 4 | 4 | 6 |
| 4 GHz | 60 | 132 | 16 | 4 | 4 | 6 |

Table 1: PRS configurations used for RSTD study for SCS = 15kHz, 30kHz, and 60 kHz.

No RF/BB impairments are included in the simulations.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 5-percentile and 95-percentile CDF of the DL RSTD error in Tc for given Es/Iot for three cells [4].

AWGN Channel

The DL RSTD error in Tc for AWGN channel, derived from the 5%-ile and 95%-ile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB and (-3, -6, -6) dB are listed in Table 2 and Table 3, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | -32 | 32 | -32 | 32 |
|  |  |  |  | 6 | 4 | -16 | 16 | -16 | 16 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -16 | 16 | -16 | -16 |
|  |  |  |  | 6 | 4 | -8 | 8 | 8 | 8 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -16 | 16 | -16 | 16 |
|  |  |  |  | 6 | 4 | -8 | 8 | -12 | 8 |

Table 2: RSTD error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | -32 | 32 | -32 | 32 |
|  |  |  |  | 6 | 4 | -16 | 16 | -16 | 16 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -16 | 16 | -16 | -16 |
|  |  |  |  | 6 | 4 | -8 | 8 | 8 | 8 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -16 | 16 | -16 | 16 |
|  |  |  |  | 6 | 4 | -8 | 8 | -12 | 8 |

Table 3: RSTD error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

TDL-A Channel

The DL RSTD error in Tc for TDL-A channel, derived from the 5%-ile and 95%-ile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB are listed in Table 4.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | 0 | 32 | -32 | 0 |
|  |  |  |  | 6 | 4 | -16 | 16 | -16 | 8 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -16 | 16 | -16 | 16 |
|  |  |  |  | 6 | 4 | -8 | 8 | -8 | 12 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -16 | 16 |  | 32 |
|  |  |  |  | 6 | 4 |  | 24 |  | 24 |

Table 4: RSTD error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

Discussion

From the simulation results reported above for the AWGN channel without RF/BB impairments, we observed:

- For a given bandwidth, the measurement accuracy improves with the increasing number of measurement samples.

- Increasing the UE total measurement bandwidth after all hops from 50 MHz (SCS 15 kHz) to 100 MHz (SCS 30 and 60 kHz) improves the measurement accuracy by ≈ 50%.

- Es/IoT of (-6, -13, -13) yields the same results as (-3, -6, -6) indicating that, based on the current simulation setup, the Es/IoT of (-6, -13, -13) is still enough to successfully detect the first path.

From the simulation results reported above for the TDL-A channel, we observed:

- Increasing the UE total measurement bandwidth after all hops from 50 MHz (SCS 15 kHz) to 100 MHz (SCS 30 and 60 kHz) generally improves the measurement accuracy. However, that is not the always the case with all the simulated scenarios.

- Increasing the sub-carrier spacing to 60 kHz resulted in a substantial increase in the measurement errors with the low Es/IoT (i.e., noise limited) case.

Thus, following observations are made:

The simulation results show that the performance changes depending on the number of measurement samples in all cases of the different SNRs with AWGN and TDL-A channel models, respectively.

Using 4 measurement samples should be the baseline to define the performance requirements. Single measurement sample still needs further investigation.

Conclusion

In this contribution we have provided first simulation results for DL RSCPD error without RF/BB impairments based on simulation assumptions in [4].

Following observations are made.

- The simulation results show that the performance changes depending on the number of measurement samples in all cases of the different SNRs with AWGN and TDL-A channel models, respectively.

- Using 4 measurement samples should be the baseline to define the performance requirements. Single measurement sample still needs further investigation.

- References

| [1] | RP-232670, Revised WID on Expanded and Improved NR positioning, 3GPP TSG RAN#101 |
| --- | --- |
| [2] | TR 38.859, "Study on Expanded and Improved NR Positioning (Release 18)," V18.0.0 |
| [3] | R4-2314353, "WF on R18 NR positioning – RedCap positioning and PRS/SRS BW aggregation," Ericsson |
| [4] | R4-2314460, " Simulation assumptions for RedCap positioning with FH," Ericsson |