3GPP TSG-RAN WG4 Meeting #108bis 	R4-2316727

Xiamen, China, October 9 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Core Requirements for Enhanced NR Sidelink Relay

Agenda item:	5.32.1

Document for:	Discussion

Introduction

The agreements in RAN4#108 are summarized in WF [1].

## Issue 1-1: Interruption time in multipath scenario - Agreements - Clarify interruption requirements caused by the remote UE on its serving cell in TS 38.133 ## Issue 1-2: Selection/reselection of relay UE in UE-to-UE relay scenario - Agreements - Applicability of requirements on selection/reselection of relay UE by the remote UE in U2U relay scenario should be explicitly captured in clause 12.10 of TS 38.133.

Discussion

We discuss the RRM requirements for SL relay. Issues mentioned in WF [1], we would like to bring following consideration into discussions on the interruption time in a multiple scenario.

## Issue 1-1: Interruption time in multipath scenario - Agreements - Clarify interruption requirements caused by the remote UE on its serving cell in TS 38.133

TS 38.133, section 12.7 captures V2X requirements related to interruptions. RAN2 has discussed that among the specified requirements the interruptions on the PCell/serving cell ‘Interruptions to WAN at transitions between active and non-active during SL-DRX’, specified in clause 12.7.4, and ‘Interruptions at NR sidelink discovery configuration’, specified in clause 12.7.8 need to be independently considered for multi-path relay.

If any scenario(s) need specific mentioning of applicability of legacy RRM requirements, those scenarios could be explicitly captured in specification. These mentioned RRM requirements are generic requirements related to SL relay operation and are the same whether a SL relayed path is part of sidelink multi-path or is the only path between the UE and gNB, hence, implicitly applicable to SL relay in multi-path scenario as well.

Current specifications on interruption requirements caused by the remote UE on its serving cell in TS 38.133 are applicable to multipath without adding a new section.

RAN4 not to add a new section to clarify interruption requirements caused by the remote UE on serving cell in multipath scenario.

Conclusions

The proposals made in this paper are as follows:

- Current specifications on interruption requirements caused by the remote UE on its serving cell in TS 38.133 are applicable to multipath without adding a new section.

RAN4 not to add a new section to clarify interruption requirements caused by the remote UE on serving cell in multipath scenario.

- References

- R4-2314352, WF on SL Relay RRM Requirements, LG Electronics, 3GPP TSG-RAN WG4 Meeting #108