3GPP TSG-RAN WG4 Meeting #108bis	R4-2316740

# Xiamen, China, October 9 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Applicability of Pre-MG for RedCap UE

Agenda item:	7.2.1

Document for:	Discussion

# 1 	Introduction

The discussion on the applicability of Pre-MG for RedCap UE was triggered at RAN4 #108 by the incoming RAN3 LS [1]. RAN4 #108 did not achieve consensus on the issue [2]. In this contribution, we provide our view on this issue.

# 2 	Discussion

## 2.1 RAN4 agreements and open issues

RAN4 received an LS from RAN3 at RAN4#108 [1], which was not replied at RAN4 #108, as no consensus was achieved and further the LS was sent to RAN2 and only in copy to RAN4. The LS is replicated below.

1. Overall Description: During Release 17 RedCap maintenance discussion, RAN3 has discussed the topic of RedCap UE configured with BWP specific Serving Cell Measurement Object (ServingCellMO) support over F1. To support the scenario of DCI based BWP switching scenario for a RedCap UE, based on the introduction of preConfGapStatus-r17 IE in BWP-DownlinkDedicated IE in TS 38.331, RAN3 is discussing how the gNB-CU can request the gNB-DU to provide configuration of all Pre-Configured Measurement Gaps (Pre-MG) that can be used after the active BWP is switched via DCI for the RedCap UE that supports this capability. However, based on “gapToAddModList IE” definition in MeasGapConfig, RAN3 would like to ask RAN2 whether it is possible that this IE can be used to configure different Pre-MGs for different BWPs, since the RedCap UE will require a MG whenever it performs a measurement outside its active BWP. 2. Actions: To RAN2: ACTION: RAN3 kindly asks RAN2 to feedback on the above questions.

At RAN4#108, the following was agreed [2]:

Topic #2: Applicability of pre-configured measurement gaps for RedCap UE Issue 2-1-2: Whether Pre-MG is supported in RAN4 RedCap requirements？ Agreement: No RAN4 RRM requirements are specified for Rel-17 RedCap with Rel-17 pre-MG. Conclusion: Do not send LS

As RAN4 agreed no requirements for Pre-MG support of RedCap UE exist, there would be considerable specification impact for Rel-17 including definition of test cases for Pre-MG activation / deactivation for Redcap BWP switching. Given the late phase of Rel-17, we propose to abandon this enhancement for Rel-17.

The applicability of Pre-MG for RedCap UE was not discussed in RAN4 so far and no requirements exist in RAN4 specifications.

RAN4 not to specify applicability of Pre-MG for RedCap in Rel-17.

However, the discussion remains for eRedCap for Rel-18. In our view, applicability should be added to eRedCap to increase its commercial benefit. As RRM core requirements for Rel-18 are still to be introduced, the addition of Pre-MG support to eRedCap is well suited now.

RAN4 to consider applicability of Pre-MG for eRedCap in Rel-18.

One alternative to the proposed Rel-18 addition is to define the Pre-MG support as release-independent feature from Rel-17. The advantage is that the RedCap Rel-17 UE can benefit from this as well, but the drawback will be somewhat higher complexity and the delayed introduction of this feature, as RedCap is deemed to be in the implementation phase.

RAN4 to consider applicability of Pre-MG for RedCap as release-independent feature from Rel-17.

In our view, RAN4 should discuss above options and provide feedback to RAN2, in that RAN2 can take into account RAN4’s view when replying to RAN3.

RAN4 to send Reply LS to RAN2 informing RAN2 on the preferred solution out of 1) addition to Rel-18 eRedCap WI scope or 2) addition as release-independent feature to RedCap from Rel-17.

# 3	Conclusions

In this document, Nokia’s view regarding the issue on the applicability of Pre-MG for RedCap UE as raised by RAN3 [1] is provided.

The following observation and proposals are made:

- The applicability of Pre-MG for RedCap UE was not discussed in RAN4 so far and no requirements exist in RAN4 specifications.

- RAN4 not to specify applicability of Pre-MG for RedCap in Rel-17.

RAN4 to consider applicability of Pre-MG for eRedCap in Rel-18.

RAN4 to consider applicability of Pre-MG for RedCap as release-independent feature from Rel-17.

RAN4 to send Reply LS to RAN2 informing RAN2 on the preferred solution out of 1) addition to Rel-18 eRedCap WI scope or 2) addition as release-independent feature to RedCap from Rel-17.

References

- R3-233478, “LS on applicability of pre-configured measurement gaps for RedCap UE”, RAN3

- RAN4 #108 RRM session meeting report, version 09 EOM2, RAN4 Vice Chair (Intel)