3GPP TSG-RAN WG4 Meeting #108bis	R4-2316754

# Xiamen, China, October 9 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for NR Carrier Phase Positioning

Agenda item:	5.22.2.6

Document for:	Discussion

Introduction

The specification of NR carrier phase positioning (NR CPP) is one of the objectives in the Rel-18 WID on expanded and improved positioning [1].

This contribution summarizes RAN4 #108 agreements and lists discussion on some of the open issues with regard to impacts to RRM core requirements for NR CPP. RAN1 #114 agreements are listed in Annex.

Discussion

RAN4 agreements and open issues

RAN4 agreements and list of open issues from RAN4 #108 in WF [2] are captured below. More details are available in [3].

## Sub-topic 2-1 Measurement period requirements #### Issue 2-1-1: PRS measurement period requirements for DL RSCP/DL RSCPD: Agreements: - RSCP/RSCPD is reported together with other positioning measurement(s) and the same measurement period requirement (i.e., Rx-Tx time difference/RSTD measurements) should apply. - FFS: whether the existing Rx-Tx time difference/RSTD measurement periods apply - Issue 2-1-2: Impact of UE mobility in RRC_CONNECTED: Agreements - Reuse Rel. 17 requirements for mobility impact on RSTD/UE Rx-Tx measurement in RRC_CONNECTED / RRC_INACTIVE state for RSCPD/RSCP measurements - Issue 2-1-3: Impact of collisions with other signals/channels in RRC_INACTIVE: Agreements - The collision handling between PRS resources and other DL signals/channels for positioning measurement in RRC_INACTIVE state in Rel-17 is reused for carrier phase measurement in RRC_INACTIVE state.

## Sub-topic 2-2 CPP measurement reporting - Issue 2-2-1: PRS measurement reporting requirements for DL carrier phase measurement: Agreements - RSCP/RSCPD is reported together with other positioning measurement(s) and the same measurement reporting delay requirement (i.e., Rx-Tx time difference/RSTD measurements) should apply. - Issue 2-2-2: Reporting range for DL RSCPD: Agreements: - The reporting range of DL RSCDP is [-180, 180) degrees. - Issue 2-2-3: Reporting granularity for DL RSCPD/RSCP: Agreements: - The reporting granularity for DL RSCPD/RSCP is 0.1 degree. - Issue 2-2-4: Integer information reporting: Agreements: - Not to pursue the issue. - Issue 2-2-6: Reporting requirements for UL carrier phase measurement: Agreements: - Not to pursue the issue.

Following open issues were identified in RAN4#108bis [2].

## Sub-topic 2-1 Measurement period requirements - Issue 2-1-4: CPP measurement in RRC_IDLE state: Way Forward: - No consensus. The proposals are listed in [1]. ## Sub-topic 2-2 CPP measurement reporting - Issue 2-2-5: Solutions to reduce the impact of carrier frequency offset: Way Forward: - Was not discussed in RAN4#108. The proposals are listed in [1]. ## Sub-topic 2-3 Accuracy requirements - Issue 2-3-1: Accuracy requirements for DL carrier phase measurement: Way Forward: - Was not discussed in RAN4#108. The proposals are listed in [1]. - Issue 2-3-2: Accuracy requirements for UL carrier phase measurement: Way Forward: - Was not discussed in RAN4#108. The proposals are listed in [1].

  - Measurement period requirements in RRC_CONNECTED

Time window configuration for DL CPP

RAN1 #114 achieved following agreements on the time window configuration for DL CPP (see Annex):

Agreement When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8}

Figure 1 illustrates the time windows according to RAN1 #113 and RAN1 #114 agreements (see Annex 1).

Figure 1: Time windows configuration for DL CPP (each window covers a duration of multiple slots/symbols including PRS resource repetition)

RAN1 has agreed how to configure start and duration of the time window for DL CPP in terms of number of consecutive slots, some further refinement on the number of time windows is still subject to agreement. LMF will configure target UE and PRU(s) to perform NR CPP DL measurement in a sequence of time windows for DL CPP, which, in case of RRC_CONNECTED state, are overlapped with measurement gap occasions.

Thus, for time windows with up to 6 slots, GP#0 or GP#1 are suitable, alternatively for time windows with up to 3 slots shorter MGs with MGL=3ms or up to 4 slots MGs with MGL= 4ms, in case of per-UE measurement gaps.  For longer time windows, use of measurement gaps for positioning use case is foreseen, i.e. for 8 slots, GP #24 with MGL=10ms is needed and for 12 and 16 slots GP #25 with MGL=20ms. Suitable gap patterns (GP) for per-UE MG or per-FR MG in FR1 are depicted in Table 1, for per-FR MG in FR2 in Table 2.

| Time window length | Suitable GP | MGRP |
| --- | --- | --- |
| 1ms 2ms | #2 (MCL=3ms), #3 (MCL=3ms) #10 (MCL=3ms), #11 (MCL=3ms) #6 (MCL=4ms), #7 (MCL=4 ms) #8 (MCL=4ms), #9 (MCL=4 ms) #0 (MCL=6ms), #1 (MCL=6ms) #4 (MCL=6ms), #5 (MCL=6ms) #24 (MCL=10ms) #25 (MCL=20ms) | 40ms, 80ms 20ms, 160ms 20ms, 40ms 80ms, 160ms 40ms, 80ms 20ms, 160ms 80ms 160ms |
| 4ms | #0 (MCL=6ms), #1 (MCL=6ms) #4 (MCL=6ms), #5 (MCL=6ms) #24 (MCL=10ms) #25 (MCL=20ms) | 40ms, 80ms 20ms, 160ms 80ms 160ms |
| 6ms | #24 (MCL=10ms) #25 (MCL=20ms) | 80ms 160ms |
| 8ms | #24 (MCL=10ms) #25 (MCL=20ms) | 80ms 160ms |
| 12ms | #25 (MCL=20ms) | 160ms |
| 16ms | #25 (MCL=20ms) | 160ms |

Table 1: Time window support in case of configured per-UE MG or per-FR MG in FR1.

| Time window length | Suitable GP | MGRP |
| --- | --- | --- |
| 1ms | #20 (MCL=1.5ms), #21 (MCL=1.5ms) #22 (MCL=1.5ms), #23 (MCL=1.5ms) #16 (MCL=3.5ms), #17 (MCL=3.5ms) #18 (MCL=3.5ms), #19 (MCL=3.5ms) #12 (MCL=5.5ms), #13 (MCL=5.5ms) #14 (MCL=5.5ms), #15 (MCL=5.5ms) #24 (MCL=10ms) #25 (MCL=20ms) | 20ms, 40ms 80ms, 160ms 20ms, 40ms 80ms, 160ms 20ms, 40ms 80ms, 160ms 80ms 160ms |
| 2ms | #16 (MCL=3.5ms), #17 (MCL=3.5ms) #18 (MCL=3.5ms), #19 (MCL=3.5ms) #12 (MCL=5.5ms), #13 (MCL=5.5ms) #14 (MCL=5.5ms), #15 (MCL=5.5ms) #24 (MCL=10ms) #25 (MCL=20ms) | 20ms, 40ms 80ms, 160ms 20ms, 40ms 80ms, 160ms 80ms 160ms |
| 4ms | #12 (MCL=5.5ms), #13 (MCL=5.5ms) #14 (MCL=5.5ms), #15 (MCL=5.5ms) #24 (MCL=10ms) #25 (MCL=20ms) | 20ms, 40ms 80ms, 160ms 80ms 160ms |

Table 2: Time window support in case of configured per-FR MG in FR2.

For each duration of the time window, RAN4 to define appropriate measurement gap patterns for per-UE MG and per-FR MG according to Table 1 and Table 2.

The sequence of time windows for DL CPP needs to have full overlapping with measurement gap occasions in RRC_CONNECTED state.

Applicable number of measurement instances

RAN1 #114 achieved following agreement on the applicable number of measurement instances for DL CPP (see Annex):

Agreement Each DL RSCP/RSCPD measurement instance is obtained with Nsample=1 sample only. Conclusion From RAN1’s perspective, the granularity and the range of the RSCP/RSCPD measurements can be defined by RAN4.

The RAN1 agreement means that one RSCP/RSCPD measurement sample is based on one time window and hence one MG occasion. Hence it refers to Rel-17 reduced number of measurement samples procedure for UE Rx-Tx time difference/RSTD with 1 measurement sample. Thus, the measurement period for RSCP/RSCPD should be defined based on Nsample=1.

In addition, the case of different signal strength for serving and neighbour TRPs needs to be considered, where legacy RAN4 measurement period requirements for RSTD, PRS-RSRP and UE Rx-Tx time difference are based on an additional sample for coarse AGC tuning. Hence in case |Ês/Iot| ≥ 6 dB, measurement period requirements are based on N=2 samples for PRS measurements for those neighbour TRPs. The same is expected for carrier phase measurements DL RSCP and RSCPD, respectively.

RAN4 to specify measurement period requirements for DL RSCP and DL RSCPD, for single measurement instance (N=1) and in case of channel condition |Ês/Iot| ≥ 6 dB for N=2 only.

Alignment of CPP measurement period with that for other positioning measurements

Joint reporting of CPP measurements with other existing PRS measurements as well as common measurement period were agreed at RAN4 #108.

#### Issue 2-1-1: PRS measurement period requirements for DL RSCP/DL RSCPD: Agreements: - RSCP/RSCPD is reported together with other positioning measurement(s) and the same measurement period requirement (i.e., Rx-Tx time difference/RSTD measurements) should apply. - FFS: whether the existing Rx-Tx time difference/RSTD measurement periods apply

- The open issue here is whether the common measurement period is based on existing measurements (RSTD or UE Rx-Tx, respectively) or based on the CPP measurements (RSCPD or DL-RSCP, respectively). Based on latest RAN1 #114 agreement, measurement periods are dependent on whether reduced number of measurement samples is configured for UE Rx-Tx time difference or RSTD, respectively. If configured, measurement periods can be aligned and are based on measurement requirements for reduced number of measurement samples as specified for UE Rx-Tx time difference or RSTD, respectively in Rel-17. If not configured, then the measurement periods are not aligned as RSCP/RSCPD are based on single sample, whilst UE Rx-Tx time difference/RSTD on 4 samples as defined for Rel-16.

Regarding the measurement period definition, RAN4 to distinguish two cases:

- If reduced number of measurement samples is configured for other ToA measurements (UE Rx-Tx time difference or RSTD, respectively), measurement periods between CPP and other measurements can be aligned and are based on measurement requirements for reduced number of measurement samples as specified for UE Rx-Tx time difference or RSTD, respectively in Rel-17.

- Else, if not configured, then the measurement periods cannot be aligned as RSCP/RSCPD are based on single sample, whilst UE Rx-Tx time difference/RSTD are based on 4 samples as defined for Rel-16. As an alternative, the UE may report 4 measurements for RCSPD/DL RSCP together with the paired single ToA measurement (RSTD/UE Rx-Tx time difference) or it may report only the latest time interval for RSCPD/DL RSCP, as configured by LMF.

PRS Muting for DL CPP

It is observed that RAN1 has not specifically discussed usage of PRS muting for DL CPP in terms of intercell interference mitigation [4]. Thus, in our understanding RRM requirements for DL CPP should be designed for both cases no PRS muting and PRS muting, respectively. Since the PRS resources which are available for UE according to the PRS muting pattern can be shared for TOA estimation as well as for carrier phase (CP) measurement, there is no further RRM impact due to PRS muting compared to TOA estimation only, other than the configured PRS muting pattern scaling the measurement period.

RAN4 to design DL CPP for both cases no PRS muting and PRS muting, respectively.

Additional measurement type

RAN1 #114 achieved following agreement on the following additional measurement type for DL/UL CPP (see Annex):

Agreement Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: - phase quality index - phase quality resolution The values of the phase quality index and phase quality resolution are left for RAN4.

The same measurement period as for RSCP or RSCPD, respectively, should be selected, so that along the carrier phase information, this additional information on phase quality is derived. RAN4 to await the measurement type definition in TS 38.215.

RAN4 to agree the measurement period is same for RSCP/RSCPD and the phase quality indication.

RAN4 to await the measurement type definition for phase quality indication in TS 38.215.

Time window configuration for UL CPP

RAN1 #114 achieved following agreements on the time window configuration for UL CPP.

Agreement Confirm the following working assumption with modification made in RAN1#113: Working assumption To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16} Agreement When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16}

RAN1 has agreed how to configure start and duration of the time window for UL CPP in terms of number of consecutive slots/symbols, some further refinement may still be subject to agreement. LMF will request serving gNB/TRP to configure UL SRS resources (UL SRS for positioning or MIMO SRS) of the target UE in one or up to 16 configured time windows for UL CPP and will also request serving gNB/TRP and neighbour gNBs/TRPs to receive these UL SRS resources in one or several configured time windows for UL CPP. Hence, the network is aware of target UE’s transmission and will not schedule data for DL or UL to target UE during the configured time windows for UL CPP and hence there is no collision with data in RRC_CONNECTED state.

RAN4 to agree during the configured time windows for UL CPP there is no collision with data in RRC_CONNECTED state.

Impact of UE mobility

RAN4 #107 has not achieved consensus on following issue.

#### Issue 2-1-4: Impact of UE mobility in RRC_CONNECTED: Agreements: FFS: The mobility impact in RRC_CONNECTED in Rel-17 can also be reused.

RAN4 #108 achieved consensus on high level, still the UE mobility impact needs further investigation.

- Issue 2-1-2: Impact of UE mobility in RRC_CONNECTED: Agreements - Reuse Rel. 17 requirements for mobility impact on RSTD/UE Rx-Tx measurement in RRC_CONNECTED / RRC_INACTIVE state for RSCPD/RSCP measurements

The impact on measurement requirements for DL CPP due to mobility procedures should be investigated and specified by RAN4 similar as done for Rel-16/Rel-17 positioning techniques.

For DL CPP, in case of handover, Rel-16 has specified following requirement for RSTD in TS 38.133, clause 9.9.2.5.

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.

In case of configured RSTD and RSCPD, the same measurement behaviour can be applied by the UE for the carrier phase measurement RSCPD as for the paired TOA measurement RSTD, hence the measurement period is extended to include the handover interruption time.

RAN4 to use the Rel-16 measurement behaviour for RSTD in case of handover as baseline for measurement behaviour for RSCPD.

For DL CPP, in case of cell change, Rel-16 has specified following requirement for UE Rx-Tx time difference in TS 38.133, clause 9.9.4.5.

When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

If the network configures DL RSCP together with UE Rx-Tx time difference, upon a serving cell change, DL RSCP of the new serving cell will be random. Thus, DL RSCP and UE Rx-Tx time difference measurements need to be restarted. This corresponds to the case of serving cell change with modified SRS configuration in Rel-16, for which the UE Rx-Tx time difference measurement needs to be restarted.

Thus, RAN4 needs to modify the RRM requirement for serving cell change for DL CPP with configured DL RSCP and UE Rx-Tx time difference.

RAN4 to modify the RRM requirement for serving cell change for DL CPP with configured DL RSCP and UE Rx-Tx time difference such, that both measurements need to be restarted upon serving cell change.

In addition, RRM impacts due to SRS configuration change and UL Tx timing change due to TA adjustment or UE autonomous timing adjustment need to be investigated by RAN4. Regarding UL Tx timing change, UE should keep using fixed Tx timing although the DL reception reference timing is changed in order to guarantee the measurement accuracy for UL RSCP.

RAN4 to investigate RRM impacts due to SRS configuration change and UL transmission timing change due to TA adjustment or UE autonomous timing adjustment.

RAN4 to investigate whether UE can keep using fixed Tx timing although the DL reception reference timing is changed in order to guarantee the measurement accuracy for UL RSCP.

Measurement period requirements in RRC_INACTIVE

Impact of collisions with other signals/channels in RRC_INACTIVE

RAN4 #107 has not achieved consensus on following issue.

- Issue 2-1-5: Impact of collisions with other signals/channels in RRC_INACTIVE: Agreements: - FFS: The collision handling in RRC_INACTIVE state in Rel-17 can also be reused.

It is noted that the RAN4 #106bis-e agreement on this matter was:

- RAN4 to investigate and specify RRM impacts from NR DL CPP in RRC_INACTIVE such as collisions with other signals/DL channels, assuming Rel-17 requirements can be re-used.

Reviewing TS 38.133, clause 5.6.1 specifies the collision handling for PRS measurements and other DL signals/channels, as replicated below.

The requirements in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 are applicable to PRS resources that do not collide with other DL signals/channels which include SSB, SIB1, CORESET0, MSG2/MSGB, paging and DL SDT. In addition, a UE is not expected to receive PRS resources that collide with a time interval starting at symbol m and ending at symbol m + N2, where symbol m is the last symbol in which the UE is configured to receive PDCCH and N2 is defined in clause 6.4 of [26, TS 38.214] for the subcarrier spacing μ of the DL PRS. If a PRS resource is outside the intitial DL BWP, a PRS resource instance collides with another DL signals/channels if any portion of the other DL signal/channel overlaps with the time interval starting X symbols before the PRS instance and ending X symbols after the PRS instance, taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD. Where X is defined in Table 5.6.1-1. Table 5.6.1-1: Value of X number of symbols FR; NR Slot length (ms); X symbols. FR1; 0; 1; 7. 1; 0.5; 14. 2; 0.25; 28. FR2; 2; 0.25; 14. 3; 0.125; 28. Note 1: The FR1 value applies if one or both of the serving cell and the positioning frequency layer are in FR1. FR2 value applies both of the serving cell and the positioning frequency layer are in FR2..

For RSTD and PRS-RSRP, it is specified that longer measurement period applies in case of collisions:

- Clause 5.6.2.5: “Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.”

- Clause 5.6.2.5: “Longer PRS-RSRP measurement period is expected when there is collision/overlap between other DL signals/channels and PRS resources in RRC_INACTIVE state.”

For DL CPP, similar collision requirements are applicable, as the PRS resources are those being overlapped with the configured time window(s). When either of these PRS resources collides with other DL signals/channels such as SSB, SIB1, CORESET0, MSG2/MSGB, paging and DL SDT, the UE behaviour needs to be clarified in the requirements.

Thus, RAN4 needs to consider the case of collision of PRS resources in configured time window(s) with DL signals/channels, for defining the measurement behaviour for DL CPP.

RAN4 to consider the case of collision of PRS resources in configured time window(s) with DL signals/channels, such as SSB, SIB1, CORESET0, MSG2/MSGB, paging and DL SDT for defining the measurement behaviour for DL CPP in RRC_INACTIVE state.

Measurement reporting requirements

RAN4 #108 discussed different aspects for measurement reporting, as listed below.

Joint reporting of DL CPP and legacy DL positioning measurements

RAN4 #108 has discussed the scenario of coexistent NR CPP and legacy NR positioning techniques and identified the following agreement on joint measurement reporting:

- Issue 2-2-1: PRS measurement reporting requirements for DL carrier phase measurement: Agreements - RSCP/RSCPD is reported together with other positioning measurement(s) and the same measurement reporting delay requirement (i.e., Rx-Tx time difference/RSTD measurements) should apply.

As described for the measurement period requirements in section 2.2.3, an alignment of measurement periods is possible in case of reduced measurement samples (Nsample = 1), whilst not possible in case of normal measurement samples (Nsample = 4). Thus, if reduced measurement samples is configured for UE Rx-Tx time difference or RSTD, respectively, common measurement reporting can be performed after each measurement sample. Else, if not configured, for UE Rx-Tx time difference or RSTD, respectively, measurement reporting can be performed after 4 measurement samples. In this case, measurement results of multiple time windows for CPP can be reported, in case several time windows are covered by the normal measurement period.

Regarding the measurement reporting for DL PRS, RAN4 to distinguish two cases:

- If reduced number of measurement samples is configured for ToA measurements (UE Rx-Tx time difference or RSRD, respectively), measurement reporting of CPP and other measurements can be done together after each measurement sample. Reporting delay requirements for reduced number of samples in Rel-17 apply.

- Else, if not configured, then the measurement reporting is based on normal measurement samples for ToA measurements (UE Rx-Tx time difference or RSTD, respectively) and UE can report measurement results from multiple time windows covered by the normal measurement period. Reporting delay requirements for normal number of samples in Rel-16 apply.

Joint reporting of UL CPP and legacy UL positioning measurements

RAN1 #112bis-e achieved following agreement on joint reporting of NR CPP with legacy NR positioning techniques.

Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF

RAN4 #106bis-e has discussed related issue and identified the following agreement:

#### Issue 2-1-2: Measurement types to be defined for UL carrier phase measurement requirements: Agreements: - RAN4 not to define measurement period requirements for UL RSCP measurement and FFS for accuracy requirements.

This earlier agreement was confirmed at RAN4 #108.

- Issue 2-2-6: Reporting requirements for UL carrier phase measurement: Agreements: Not to pursue the issue.

Thus, no measurement period requirements will be specified for UL RCSP. Whether being jointly reported with legacy UL positioning measurements, i.e. together with RTOA and/or gNB Rx-Tx time difference, can be left implementation specific, however according RAN1 agreement NRPPa needs to support joint measurement reporting, thus NRPPa signalling needs to be extended by the UL RSCP measurement type.

Whether network supports joint measurement reporting of NR CPP UL measurements (i.e. UL RSCP) and legacy UL positioning measurements, is left implementation specific, however NRPPa needs to support it.

Solutions to reduce the impact of carrier frequency offset

#### At RAN4 #108, solutions to reduce the carrier frequency offset were agreed to be further studied.

- Issue 2-2-5: Solutions to reduce the impact of carrier frequency offset: Way Forward: - Was not discussed in RAN4#108. The proposals are listed in [1].

#### At RAN4 #108, a proposal was made in [4] that target UE and PRU both assess the carrier frequency offset (CFO) by measuring the phase difference between subsequent DL PRS symbols in time transmitted by the same TRP. The CFO estimate should be performed for each TRP and then reported to LMF with the used separation in symbols between any two TRPs.

#### In our view, RAN4 should continue the study of proposals to mitigate the carrier frequency offset.

RAN4 to continue the investigation on solutions reducing the carrier frequency offset.

  - Measurement accuracy requirements

Targeted PRS bandwidth range for accuracy requirements

As NR CPP will use MG assisted measurements performed on a single PFL/carrier, the targeted PRS BW range for NR DL CPP in FR1 and FR2 should be aligned to Rel-16 for RRC_CONNECTED and to Rel-17 for RRC_INACTIVE. Previously, it has already been agreed that the measurement period requirements (formula) for DL measurements are agnostic to the PRS BW. Hence the same PRS BW range as for Rel-16 and Rel-17 should be considered for BW related RRM performance requirements for DL CPP in TS 38.133.

- RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to BW related RRM performance requirements.

Channel models for CPP measurement accuracy requirements

At RAN4 #107, agreement on following channel models for NR CPP was achieved:

- Channel model: - AWGN - TDL-A (30 ns delay spread, 5Hz Doppler spread) - 2-tap model (same as for PRS-RSRPP)

While we support AWGN and the 2-tap-model, we see some issues with TDL-A channel model.

First, TDL-A is an NLOS channel model, hence not suited for NR CPP, which focus on LOS or ALOS channels. Second, RAN1 had agreement all NR CPP measurements are path related [5].

- For AWGN, the carrier phase of the static channel needs to be determined.

- For the 2-tap-channel model, the carrier phase of the first path needs to be determined.

- For TDL-A, the carrier phase is random for each path (Rayleigh fading applies for the tap coefficient) and independent between different channel realizations.

Hence for TDL-A, there is the difficulty how to define the carrier phase given the above. Thus, RAN4 would need to achieve consensus how to determine the carrier phase for TDL-A channel model. Until then, for the link level simulation assumptions [6] we propose to use AWGN and 2-tap-channel model only.

Also, at RAN4 #108, most companies contributed results for AWGN and the 2-tap-channel model [7]. Thus, we propose to specify NR CPP performance requirements for AWGN and the 2-tap-channel model for the fading channel.

RAN4 to specify NR CPP performance requirements for AWGN and the 2-tap-channel model for the fading channel.

For AWGN channel model, we provide revised simulation results in [8]. These results show, when comparing carrier phase accuracy for 1 measurement sample against 4 measurement samples, the RSCPD error in the 90 percentile is roughly halved. Thus, we propose to further continue the investigation of 4 measurement samples performance, although RAN1 agreed to restrict CP measurements to 1 measurement sample.

RAN4 to continue the evaluation of 4 measurement samples performance aside 1 measurement sample performance, and thereafter provide feedback to RAN1, as RAN1 has restricted CP measurements to 1 measurement sample.

SINR side conditions for DL RSCP

At RAN#107, SINR side conditions for RSCPD were agreed as follows [9]:

| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] in FR1 and FR2 | For RSCPD: (-6, -13, -13) dB for NSample = 4; (-3, -6, -6) dB for NSample = 1. |
| --- | --- |

This is aligned to simulated side conditions for RSTD and PRS-RSRP [10] and to Rel-16/Rel-17 requirements in TS 38.133:

| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] in FR1 and FR2 | (-6, -13, -13) dB for NSample = 4; (-3, -6, -6) dB for NSample = 1 |
| --- | --- |

Thereagainst, side conditions for DL RSCP were left open and were also not addressed at RAN4 #108.

Considering simulated side conditions for UE Rx-Tx time difference [11] and Rel-16/Rel-17 requirements in TS 38.133:

- Es/Iot = -3dB for serving cell and Es/Iot = -13dB for neighbor cell for NSample = 4 in FR1 and FR2, and

- Es/Iot = 0dB for serving cell and Es/Iot = -6 dB for neighbor cell for NSample = 1 in FR1 and FR2,

We propose to align side conditions for DL RSCP simulations with 3 cells as follows:

| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-3, -13, -13) dB for NSample = 4; (0, -6, -6) dB for NSample = 1; | (-3, -13, -13) dB for NSample = 4; (0, -6, -6) dB for NSample = 1; |
| --- | --- | --- |

RAN4 to specify the same SINR side condition for the NR carrier phase measurement DL RSCP as for UE Rx-Tx time difference for Rel-16/Rel-17 positioning, i.e.

| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-3, -13, -13) dB for NSample = 4; (0, -6, -6) dB for NSample = 1; | (-3, -13, -13) dB for NSample = 4; (0, -6, -6) dB for NSample = 1; |
| --- | --- | --- |

Conclusion

RRM impacts for NR carrier phase positioning (NR CPP) being one of the objectives in the Rel-18 WID on expanded and improved positioning [1], are investigated in this contribution.

The following proposals are made.

Measurement period requirements in RRC_CONNECTED

- For each duration of the time window, RAN4 to define appropriate measurement gap patterns for per-UE MG and per-FR MG according to Table 1 and Table 2.

| Time window length | Suitable GP | MGRP |
| --- | --- | --- |
| 1ms 2ms | #2 (MCL=3ms), #3 (MCL=3ms) #10 (MCL=3ms), #11 (MCL=3ms) #6 (MCL=4ms), #7 (MCL=4 ms) #8 (MCL=4ms), #9 (MCL=4 ms) #0 (MCL=6ms), #1 (MCL=6ms) #4 (MCL=6ms), #5 (MCL=6ms) #24 (MCL=10ms) #25 (MCL=20ms) | 40ms, 80ms 20ms, 160ms 20ms, 40ms 80ms, 160ms 40ms, 80ms 20ms, 160ms 80ms 160ms |
| 4ms | #0 (MCL=6ms), #1 (MCL=6ms) #4 (MCL=6ms), #5 (MCL=6ms) #24 (MCL=10ms) #25 (MCL=20ms) | 40ms, 80ms 20ms, 160ms 80ms 160ms |
| 6ms | #24 (MCL=10ms) #25 (MCL=20ms) | 80ms 160ms |
| 8ms | #24 (MCL=10ms) #25 (MCL=20ms) | 80ms 160ms |
| 12ms | #25 (MCL=20ms) | 160ms |
| 16ms | #25 (MCL=20ms) | 160ms |

Table 1: Time window support in case of configured per-UE MG or per-FR MG in FR1.

| Time window length | Suitable GP | MGRP |
| --- | --- | --- |
| 1ms | #20 (MCL=1.5ms), #21 (MCL=1.5ms) #22 (MCL=1.5ms), #23 (MCL=1.5ms) #16 (MCL=3.5ms), #17 (MCL=3.5ms) #18 (MCL=3.5ms), #19 (MCL=3.5ms) #12 (MCL=5.5ms), #13 (MCL=5.5ms) #14 (MCL=5.5ms), #15 (MCL=5.5ms) #24 (MCL=10ms) #25 (MCL=20ms) | 20ms, 40ms 80ms, 160ms 20ms, 40ms 80ms, 160ms 20ms, 40ms 80ms, 160ms 80ms 160ms |
| 2ms | #16 (MCL=3.5ms), #17 (MCL=3.5ms) #18 (MCL=3.5ms), #19 (MCL=3.5ms) #12 (MCL=5.5ms), #13 (MCL=5.5ms) #14 (MCL=5.5ms), #15 (MCL=5.5ms) #24 (MCL=10ms) #25 (MCL=20ms) | 20ms, 40ms 80ms, 160ms 20ms, 40ms 80ms, 160ms 80ms 160ms |
| 4ms | #12 (MCL=5.5ms), #13 (MCL=5.5ms) #14 (MCL=5.5ms), #15 (MCL=5.5ms) #24 (MCL=10ms) #25 (MCL=20ms) | 20ms, 40ms 80ms, 160ms 80ms 160ms |

Table 2: Time window support in case of configured per-FR MG in FR2.

The sequence of time windows for DL CPP needs to have full overlapping with measurement gap occasions in RRC_CONNECTED state.

RAN4 to specify measurement period requirements for DL RSCP and DL RSCPD, for single measurement instance (N=1) and in case of channel condition |Ês/Iot| ≥ 6 dB for N=2 only.

Regarding the measurement period definition, RAN4 to distinguish two cases:

- If reduced number of measurement samples is configured for other ToA measurements (UE Rx-Tx time difference or RSTD, respectively), measurement periods between CPP and other measurements can be aligned and are based on measurement requirements for reduced number of measurement samples as specified for UE Rx-Tx time difference or RSTD, respectively in Rel-17.

- Else, if not configured, then the measurement periods cannot be aligned as RSCP/RSCPD are based on single sample, whilst UE Rx-Tx time difference/RSTD are based on 4 samples as defined for Rel-16. As an alternative, the UE may report 4 measurements for RCSPD/DL RSCP together with the paired single ToA measurement (RSTD/UE Rx-Tx time difference) or it may report only the latest time interval for RSCPD/DL RSCP, as configured by LMF.

RAN4 to design DL CPP for both cases no PRS muting and PRS muting, respectively.

RAN4 to agree the measurement period is same for RSCP/RSCPD and the phase quality indication.

RAN4 to await the measurement type definition for phase quality indication in TS 38.215.

RAN4 to agree during the configured time windows for UL CPP there is no collision with data in RRC_CONNECTED state.

RAN4 to use the Rel-16 measurement behaviour for RSTD in case of handover as baseline for measurement behaviour for RSCPD.

RAN4 to modify the RRM requirement for serving cell change for DL CPP with configured DL RSCP and UE Rx-Tx time difference such, that both measurements need to be restarted upon serving cell change.

RAN4 to investigate RRM impacts due to SRS configuration change and UL transmission timing change due to TA adjustment or UE autonomous timing adjustment.

RAN4 to investigate whether UE can keep using fixed Tx timing although the DL reception reference timing is changed in order to guarantee the measurement accuracy for UL RSCP.

- Measurement period requirements in RRC_INACTIVE

RAN4 to consider the case of collision of PRS resources in configured time window(s) with DL signals/channels, such as SSB, SIB1, CORESET0, MSG2/MSGB, paging and DL SDT for defining the measurement behaviour for DL CPP in RRC_INACTIVE state.

Measurement reporting requirements

Regarding the measurement reporting for DL PRS, RAN4 to distinguish two cases:

- If reduced number of measurement samples is configured for ToA measurements (UE Rx-Tx time difference or RSRD, respectively), measurement reporting of CPP and other measurements can be done together after each measurement sample. Reporting delay requirements for reduced number of samples in Rel-17 apply.

- Else, if not configured, then the measurement reporting is based on normal measurement samples for ToA measurements (UE Rx-Tx time difference or RSTD, respectively) and UE can report measurement results from multiple time windows covered by the normal measurement period. Reporting delay requirements for normal number of samples in Rel-16 apply.

Whether network supports joint measurement reporting of NR CPP UL measurements (i.e. UL RSCP) and legacy UL positioning measurements, is left implementation specific, however NRPPa needs to support it.

RAN4 to continue the investigation on solutions reducing the carrier frequency offset.

- Measurement accuracy requirements

- RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to BW related RRM performance requirements.

Channel models for CPP measurement accuracy requirements

RAN4 to specify NR CPP performance requirements for AWGN and the 2-tap-channel model for the fading channel.

Number of measurement samples for CPP

RAN4 to continue the evaluation of 4 measurement samples performance aside 1 measurement sample performance, and thereafter provide feedback to RAN1, as RAN1 has restricted CP measurements to 1 measurement sample.

SINR side conditions for DL RSCP

RAN4 to specify the same SINR side condition for the NR carrier phase measurement DL RSCP as for UE Rx-Tx time difference for Rel-16/Rel-17 positioning, i.e.

| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-3, -13, -13) dB for NSample = 4; (0, -6, -6) dB for NSample = 1; | (-3, -13, -13) dB for NSample = 4; (0, -6, -6) dB for NSample = 1; |
| --- | --- | --- |

- References

- RP-232670, “Revised WID on Expanded and Improved NR positioning”, 3GPP TSG RAN#101

- R4-2314354, “WF on R18 NR positioning – SL positioning and Carrier Phase Positioning”, CATT

- R4-2314165, “Topic summary for [108][220] NR_pos_enh2_part2”, Moderator (CATT)

- R4-2311991, “Effect of Carrier Frequency Offset on Carrier Phase Positioning”, Lenovo, Motorola Mobility

- R1-2305972, “FL Summary #3 for NR DL and UL carrier phase positioning”, Moderator (CATT)

- R4-2310078, “Simulation assumptions for CPP measurement”, CATT

- R4-2314363, “Summary of the simulation results for CPP measurement”, CATT

- R4-2316755, “Simulation results for DL RSCPD”, Nokia, Nokia Shanghai Bell

- R4-2310163, “WF on NR Positioning - SL positioning and CPP”, CATT

- R4-2002287, “Updated link level simulation assumption for RSTD and RSRP”, Huawei

- R4-2009127, “Link-level simulation assumptions for UE Rx-Tx time difference measurements”, Qualcomm

- Annex 1: RAN1 Agreements on NR CPP

RAN1 #112

Conclusion A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report. Agreement RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. RAN1 will include the outcome in an LS to SA2 and RAN2, and cc RAN3. Note: Enhancements might or might not have RAN1 specification impact. Agreement The draft LS reply in R1-2302145 is endorsed. Final LS in R1-2302146. Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

RAN1 #112bis

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113). - Option 1: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. - Option 2: - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2. - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. Agreement The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them. Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements. Agreement For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Agreement Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113): - Option 1: - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot. - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp. - Option 2: - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements. Agreement To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113): - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs. - FFS: the details of \the UE/TRP Tx/Rx PEGs - Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals. - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID. - Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals. - The transmission/reception associated with the same ARP ID is assumed from the same ARP. - FFS: the maximum number of ARP IDs. - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. - Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

RAN1 #113

Agreement Support the following definition of the reference point of the UE/TRP carrier phase measurements: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for both frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for both frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for measurement reporting. Agreement For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc. Agreement If a UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TRP for RSCPD is the same as the reference TRP reported for RSTD. - The target and the reference TRP are in the same PFL Agreement From RAN1’s perspective, carrier phase positioning for UE in RRC_IDLE state is supported for UE-based and UE-assisted positioning in Rel-18. - Note: No additional specification work is expected specifically related to carrier phase positioning for UE in RRC_IDLE state in RAN1. Agreement To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355. - FFS: the maximum number of the windows

RAN1 #114

Agreement Confirm the following working assumption with modification made in RAN1#113: Working assumption To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16} Agreement When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16} Agreement When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8} Agreement Each DL RSCP/RSCPD measurement instance is obtained with Nsample=1 sample only. Conclusion From RAN1’s perspective, the granularity and the range of the RSCP/RSCPD measurements can be defined by RAN4. Agreement For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. Agreement Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: - phase quality index - phase quality resolution The values of the phase quality index and phase quality resolution are left for RAN4.