3GPP TSG-RAN WG4 Meeting #108bis                 	R4-2316756

Xiamen, China, October 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on SCG activation delay in FR1+FR1 NR-DC

Agenda item:	5.8.3

Document for:	Discussion

- Introduction

RAN plenary has agreed Rel-18 WI for Even Further RRM requirement for NR and MR-DC [1], to define RRM requirements for FR1+FR1 NR-DC scenarios is one of the objectives in this WI. The discussion started in RAN4#104e. Till RAN4#108 meeting, most of the impacted RRM requirements for FR1+FR1 NR-DC scenarios have conclusion and big CR is endorsed in RAN4#107 meeting [6]. There are still a few open issues in SCG activation requirements. All the agreements and open issues in RAN4#108 meeting are captured in the agreed way forward [3]. This issue also discussed in Rel-17 and agreed WF in Rel-17 [4].

In this paper, we will provide our view on the remaining open issues in SCG activation requirements for FR1+FR1 NR-DC scenarios. The issues are closely related to similar discussions in Rel-17.

- Discussion

In last meeting, we discussed Tsearch in SCG activation delay for RACH-less based and RACH based SCG activation and the BFD and RLM related aspects in SCG activation delay. For RACH-less based SCG activation, we further discussed whether to update the side conditions for Tsearch to consider BFD and RLM related aspects for the case when target PSCell is known and TCI is known.

For RACH based SCG activation, in the agreed requirements the delay only considers if the UE is not configured with bfd-and-RLM with value true. Whether to define RACH-based SCG activation delay for the case when UE is configured with bfd-and-RLM with value true is continued.

RAN4 has earlier agreed to jointly define SCG activation delay in R18 WI and R17 maintenance since the same discussions are still ongoing in R17 WI. The agreements and open issues are captured in the WF [3]. The remaining open issues are listed below (details for each options can be found in WF[3]):

Agreement: RAN4 can leverage the conclusion from R17 maintenance of SCG activation/deactivation if any. #### Issue 1-1: Side condition for RACH-less SCG activation/deactivation No consensus. Keep using current definition of side conditions for CR drafting with bracket. - Option 1(Apple, OPPO): keep current side conditions for Tsearch and - known target cell - known TCI state - UE is configured with bfd-and-RLM with value true and without detecting RLF or BFD - Option 2(Ericsson): New known/unknown PScell definition #1 - Option 3(Nokia): New known/unknown PScell definition #2 #### Issue 1-2: Tsearch requirement for RACH-less PSCell activation - Option 1(Apple, OPPO): current requirements + current known/unknown PScell definition. - Option 2(Ericsson): current requirements + new known/unknown PScell definition#1. - Option 3(Nokia): current requirements + new known/unknown PScell definition#2. - Option 4(Nokia): updated requirements + current side condition unchanged #### Issue 1-3: LS on clarification of UE behaviour during RACH-less SCG activation No conclusion is reached on LS out in this meeting. - Option 1(vivo): LS is not needed to check with RAN2 on the indication for the case that beam failure has been detected during SCG activation procedure - RAN4 assumes UE should not indicate beam failure to the network if beam failure has been detected during SCG activation procedure. - Option 2(Ericsson): Send LS to RAN2 on clarification of UE behavior during RACH-less SCG activation #### Issue 2-1: Tsearch for RACH-based PSCell activation - Option 1(Apple, OPPO): Not necessary to change the RACH based PSCell activation requirement in agreed CR R4-2310080, i.e., - If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms. - Option 2 (Nokia): updated requirements + current side condition unchanged - Option 3(Nokia): current requirements + new known/unknown PScell definition#2.

The same discussion is also in Rel-17, and based on the online discussion, the way forward is captured in WF[4] as below:

# Way Forward Interested companies to provide views on: - Known and unknown conditions for the deactivated PSCell. - Tsearch delay for known and unknown PSCell being activated.

Based on the last meeting’s discussion, the remaining open issues for RACH-less based and RACH based SCG activation are known conditions and the Tsearch delay, and it could be discussed from below two aspects:

- UE is configured with bfd-and-RLM with true and without detecting RLF or BFD

- The conditions for a known target PSCell

2.1	Aspects related to BFD and RLM

If the UE is configured ‘bdf-and-RLM’ with value ‘true’ the UE shall perform BFD (according to section 8.5) and radio link monitoring (according to section 8.1) on the deactivated PSCell. For the UE to perform these procedures, the UE is required to receive and evaluate the associated RSs according to the defined requirements. And such evaluation is performed in a regular and periodic manner.

There is no difference between performing BFD and RLM on the deactivated PSCell and performing the same procedures on the PCell or activated PSCell in DRX. Hence, just like in non-deactivated state the UE shall measure the deactivated PSCell regularly to evaluate the downlink radio link quality.

If the UE is required to perform bfd-and-RLM on the deactivated PSCell, and has not experienced either beam failure and RLF on the deactivated PSCell, there is no need for cell detection, measurements, or beam recovery time upon PSCell activation. If either beam failure or radio link failure occurs while the PSCell is deactivated there may be a need for such additional delays upon activation.

- If UE has not detected either beam failure or RLF on the deactivated PSCell, there is no need for additional search or measurements at PSCell activation.

If UE has experienced link quality problems leading to a failure (either beam failure or RLF) on the PSCell while in deactivated state, some additional search or measurements is likely needed.

- If UE has detected either beam failure or RLF while the PSCell was deactivated, additional search or measurements at PSCell activation might be needed.

However, there is a difference whether the failure was a beam failure, and hence there is a need for beam recovery, or a radio link failure, where a new cell search might be needed.

A UE which has not detected either BFD or RLF on the deactivated PSCell, need no additional search or measurements at PSCell activation.

A UE which has detected BFD on the deactivated PSCell is allowed additional measurements at PSCell activation.

A UE which has detected RLF on the deactivated PSCell is allowed additional search and measurements at PSCell activation.

It should be noted that this discussion is orthogonal to whether the deactivated PSCell is known or unknown as this is strongly dependent on when an L3 measurement report was sent from the deactivated PSCell (among other conditions).

2.2	Tsearch in RACH-less based PSCell activation delay

Based on the discussion in 2.1 and proposals 1, 2 and 3, we propose to account the UE BFD and RLF conditions at PSCell activation, for a UE which is configured with bfd-and-RLM (value true) for the deactivated PSCell. As discussed in 2.1 the need for additional search and measurement time during the PSCell activation would depend on whether the UE has experienced either BFD or RLF or not on the deactivated PSCell.

For example, for the scenario where neither BFD has occurred on the deactivated PSCell, the PSCell is still detected, measured and the TCI state is known (otherwise BFD would have been detected).

Hence, we suggest that for the RACH-less based PSCell activation, Tsearch = 0ms at least when no BFD has been detected while the PSCell was deactivated. Hence, independently of when last valid measurement report was sent.

The TCI state known conditions are defined as:

If the UE is configured to perform BFD while the SCG is deactivated

-	The TCI state is known if the following conditions are met:

-	During the period from the PSCell deactivation to the completion of PSCell activation, while PSCell was deactivated,

-	UE has not detected beam failure

-	Otherwise, the TCI state is unknown.

It means UE shall assess the downlink radio link quality of a serving cell based on the reference signal used for BFD. It does not seem justified that just because the PSCell is not ‘known’ (based on time since last L3 measurement report) should not unconditionally lead to that that UE falls back to RACH based unknown PSCell activation.

Reason why the current known condition is not suitable is the condition related to the L3 measurement reporting:

In FR2, the PSCell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the SCG activation command:

-	the UE has sent a valid measurement report for the PSCell being activated and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

The measurement reporting condition means that even if the PSCell remains in good conditions and UE has measured the PSCell regularly for bfd-and-RLM, the PSCell will automatically become unknown after 5 seconds unless a valid measurement report has been sent within 5 second.

We suggest enabling RACH-less based activation delay such that it is not conditioned on the when the last valid measurement report was sent.

Other two conditions for the known PSCell condition should remain:

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

These two conditions are related to the actual signal conditions and these should still be applied.

For the RACH-less based PSCell activation, Tsearch = 0ms provided the TCI state is known.

Define RACH-less based activation delay such that it is not conditioned on the when the last valid measurement report was sent.

Based on this we propose to update the RACH-less based PSCell activation as follows (using FR2 SCG as example due to Rel-17 discussions):

For RACH-less based PSCell activation, if bfd-and-RLM are configured with value true and TCI state is known or if the target cell is a known FR1 PScell, Tsearch = 0 ms. Otherwise, there are no requirements.

Another way to enable RACH-less based PSCell activation delay such that it is not conditioned on when the last valid measurement report was sent, is by having different known conditions for deactivated PSCell. One known condition which applies for RACH-based PSCell activation and one known condition which applies for RACH-less based PSCell activation. Hence, conditions for known deactivated FR1 PSCell when configured with bfd-and-RLM could be defined as:

In FR1, the PSCell configured with bfd-and-RLM with value true is known if it has been meeting the following conditions:

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

2.3	Tsearch in RACH-based PSCell activation delay

Similar discussion applies for RACH-based PSCell activation delay.

Based on the discussion in 2.1 and 2.2 we propose that for the UE which is configured with bfd-and-RLM for the deactivated and has not experienced either beam failure or RLF on the deactivated PSCell while deactivated, there is no need for long Tsearch.

Reasoning is similar as discussed in section 2.2. If the UE is configured with bfd-and-RLM and no RLF has been detected it means that the deactivated PSCell is surely still available and has been measured recently (according to the measurement requirements for a deactivated PSCell). Hence, there seems no justification why Tsearch should be 3*Trs at PSCell activation.

Hence, we suggest that for the RACH-based PSCell activation the condition when Tsearch reduced from 3*Trs for an unknown PSCell if no RLM has occurred.

For example:

For RACH based PSCell activation, if the target cell is a known NR FR2 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR1 PSCell configured with bfd-and-RLM with value true and no RLM has occurred, then Tsearch = [1]* Trs ms, otherwise if Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

For RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR1 PSCell configured with bfd-and-RLM with value true and no RLM has occurred, then Tsearch = [1]* Trs ms, otherwise if Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

2.4	Known condition for PSCell activation

RAN4 has discussed how to account the known TCI state in the definition of the known PSCell for deactivated PSCell. Based on a similar discussion as above it has been proposed that if a UE is configured with bfd-and-RLM for the deactivated PSCell and the UE has not experienced BFD while PSCell is deactivated, the PSCell can be considered as known.

Reason behind the discussion is based on the current requirements concerning link recovery. If the UE is configured ‘bdf-and-RLM’ with value ‘true’ the UE shall perform BFD measurements (according to section 8.5) on the deactivated PSCell. For the UE to perform this procedure, the UE is required to receive and evaluate the associated RSs according to the defined requirements. The evaluation is performed regularly.

Hence, if the UE is required to perform bfd-and-RLM and has not experienced either beam failure or RLF on the deactivated PSCell, there is no need for cell detection, measurement, or beam recovery time upon PSCell activation.

We propose to capture this in the known condition for deactivated PSCell (8.17.2):

In FR1 and FR2, the deactivated PSCell is known if it has been meeting the following conditions:

- If bfd_and_RLM with value true is configured for the deactivated PSCell and the TCI state is known, and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

or

-	During the last 5 seconds before the reception of the SCG activation command:

-	the UE has sent a valid measurement report for the PSCell being activated and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

Capture the condition that when bfd_and_RLM with value true is configured and the TCI state is known when defining the PSCell is known.

In FR1 and FR2, the deactivated PSCell is known if it has been meeting the following conditions:

- If bfd_and_RLM with value true is configured for the deactivated PSCell and the TCI state is known, and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

or

-	During the last 5 seconds before the reception of the SCG activation command:

-	the UE has sent a valid measurement report for the PSCell being activated and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

It should be noticed that this proposal is an alternative to the former proposals.

The same discussion is still ongoing for FR2 SCG activation in R17 WI, RAN4 has agreed that RAN4 can leverage the conclusion from R17 maintenance of SCG activation/deactivation if any. Hence, the discussion in SCG activation delay in R18 WI and R17 WI should be joint together.

- Conclusion

In this paper we address the remaining open issues related to SCG activation/deactivation delay in FR1+FR1 NR-DC. Based on the discussion in this and former meetings we propose:

General aspects:

- If UE has not detected either beam failure or RLF on the deactivated PSCell, there is no need for additional search or measurements at PSCell activation.

- A UE which has not detected either BFD or RLF on the deactivated PSCell, need no additional search or measurements at PSCell activation.

A UE which has detected BFD on the deactivated PSCell is allowed additional measurements at PSCell activation.

A UE which has detected RLF on the deactivated PSCell is allowed additional search and measurements at PSCell activation.

Tsearch in RACH-less based PSCell activation delay:

For the RACH-less based PSCell activation, Tsearch = 0ms provided the TCI state is known.

Define RACH-less based activation delay such that it is not conditioned on the when the last valid measurement report was sent.

For RACH-less based PSCell activation, if bfd-and-RLM are configured with value true and TCI state is known or if the target cell is a known FR1 PScell, Tsearch = 0 ms. Otherwise, there are no requirements.

Tsearch in RACH based PSCell activation delay:

For RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR1 PSCell configured with bfd-and-RLM with value true and no RLM has occurred, then Tsearch = [1]* Trs ms, otherwise if Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

Known condition for PSCell activation:

Capture the condition that when bfd_and_RLM with value true is configured and the TCI state is known when defining the PSCell is known. The text proposal is highlighted as below:

In FR1 and FR2, the deactivated PSCell is known if it has been meeting the following conditions:

- If bfd_and_RLM with value true is configured for the deactivated PSCell and the TCI state is known, and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

or

-	During the last 5 seconds before the reception of the SCG activation command:

-	the UE has sent a valid measurement report for the PSCell being activated and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

It should be noticed that Proposal8 is an alternative to the former proposals.

We have captured these changes in our CR [12], two change options provided based on the above proposals.

- References

- RP-221696	Revised WID on Rel-18 even Further RRM enhancement for NR and MRDC, Apple, Vivo, RAN#96

- TS38.133, i20

- R4-2314337	WF on NR FeRRM (part2), Oppo, RAN4#108

- R4-2314395	WF on LTE_NR_DC_enh2 maintenance - PSCell activation, Nokia, RAN4#108

- R4-2310167 WF on FeRRM part2, Oppo, RAN4#107

- R4-2310080	Big CR for R18 RRM enhancement- FR1+FR1 NR-DC, Oppo, RAN4#107

- R4-2306322 WF on R18 RRM enhancement - FR1+FR1 NR-DC, Oppo, RAN4#106bis-e

- R4-2303304 Way forward on R18 RRM enhancement (part 2), Oppo, RAN4#106

- R4-2220736 WF on FR1-FR1 NR-DC in eFeRRM, Oppo, RAN4#105

- R4-2217250 WF on R18 eFeRRM – RRM requirements for FR1-FR1 NRDC, Oppo, RAN4#104bis-e

- R4-2214345	WF on R18 eFeRRM, Apple, RAN4#104e

- R4-2316757 Draft CR on SCG activation delay for FR1+FR1 NR-DC, Nokia, Nokia Shanghai Bell