3GPP TSG-RAN WG4 Meeting #108bis	R4-2316759

Xiamen, China, October 9 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for non-collocated FR1 intra-band scenarios

Agenda item:	5.11.2

Document for:	Discussion

Introduction

At RAN#101 meeting, it was agreed to introduce a new BS signaling to indicate whether Type 1 or Type 2 capability requirements will apply and an LS was sent to RAN2 for signaling design [1]. In this paper, we are discussing the potential impact to RRM due to the new BS signalling.

RP-232692:

2) New BS signalling

During RAN#101 meeting, RAN agreed to introduce new BS signalling by RRC to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 2 and task RAN2 to develop relevant signalling.

This new BS signalling is applied to the below cases.

For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands

This new BS signalling is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands provided the UE has indicated “interBandMRDC-WithOverlapDL-Bands-r16”.

For TDD-TDD intra-band NR-CA

This new BS signalling is expected to be applicable for TDD-TDD intra-band NR-CA provided the UE has indicated “New UE capability”.

Discussion

At RAN4#106bis, the draft BigCR on RRM requirements for intra-band non-collocated scenarios was endorsed in [2]. It was understood the UE indicating the capability [intraBandNRCA-NonCollocated-r18] is a Type 2 UE and it shall fulfill the MRTD/MTTD and the other RRM requirements as defined for inter-band CA. Otherwise, if the UE does not indicate the capability, it is a Type 1 UE and shall fulfill the legacy requirements for intra-band CA.  That is, the UE type has been considered as a static UE capability and RRM requirements are defined based on the UE capability indication as exampled below on MRTD.

### 7.6.4	Minimum Requirements for NR Carrier Aggregation

For intra-band contiguous CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE not capable of [intraBandNRCA-NonCollocated-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference as shown in 7.6.4-2, between slot timing of different FR1 carriers to be aggregated at the UE receiver provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18].

Observation #1: In RRM, the UE type has been considered as UE capability and RRM requirements are defined based on the UE capability indication.

However, from RAN#101 conclusion, a Type 2 UE may be configured to switch between Type 1 and Type 2 with a new BS signalling. This means, for a Type 2 UE capable of [intraBandNRCA-NonCollocated-r18], the UE may operate either Type 1 or Type 2 based on the new BS signaling.

Observation #2: From RAN#101 conclusion, a Type 2 UE indicating the capability of [intraBandNRCA-NonCollocated-r18] may operate either Type 1 or Type 2 based on the new BS signaling.

As a Type 2 UE may operate Type 1 architecture, the RRM requirement shall not consider UE capability alone, instead it is dependent on the UE Type in operation based on BS signaling. For instance, if a Type 2 UE is configured to apply the Type 1 capability requirement via the BS signaling, the RRM requirements for Type 1 UE shall apply. Therefore, the RRM requirements need to be adapted based on the UE type in operation as indicated by the BS signaling.

Proposal 1: The RRM requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

Based on the proposals, we provide the draftCR illustrating the impact on MRTD/MTTD due to the introduction of new BS signaling for intra-band non-collocated NR-CA [3].

- Conclusion

This contribution discusses the potential impact to RRM requirements for non-collocated FR1 intra-band scenarios due to the new BS signaling agreed in RAN#101. The observations and proposals are summarized as below:

Observation #1: In RRM, the UE type has been considered as UE capability and RRM requirements are defined based on the UE capability indication.

Observation #2: From RAN#101 conclusion, a Type 2 UE indicating the capability of [intraBandNRCA-NonCollocated-r18] may operate either Type 1 or Type 2 based on the new BS signaling.

Proposal 1: The RRM requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

References

- RP-232692, LS on signaling support for intra-band non-collocated NR-CA,EN-DC, KDDI

- R4-2306399, Draft Big CR on RRM requirements for support of intra-band non-collocated EN-DC/NR-CA deployment, Huawei

- R4-2316760, draftCR on MRTD/MTTD due to UE type switching, Nokia, Nokia Shanghai Bell