3GPP TSG-RAN WG4 Meeting #108bis	R4- 2316774

Xiamen, China, Oct 9 – Oct 13, 2023

Source: 	Huawei, HiSilicon

Title:	Discussion on adding 30MHz CBW for band n8

Agenda Item:	4.26.2.1

WI Code:	NR_bands_R18_BWs-Core

Document for:	Approval

# 1	Introduction

New channel bandwidth of 30MHz has been requested for band n8 [1]. This paper discusses the REFSENS and RSD for this new bandwidth.

# 2	Discussion

The existing REFSENS for band n8 is defined as follows:

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n8 | 15 | -97.0 | -93.8 | -91.4 | -85.8 | -83.6 |  | -78.4 |  |  |  |
|  | 30 |  | -94.1 | -91.7 | -87.2 | -84.7 |  | -78.5 |  |  |  |

And the UL configuration for REFSENS is:

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n8 | 15 | 25 | 251 | 201 | 201 | Note 5 |  | Note 5 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 | Note 5 |  | Note 5 |  |  |  |  |  |  |  |  |  |
| Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Assuming that the 30MHz BW is only for DL and the UL BW is limited to 20MHz, the REFSENS for 30MHz can be interpolated from that for 25MHz and 35MHz.

Proposal 1: For band n8, the REFSENS for BW=30MHz DL (20MHz UL) is set to -80.6dBm for SCS=15kHz and -80.7dBm for SCS=30kHz.

Following the same methodology used for other PC2 FDD bands [2], the PC2 RSD for 30MHz in band n8 is estimated to be 3.6dB for 1Tx PC2 and 6.8dB for 2Tx PC2.

Proposal 2: For band n8, the PC2 RSD for 30MHz is set to 3.6dB for 1Tx PC2 and 6.8dB for 2Tx PC2.

# 3	Conclusion

Based on the analysis in Section 2, we have the following proposal:

Proposal 1: For band n8, the REFSENS for BW=30MHz DL (20MHz UL) is set to -80.6dBm for SCS=15kHz and -80.7dBm for SCS=30kHz.

Proposal 2: For band n8, the PC2 RSD for 30MHz is set to 3.6dB for 1Tx PC2 and 6.8dB for 2Tx PC2.

# References

- RP-231706 Basket WID on adding channel bandwidth support to existing NR bands, Ericsson, RAN#101

- R4-2300715 MSD for PC2 FDD bands, Huawei, HiSilicon, RAN4#106