3GPP TSG-RAN WG4 Meeting # 108-bis									          R4-2316776

Xiamen, China, 9th October – 13th October, 2023

Title: 	Uplink CA_n5B BCS1 MSD

Agenda Item:	4.1.1.2

Source: 	Skyworks Solutions, Inc.,

Document for:	Approval

# 1	Introduction

At RAN4 meeting #108, a CR was agreed upon to introduce new BCS4-5 support for uplink (UL) class B intra-band contiguous carrier aggregation (CA) in band n7, aka "CA_n7B" [1]. This contribution proposes an MSD test point that complies with the guidelines of Way Forward (WF) [2].

# Discussion

## 2.1 Background

Figure 1 and Figure 2 below summarize respectively the agreed MSD test point and the corresponding IMD landscape for CA_n7B BCS 0, for which the maximum aggregated bandwidth is 50MHz [3].

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7B | 15/15 | 10MHz + 40MHz | 9 (RBstart = 26) | 36 (RBstart = 180) | 34 | 25 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |

Figure 1: CA_n7B BCS 0 MSD test point [3]

Figure 2: CA_n7B BCS0 IMD landscape [3]

The PCC/SCC MSD is specified at 34dB and 25dB due to IMD5 and IMD7 PCC/SCC overlap, respectively.

In this contribution, we propose an MSD test point for BCS4-5 that complies with WF [2].

## 2.2 CA_n7B BCS4-5 MSD Test Point

With BCS4-5, the maximum aggregated BW now becomes 70MHz, i.e., the full band n7 bandwidth. According to WF [2], the MSD test point should be selected to maximize the PCC MSD. For 70MHz aggregated BW, this condition may be met for the smallest PCC CBW, i.e., for PCC/SCC respective CBW configurations of 20MHz+50MHz. It can be anticipated from the BCS0 IMD landscape that the BCS4-5 configuration may create conditions where IMD3 overlaps the PCC and IMD5 overlaps the SCC. Hence, the MSD is likely to be even higher for BCS4-5 than it currently is for BCS0.

Also, this BCS4-5 request is equivalent to configuring the UE/network with component carriers that occupy the whole bandwidth of band n7. To our knowledge, there  no operators have such spectrum allocation. This BCS4-5 request is therefore questionable.

Observation: With 70MHz maximum aggregated BW, the CA_n7B BCS4-5 request is equivalent to configuring the UE/network with component carriers that occupy the whole bandwidth of band n7. The benefits of this BCS4-5 request are therefore questionable since:

- To our knowledge, there are no operators with such spectrum allocation, so this BCS4-5 request may not represent commercial deployments;

- The MSD is expected to be very high, in the range of ~ 42 dB for PCC, therefore the downlink radio link performance may be poor; and

- This request may lead to very high A-MPR, the benefits of such uplink configuration are unclear.

We propose an MSD test point which ensures a full IMD3 overlap of the PCC and a partial IMD5 overlap of SCC.

Proposal: Adopt the MSD test point specific to UL CA_n7B BCS4-5 highlighted in green in Table 1 below. A new footnote is added to distinguish the test point applicable to BCS4-5 vs that previously agreed for BCS0.

Table 1 CA_n7B BCS4-5 MSD test point proposal

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7BX | 15/15 | 20MHz + 50MHz | 9 (RBSTART = 4) | 36 (RBSTART = 230) | TBD | TBD | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 4-5 |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we propose an MSD test point for CA_n7B BCS4-5 which ensures a full IMD3 overlap of the PCC and a partial IMD5 overlap of SCC. We invite interested companies to evaluate the PCC/SCC MSD.

Observation: With 70MHz maximum aggregated BW, the CA_n7B BCS4-5 request is equivalent to configuring the UE/network with component carriers that occupy the whole bandwidth of band n7. The benefits of this BCS4-5 request are therefore questionable since:

- To our knowledge, there are no operators with such spectrum allocation, so this BCS4-5 request may not represent commercial deployments;

- The MSD is expected to be very high, in the range of ~ 42 dB for PCC, therefore the downlink radio link performance may be poor; and

- This request may lead to very high A-MPR, the benefits of such uplink configuration are unclear.

Proposal: Adopt the MSD test point specific to UL CA_n7B BCS4-5 highlighted in green in Table 1 below. A new footnote is added to distinguish the test point applicable to BCS4-5 vs that previously agreed for BCS0.

Table 1 CA_n7B BCS4-5 MSD test point proposal

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7BX | 15/15 | 20MHz + 50MHz | 9 (RBSTART = 4) | 36 (RBSTART = 230) | TBD | TBD | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 4-5 |  |  |  |  |  |  |  |

# References

- R4-2311102 big CR 38.101-1 new combinations Rel-18 NR Intra-band, 3GPP TSG-RAN WG4 #108 Toulouse, France, Ericsson.

- R4-2314864 WF on intra-band contiguous UL-CA MSD, 3GPP TSG-RAN WG4 #108 Toulouse, France, Skyworks Solutions, Inc., Murata.

- R4-2202028 Corrections to Intra-band CA MSD for CA_n5B and CA_n7B, 3GPP TSG-RAN WG4 Meeting # 101bis-e, Electronic Meeting, Skyworks Solutions, Inc.