3GPP TSG RAN WG4 Meeting #108bis	R4-2316827

Xiamen, China, 09 Oct. 2023 – 13 Oct. 2023

Agenda Item:	5.29.2.1

Source: 	Ericsson

Title:	Discussion on introducing TDCP requirements

Document for:	Discussion

# Introduction

In this contribution we analyze the RRM requirements for Rel-18 TDCP measurements.

# Discussion

## TDCP measurements

In RAN4#108, following agreements are made for TDCP amplitude channel autocorrelation.

  - Do not define TDCP measurement delay requirements.

  - Further identify feasibility of methodology and test setup to define TDCP accuracy requirements including at least ideal TDCP definition, channel models.

  - Define TDCP accuracy requirements subject to conclusions of feasibility analysis.

  - For TDCP measurement averaging across RX ports should be left to UE implementation.

    - The reported TDCP amplitude is expected to between the minimum and maximum measured values across the Rx branches if receiver diversity is used.

Based on the last meeting agreement, RAN4 needs to identify and agree on following aspects to define the TDCP accuracy requirements.

- Ideal TDCP definition

- Channel models

- Accuracy requirements framework

## Ideal TDCP definition or determination

For any feature in RAN4, RAN4 introduced core or performance requirements are to be verified with test cases to guarantee minimum performance for the feature. In the test, the UE measured value will be reported to test equipment (TE) and the TE compare the UE reported value with expected value (i.e., the ideal value). In last meeting there were concerns from companies on how to determine the ideal value or expected value for verification in the tests. This is mainly due to the fact that majority of the RAN4 tests are introduced with AWGN channel and since TDCP is a channel auto correlation matric, RAN4 cannot introduce TDCP tests with AWGN channel but with fading channels. For the test cases of other features, ideal value or expected value computation for AWGN channel is straight forward. The method to compute the ideal value for fading channels has to be discussed and agreed in RAN4.

In last meeting, following channel models are briefly discussed concerning which channel models to use for feasibility study of TDCP requirements.

- TDL-A/B/C

- CDL

To determine which channel model to use, we look at the autocorrelation equation for TDL and CDL channel models to determine which channel model to consider for feasibility study.

For TDL-A/B/C channel model, the autocorrelation is given by

  - A∆t=J02π∙fc∙vc∙∆t

Where,

- A∆t is the channel auto-correlation.

- J0 () is Bessel function.

For CDL channel model, the autocorrelation is given by

  - A∆t=kPk∙ej2π∙fc∙vc∙cosβk∙∆t

Where,

- βk is the angle between the UE direction and the direction of arrival of channel ray k.

- Pk is the normalized power of channel ray k after taking into account of the UE and BS antenna gain which depends on the direction of arrival and the direction of departure of channel ray k.

Autocorrelation for CDL-A, for different UE directions phi, using omnidirectional UE and BS antennas is shown in figure 1.

Figure 1: Auto correlation for CDL-A for different UE directions in the horizontal plane as given by the azimuthal angle phi

Which channel model to use:

From the above auto correlation equations and figure, we compare the auto correlation of CDL and TDL below to highlight the differences in CDL and TDL based auto correlations.

| CDL Channel | TDL Channel |
| --- | --- |
| Spatial characteristics modelled | Spatial characteristics not modelled as we can assume to apply the same channel to all UE antennas. |
| Autocorrelation dependent on UE location, UE orientation, UE direction, UE antenna gains and BS antenna gain. | Autocorrelation independent on UE location, UE orientation, UE direction, UE antenna gains and BS antenna gain. |
| The ideal value can be calculated based on simulations or through a complex formula. | Well known ideal value given by Bessel function without any need for simulations. |
| Risk that companies make slightly different implementations that result in differences in the autocorrelation. | Easy for companies to agree on ideal value. |
| Note: Autocorrelation dependent on detailed implementation of angular scaling which is not correctly described in 38.901 as per our understanding |  |

Based on the analysis in the above table, we can clearly observe that using TDL channels is convenient and contains lesser workload for RAN4.

- Use TDL channel model for deriving ideal autocorrelation value for the TDCP tests and feasibility.

If the TDL channel is assumed, ideal auto correlation value can be derived/calculated from the formula mentioned above or using the simulation results. Since it can be derived using the formula, we think RAN4 can skip simulation effort and derive the ideal value from the formula. If we observe the formula, A∆t=J02π∙fc∙vc∙∆t, auto correlation depends on doppler spread which can be denoted as fmax=fc∙vc and the delay (∆t). For the delay value,  RAN1 agreed on the set of values that are supported or configurable.  For each delay, auto correlation can be calculated for different fmax.

In the tables below we tabulate the autocorrelation for some example values of the doppler spread fmax and the RAN1 agreed correlation delays. Note that the tables are independent of carrier frequency but depends on the subcarrier spacing since the slot length depends on the subcarrier spacing.

Table 1 Autocorrelation for different combinations of Doppler spread fmax and correlation delay ∆t for 30kHz subcarrier spacing (slot length of 0.5ms)

|  | Correlation delay ∆t [slots] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fmax | 4/14 | 1 | 2 | 3 | 4 | 5 | 6 | 10 |
| 8Hz | 0,999987 | 0,999842 | 0,999368 | 0,998579 | 0,997475 | 0,996056 | 0,994323 | 0,984271 |
| 16Hz | 0,999948 | 0,999368 | 0,997475 | 0,994323 | 0,989919 | 0,984271 | 0,977389 | 0,937825 |
| 32Hz | 0,999794 | 0,997475 | 0,989919 | 0,977389 | 0,959981 | 0,937825 | 0,911089 | 0,762857 |
| 64Hz | 0,999175 | 0,989919 | 0,959981 | 0,911089 | 0,844717 | 0,762857 | 0,667953 | 0,21777 |
| 128Hz | 0,996703 | 0,959981 | 0,844717 | 0,667953 | 0,450557 | 0,21777 | -0,0041 | -0,39566 |
| 256Hz | 0,986843 | 0,844717 | 0,450557 | -0,0041 | -0,32457 | -0,39566 | -0,23276 | 0,161559 |

Table 2 Autocorrelation for different combinations of Doppler spread fmax and correlation delay ∆t for 15kHz subcarrier spacing (slot length of 1ms)

|  | Correlation delay |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| fmax | 0,29 | 1 | 2 | 3 | 4 | 5 | 6 |
| 8 | 0,999948 | 0,999368 | 0,997475 | 0,994323 | 0,989919 | 0,984271 | 0,977389 |
| 16 | 0,999794 | 0,997475 | 0,989919 | 0,977389 | 0,959981 | 0,937825 | 0,911089 |
| 32 | 0,999175 | 0,989919 | 0,959981 | 0,911089 | 0,844717 | 0,762857 | 0,667953 |
| 64 | 0,996703 | 0,959981 | 0,844717 | 0,667953 | 0,450557 | 0,21777 | -0,0041 |
| 128 | 0,986843 | 0,844717 | 0,450557 | -0,0041 | -0,32457 | -0,39566 | -0,23276 |
| 256 | 0,947892 | 0,450557 | -0,32457 | -0,23276 | 0,249325 | 0,161559 | -0,2158 |

Based on the above analysis we think it is feasible to calculate the ideal value that has to be captured in the spec for tests cases.

- RAN4 to agree that ideal value can be calculated for each delay value and over different doppler spread (fmax).

- RAN4 to define TDCP accuracy requirements as the feasibility (i.e., ideal value calculation) is identified.

With TDCP accuracy requirements are feasible to define, we look at how to define the requirements. In other words, we look at how to derive the accuracy value of auto correlation in the subsequent sections.

## TDCP accuracy

In our understanding, to define the accuracy requirement, we first need to understand the sources of error that can introduce the in accuracy in the measurement at the UE. The sum of errors amy be specified as the range for the accuracy. In this section we look at the error sources for channel auto correlation to compute the accuracy error.

### Error sources for autocorrelation estimation

In last meeting there were concerns from companies regarding estimation of the autocorrelation amplitude very close to one. We understand that this is a fair concern and, in some cases, noise will make such measurements impossible. Our intention is not to set impossible requirement in such cases. In other cases, it is however, fully possible to make accurate measurements of the autocorrelation quite close to one. To understand this, we need to understand the error sources at hand.

When estimating the autocorrelation there are of course errors due to noise as normal. There is, however, also a second important error source.

The instantaneous channel correlation which the UE measures can be written as a sum of

- The time independent autocorrelation

  - consisting of a sum of the diagonal correlation terms correlating each channel ray with itself

- The time dependent sum of correlation cross terms

  - correlating channel ray k with channel ray m, for 𝑘≠𝑚

For autocorrelation estimation the cross terms form an extra error source in addition to noise. However, we think we can solve or mitigate this source using averaging techniques (e.g., these cross terms one can average over time and/or frequency).

Another way to say this is that the autocorrelation is the expectation value of the instantaneous channel correlation. The instantaneous channel varies with time and frequency. This time variation is an error source when estimating the autocorrelation and to suppress this error source we have to average over time and/or frequency. To observe and figure out solutions to suppress this error source we did some simulations, and we discuss those simulation results below.

- RAN4 to agree that TDCP accuracy error is a function of channel estimation noise error and error due to auto-correlation cross terms.

Auto correlation with single shot measurement:

In Figure 2 we show the mean and standard deviation (as given by the error bars) of the autocorrelation estimate based on a single measurement occasion in time but averaged over the full bandwidth of 100MHz. The SNR is large (20dB) so the errors are dominated by the cross term errors. We can see that the errors get larger when the autocorrelation gets further away from 1. In fact the standard deviation of the estimate is roughly proportional to one minus the autocorrelation. The cross-term error is quite large, roughly 50% of (1-autocorrelation). Still, even with such a large errors good gains have been seen for the use-cases studied in RAN1.

The cross-term error often dominates over the noise error, but of course this isn’t always the case. In Figure 3 the RMS error is given for different noise levels both in absolute terms and in relation to (1-autocorrelation). We can see that for low UE speeds, when the autocorrelation is very close to one, noise starts to dominate. The error then quickly blows up becoming larger than (1-autocorrelation). This means that based on the estimate we can’t even distinguish the channel from a constant channel for which the autocorrelation is identically equal to 1. In this region there is no reason to define requirements. However, as long as the cross terms are dominating the error, the error is roughly 50% of (1-autocorrelation). Thus, as long as the cross terms are dominating the error it doesn’t become any harder to perform the estimates of an autocorrelation close to 1 than an autocorrelation further away from one.

Figure 2 The mean autocorrelation estimate and the standard deviation of the autocorrelation estimate (given by the error bars) for an estimate averaged over the full bandwidth of 100MHz but based on a single measurement occasion in time for an SNR of 20dB.

Figure 3 The RMS error of the autocorrelation estimate, to the left given in absolute terms and to the right given as a percentage of (1-autocorrelation). The estimate was based on averaging over the full bandwidth of 100MHz but only on a single measurement occasion in time

Auto correlation with time averaging over last 4 samples:

In Figure 4 we again show the RMS error but now for an autocorrelation estimate averaged over 4 measurement occasions each separated by 40ms. We see that this averaging over time reduce the errors significantly. As long as the cross-term errors dominate the RMS error is roughly 25% of (1-autocorrelation).

Figure 4 The RMS error of the autocorrelation estimate, to the left given in absolute terms and to the right given as a percentage of (1-autocorrelation). The estimate was based on averaging over the full bandwidth of 100MHz and over four measurement occasion in time, each separated by 40ms.

Based on the above analysis we think time averaging can help reducing errors and accuracy.

- RAN4 to consider time and frequency averaging for setting TDCP accuracy requirements.

## Configurations for TDCP requirements definition

A typical configuration in the field is a TRS periodicity of 40ms and a TRS bandwidth of the full bandwidth part. We propose that this can be used for the TDCP requirements.

The range of quantization levels for the autocorrelation amplitude put limits to what TDL UE speeds (or equivalently, what Doppler spreads) are relevant to test for the different correlation lags. Too small UE speeds will result in autocorrelation that is larger than the largest quantization level. Too large UE speed will result in an autocorrelation amplitude that is smaller than the smallest quantization level or is in the oscillatory region of the autocorrelation function which makes the autocorrelation report hard to utilize in an unambiguous way, see Figure 2. The resulting limits are given in Table 1 for 30kHz subcarrier spacing (1ms slot length) and for 15kHz subcarrier spacing in Table 2. The limits are given in terms of fmax to make them independent of the carrier frequency but example limits for the UE speed are also given for the case of 3.5GHz carrier frequency with 30kHz subcarrier spacing and for 900MHz carrier frequency with 15kHz subcarrier spacing. Requirements need to be defined only within these limits.

Figure 4 The autocorrelation amplitude for the Jakes model as a function of UE speed at fixed correlation lag for a carrier frequency of 3.5GHz and 30kHx subcarrier spacing.

Table 3 Maximum and minimum levels for the setting of fmax in TDL models in order for the autocorrelation amplitude to be within the range of the quantization levels of the TDCP report for a subcarrier spacing of 30kHz (slot length of 0.5ms). In addition the corresponding maximum and minimum levels for the UE speed are given for the case of a carrier frequency of 3.5GHz. Note that the fmax limits are independent of the carrier frequency but that this isn’t the case for the speed limits.

| Correlation lag | 4symb 0.14ms | 1slot 0.5ms | 2slots 1ms | 3slots 1.5ms | 4slots 2ms | 5slots 2.5ms | 6slots 3ms | 10slots 5ms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min fmax [Hz] | 139 | 40 | 20 | 13 | 10 | 8.0 | 6.6 | 4.0 |
| Max fmax [Hz] | 2095 | 599 | 299 | 200 | 150 | 120 | 100 | 60 |
| Min speed at 3.5GHz [km/h] | 43 | 12 | 6.1 | 4.1 | 3.1 | 2.5 | 2.0 | 1.2 |
| Max speed at 3.5GHz [km/h] | 647 | 185 | 92 | 62 | 46 | 37 | 31 | 18 |

Table 4 Maximum and minimum levels for the setting of fmax in TDL models in order for the autocorrelation amplitude to be within the range of the quantization levels of the TDCP report for a subcarrier spacing of 15kHz (slot length of 1ms). In addition the corresponding maximum and minimum levels for the UE speed are given for the case of a carrier frequency of 900MHz. Note that the fmax limits are independent of the carrier frequency but that this isn’t the case for the speed limits.

| Correlation lag | 4symb 0.29ms | 1slot 1ms | 2slots 2ms | 3slots 3ms | 4slots 4ms | 5slots 5ms | 6slots 6ms | 10slots 10ms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min fmax [Hz] | 84 | 24 | 12 | 8.0 | 6.0 | 4.8 | 4.0 | 2.4 |
| Max fmax [Hz] | 1257 | 359 | 180 | 120 | 90 | 72 | 60 | 36 |
| Min speed at 900MHz [km/h] | 70 | 20 | 10 | 6.6 | 5.0 | 4.0 | 3.3 | 2.0 |
| Max speed at 900MHz [km/h] | 1048 | 299 | 150 | 100 | 75 | 60 | 50 | 30 |

To understand what UE speeds are most important to put requirements for, we need to look at the use cases studied by RAN1. The use case studied in most depth was TDCP based switching between CSI Type I and CSI Type II feedback based precoding. In Figure 3 we see that for the CDL-A channel with random UE direction we see that CSI Type II gives highest throughput below 8km/h while CSI Type I gives highest throughput above 8km/h. Switching between CSI Type I and CSI Type II based on a threshold of 0.984 for the reported autocorrelation amplitude for a lag of five slots (i.e. the TDCP report) gives a throughput that clearly outperforms both CSI Type I and CSI Type II performance.

The reason why TDCP based switching can be significantly higher than the maximum of the CSI Type I and Type II throughputs is that the channel variability for a spatial channel like CDL-A depends strongly on the UE direction, as can be seen in Figure 4. The switchover point between Type I and Type II occurs at different UE speeds for different UE directions. In terms of the autocorrelation amplitude for a lag of 5slots, the switching point is, however, at 0.984, independently of the UE direction. This corresponds to a Doppler spread (defined as the second moment of the Doppler power spectrum relative to the first moment of the Doppler power spectrum) of

fd=1-A∆t2∙π∙∆t+o∆t3≈∆t=0.0025sA∆t=0.984≈11.4Hz

To find a TDL-A model with the same Doppler spread we note that for the Jakes model we have

fd=fmax2=vcfc2

A TDL-A channel should thus have fmax=16Hz (or for 3.5GHz carrier equivalently a UE speed of 5.0km/h) in order to have the same Doppler spread as the CDL-A channel at the CSI Type I - Type II switching point. As a double check, one may use the Jakes expression for the Autocorrelation function to check that fmax=16Hz  does give an autocorrelation value of 0.984 for a correlation lag of 5 slots.

We conclude that an important scenario to test is TDL-A with fmax around the switching point of 16Hz, which for a 3.5GHz carrier corresponds to 5km/h. Suitable settings for requirements could be 8Hz, 16Hz and 32Hz for fmax.

Figure 5 PDSCH throughput for 100Mhz bandwidth at 3.5GHz carrier frequency and 30kHz subcarrier spacing for CDL-A, 100ns delay spread, 10km/h, averaged over UE directions in the horizontal plane. Separate throughput curves are given (1) for fixed CSI Type I feedback based precoding, (2) for fixed CSI Type II feedback based precoding, (3) for TDCP based switching between CSI-Type I and CSI Type II feedback based precoding, (4) for ideal switching between CSI-Type I and II and (5) for random switching between CSI-Type I and II. The correlation lag for the TDCP autcorrelation amplitude estimation was 5 slots and the autocorrelation amplitude threshold used for switching was 0.984.

Figure 6 The autocorrelation amplitude for the TDL-A channel as well as for the CDL-A channel with different UE directions in the horizontal plane. The UE speed is 10km/h. The right hand figure is zoomed in on the low correlation lags most relevant here.

- RAN4 to consider table 3 and 4 as configuration parameters for TDCP accuracy definition.

## Simulation results and preliminary requirements

In this section we provide simulation results for estimation of the autocorrelation amplitude for the lags 10slots, 1 slot and 4symbols, showing the feasibility of setting requirements and discussing the importance of each lag in relation to different use-cases.

We think it’s very important to set requirements for the autocorrelation amplitude estimation for large lags since large lags is necessary to support the most important use-cases studied in RAN1. This is our top priority. We also think it’s important to set requirements for the basic TDCP feature which only support up to 1slot lag.

Finally, we think it’s important to set requirements for a lag of two slots, since some chipset manufacturers indicated that it’s significantly lower complexity for them to implement lags smaller than or equal to two slots, than to implement longer lags.

Thus, we propose that RAN4 define accuracy requirements at least for

- Large lags, e.g. 4slots and 10slots (top priority)

- Basic feature lags, i.e. 4symbol and 1slot lag

- Two slot lag

- RAN4 to define channel autocorrelation accuracy for following delay values.

- 10 slots

- 4 symbols or 1 slot.

- 2 slots

### Simulation results for delay value of 10 slots

Impact of averaging over multiple measurement occasions in time

For a delay spread of 100ns autocorrelation estimation gives decent accuracy even without averaging over time as can be seen in the bottom figure of Figure 3. The accuracy is, however, significantly improved by averaging over two or four measurement occasions in time as can be seen in the mid and top figures of Figure 3. Without averaging over time, the allowed reporting range would need to extend over about 5 quantization levels in order to ensure that the probability of UE reporting outside the allowed range is smaller than 10%. Averaging over four measurement occasions can bring down the allowed reporting range to extend over only 2 to 3 quantization levels.

Without averaging over time, one may note from the bottom figure of Figure 3 that it’s possible to separate 2km/h from 5km/h with good confidence (with a separation threshold between quantization levels Q4 and Q5), but that it’s not possible to separate 2km/h from 3km/h or 3km/h from 5km/ with any good confidence.

Averaging over four measurement occasions one may note from the top figure of Figure 3 that it is possible to separate 2km/h from 3km/h (with threshold between quantization levels Q3 and Q4) as well as 3km/h from 5km/ (with threshold between quantization levels Q6 and Q7), with good confidence.

Low delay spread

If the delay spread is small the accuracy without averaging over time gets worse (averaging over frequency gives worse channel correlation cross term suppression when the coherence frequency is large). I this case, averaging over time is necessary to get decent accuracy, and thus to allow for switching gains in the use cases studied by RAN1. Without averaging over time, the allowed reporting range would need to extend over about 7 quantization levels in order to ensure that the probability of UE reporting outside the allowed range is smaller than 10%. Averaging over four measurement occasions can bring down the allowed reporting range to extend over about 4 quantization levels.

Impact of noise

For many scenarios the errors coming from the channel correlation cross terms dominate and the impact of noise is very small, as can be seen in the top figure of Figure 5. For low UE speeds, corresponding to an autocorrelation close to one, there is, however, an effect of noise as can be seen in the mid and bottom figures of Figure 5. Averaging over time makes it possible to support lower UE speeds and/or lower SNR with reasonable accuracy, as can be seen in Figure 6.

Example requirements

In Table 3, Table 4, Table 5 and Table 6 in the Appendix a wide set of numerical results are given covering SNRs from 0dB to 20dB and delay spread of 30ns and 100ns. Based on these results it’s clearly possible to design accuracy requirements for the TDCP autocorrelation amplitude.

As an example, consider CDL-A with 100ns delay spread at 3.5GHz carrier frequency, 100MHz bandwidth and 30kHz subcarrier spacing.

If we assume that the UE can average over four consecutive measurement occasions then for a UE speed of 2km/h with autocorrelation=0.98966 and 10dB or 20dB SNR we can set the lower limit between quantization levels Q4=0.9844 and Q3=0.9890 and the upper limit between quantization levels Q2=0.9922 and Q1=0.9945, e.g as

Lower limit = ideal value - 0.003 = 0.98966 – 0.003 = 0.98666

Upper limit = ideal value + 0.003 = 0.98966 + 0.003 = 0.99266

Based on Table 3 this gives less than 10% probability of reporting outside the allowed interval both for 10dB and 20dB SNR. For lower SNR a larger allowed interval would be needed.

Figure 7 Probability distribution over quantization levels for the estimated autocorrelation amplitude for a 10 slot lag for TDL-A with 100ns delay spread, 20dB SNR, 3.5GHz carrier frequency, 100MHz bandwidth. The bottom figure is without averaging over measurement occasions in time, while the mid and top figures are for averaging over respectively two and four measurement occasions, each separated by 40ms.

Figure 8 Effect of low delay spread. Probability distribution over quantization levels for the estimated autocorrelation amplitude for a 10 slot lag for TDL-A with 30ns delay spread, 20dB SNR, 3.5GHz carrier frequency, 100MHz bandwidth. The bottom figure is without averaging over measurement occasions in time, while the mid and top figures are for averaging over respectively two and four measurement occasions, each separated by 40ms.

Figure 9 Effect of Noise. Probability distribution over quantization levels for the estimated autocorrelation amplitude for a 10 slot lag for TDL-A with 100ns delay spread, 3.5GHz carrier frequency, 100MHz bandwidth. The UE speed is respectively 5, 3 and 2 km/h in the top, mid and bottom figures. No averaging over time was performed.

Figure 10 Effect of noise. Probability distribution over quantization levels for the estimated autocorrelation amplitude for a 10 slot lag for TDL-A with 100ns delay spread, 3.5GHz carrier frequency, 100MHz bandwidth. The UE speed is respectively 5, 3 and 2 km/h in the top, mid and bottom figures. Averaging was performed over four measurement occasions each separated by 40ms.

### Simulation results and requirements for delay of 4 symbols

From Table 3 we know that for 30kHz subcarrier spacing the range in Dopplerspread as defined by fmax, covered by the 1 slot lag autcorrelation report is 139Hz to 2095Hz. For a carrier frequency of 3.5GHz this corresponds to a range in UE speed from 43km/h to 647km/h. From section 2.4 we know that for the use case of switching between CSI type I and CSI type II based precoding we would ideally like to be able to separate different Doppler spreads as defined by fmax around 16Hz or equivalently to separate different UE speeds around 5km/h. Clearly reporting of the autocorrelation amplitude for a lag of 4symbols would not be useful for Type I – Type II switching. It could, however, be useful for other use-cases like e.g. switching between two and three additional DMRS symbols (see Figure 10).

In addition, the basic TDCP feature only supports reporting of autocorrelation for lags of 4symbols and 1slot, and clearly the basic feature needs to be tested.

Based on simulation results (see Figure 9) it’s clearly feasible to design accuracy requirements and tests.

Thus we propose that accuracy requirement are specified for reporting of the autocorrelation amplitude for a lag of 4symbols.

Figure 11: Probability distribution over quantization levels for the estimated autocorrelation amplitude for a 4 symbol lag for TDL-A with 100ns delay spread, 3.5GHz carrier frequency, 100MHz bandwidth. Averaging was performed over four measurement occasions each separated by 40ms.

Figure 12 Throughput for switching between 1 and 2 additional DMRS symbols based on the estimated channel correlation amplitude over a delay of 4 symbols at an SNR of 18dB. The cross over point between one and two additional DMRS symbols is at roughly 280km/h. TDCP based switching gives close to ideal switching performance.

### Simulation results and requirements for delay of 1 slot

From Table 3 we know that for 30kHz subcarrier spacing the range in Dopplerspread as defined by fmax, covered by the 1 slot lag autcorrelation report is 40Hz to 599Hz. For a carrier frequency of 3.5GHz this corresponds to a range in UE speed from 12km/h to 185km/h. From section 2.4 we know that for the use case of switching between CSI type I and CSI type II based precoding we would ideally like to be able to separate different Doppler spreads as defined by fmax around 16Hz or equivalently to separate different UE speeds around 5km/h. This is clearly not possible with the 1 slot lag report. As can be seen from Figure 11 the best one could do would be to set the threshold between quantization levels Q0 and Q1. This would put the Type I – Type II switch at roughly 15km/h. Based on Figure 11 this threshold would select Type II for UE speeds below or equal to 10km/ and would select Type I for UE speeds higher than or equal to 20km/h. At 15km/h the selection between Type I and Type II would be random. Such switching could possibly give some minor gains for the use case of switching between CSI type I and CSI type II based precoding.

Reporting of the 1 slot lag autocorrelation could of course also be useful for some other use case for which some mode switching needs to be done within the range of Doppler spread / UE speeds covered by the 1 slot range.

Reporting of the 1 slot lag autocorrelation could also be useful together with reporting of the autocorrelation for a larger lag (TDCP support simultaneous reporting of the autocorrelation for up to four different correlation lags). When taking decisions based the reported autocorrelation amplitude it’s typically assumed that the reported value comes from the low lag region of the autocorrelation function, before the start of the oscillations (see Figure 1 and Figure 2). If a reported high autocorrelation value would come from one of the later oscillations of the autocorrelation function rather than from the low lag region of the autocorrelation function this could therefore lead to the BS taking the wrong decision. Such ambiguities could be avoided by reporting the autocorrelation for multiple lags.

The basic TDCP feature only supports lags up to one slot and clearly the basic feature needs to be tested.

Based on above arguments we think it would be useful to set requirements for TDCP reporting of the autocorrelation amplitude for a lag of one slot.

Figure 13: Probability distribution over quantization levels for the estimated autocorrelation amplitude for a 1 slot lag for TDL-A with 100ns delay spread, 3.5GHz carrier frequency, 100MHz bandwidth. Averaging was performed over four measurement occasions each separated by 40ms.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- Use TDL channel model for deriving ideal autocorrelation value for the TDCP tests and feasibility.

- RAN4 to agree that ideal value can be calculated for each delay value and over different doppler spread (fmax).

- RAN4 to define TDCP accuracy requirements as the feasibility (i.e., ideal value calculation) is identified.

- RAN4 to agree that TDCP accuracy error is a function of channel estimation noise error and error due to auto-correlation cross terms.

- RAN4 to consider time and frequency averaging for setting TDCP accuracy requirements.

- RAN4 to consider table 3 and 4 as configuration parameters for TDCP accuracy definition.

Table 3 Maximum and minimum levels for the setting of fmax in TDL models in order for the autocorrelation amplitude to be within the range of the quantization levels of the TDCP report for a subcarrier spacing of 30kHz (slot length of 0.5ms).

| Correlation lag | 4symb 0.14ms | 1slot 0.5ms | 2slots 1ms | 3slots 1.5ms | 4slots 2ms | 5slots 2.5ms | 6slots 3ms | 10slots 5ms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min fmax [Hz] | 139 | 40 | 20 | 13 | 10 | 8.0 | 6.6 | 4.0 |
| Max fmax [Hz] | 2095 | 599 | 299 | 200 | 150 | 120 | 100 | 60 |
| Min speed at 3.5GHz [km/h] | 43 | 12 | 6.1 | 4.1 | 3.1 | 2.5 | 2.0 | 1.2 |
| Max speed at 3.5GHz [km/h] | 647 | 185 | 92 | 62 | 46 | 37 | 31 | 18 |

Table 4 Maximum and minimum levels for the setting of fmax in TDL models in order for the autocorrelation amplitude to be within the range of the quantization levels of the TDCP report for a subcarrier spacing of 15kHz (slot length of 1ms).

| Correlation lag | 4symb 0.29ms | 1slot 1ms | 2slots 2ms | 3slots 3ms | 4slots 4ms | 5slots 5ms | 6slots 6ms | 10slots 10ms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min fmax [Hz] | 84 | 24 | 12 | 8.0 | 6.0 | 4.8 | 4.0 | 2.4 |
| Max fmax [Hz] | 1257 | 359 | 180 | 120 | 90 | 72 | 60 | 36 |
| Min speed at 900MHz [km/h] | 70 | 20 | 10 | 6.6 | 5.0 | 4.0 | 3.3 | 2.0 |
| Max speed at 900MHz [km/h] | 1048 | 299 | 150 | 100 | 75 | 60 | 50 | 30 |

- RAN4 to define channel autocorrelation accuracy for following delay values.

- 10 slots

- 4 symbols or 1 slot.

- 2 slots

# References

[1] 	RP-223276, WID Update: MIMO Evolution for Downlink and Uplink. Samsung
	
[2]		R4-2314453

[3]		R1-2305715

[4]		3GPP TS 38.133 v18.3.0 	NR; Requirements for support of radio resource management

# Appendix

- Appendix A, Autocorrelation estimator

Let Rn be the received signal for the TRS subcarrier n

Rn=hn∙Sn+ηn

where ηn is AWGN with

Eηn2=σ2

and Sn is the known transmitted signal (i.e. the TRS) on the TRS subcarrier n and Sn=1.

We also define

Xn=Rn∙Sn*

(which may be viewed as a first rough estimate of the channel hn) and

Yk=ifftXn

The autocorrelation estimator we used in the evaluations can be written as

At,∆t=k∈ΓYkt+∆t∙Yk*tk∈ΓYkt+∆t2-Γ∙σt+∆t 2k∈ΓYkt2-Γ∙σt2

Here Γ is an interval in the time domain containing the part of the channel which is above noise. Identifying this interval is a standard part of channel estimation and should already be performed by UEs as a part of channel estimation. Γ is the length of the interval in samples and σ is an estimate of the noise. The negative term -Γ∙σ2 in the denominator corrects for the noise bias in Yk2. Note that there is no corresponding bias in the numerator since the noise at the instances t and t+∆t is uncorrelated.

The estimator may alternatively be calculated in the frequency domain in order to avoid the FFT as

At,∆t=n=0N-1Xnt+∆t∙Xn*tnXnt+∆t2-Γ∙σt+∆t 2nXnt2-Γ∙σt 2

where Xn is Xn filtered with a sinc-filter corresponding to the interval Γ.

Averaging over multiple symbols within a TRS burst was done coherently. Averaging over multiple measurement occasions was, however, done incoherently taking the absolute value of At,∆t for each measurement occasion before averaging. This makes the estimator robust against phase jumps.

For normalization (i.e. in the denominator) we used the geometric average over the time instances t and t+∆t in order to make the estimator robust towards AGC as requested by UE manufacturers.

- Appendix B, Simulation assumptions

All simulations were performed for a carrier frequency of 3.5GHz with 30kHz subcarrier spacing. A TRS with two slot bursts and a periodicity of 40ms was used. The system bandwidth as well as the TRS bandwidth was 100MHz unless stated otherwise. The channel used was either CDL-A or TDL-A. The Doppler spread was 100ns unless stated otherwise.

- Appendix C, Simulation results in tabular form for 10 slot lag

Table 5 Autocorrelation amplitude estimation probability distribution in percent over reporting quantization levels averaging over 4 consecutive measurement occasions each separated by 40ms for the TDL-A channel with a delay spread of 100ns for 3.5GHz carrier frequency, 100MHz bandwidth and 30kHz subcarrier spacing. The correlation lag used was 10 slots.

| UE speed[km/h] and autocorrelation | SNR [dB] | Q15 0.2929 | Q14 0.5000 | Q13 0.6464 | Q12 0.7500 | Q11 0.8232 | Q10 0.8750 | Q9 0.9116 | Q8 0.9375 | Q7 0.9558 | Q6 0.9688 | Q5 0.9779 | Q4 0.9844 | Q3 0.9890 | Q2 0.9922 | Q1 0.9945 | Q0 0.9961 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2km/h, ac=0.98966 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.19489 | 4.9664 | 47.46 | 44.785 | 2.5941 | 0 |
| 2km/h, ac=0.98966 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.14113 | 3.1586 | 36.243 | 52.997 | 7.2984 | 0.16129 |
| 2km/h, ac=0.98966 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.10081 | 1.9422 | 21.022 | 47.601 | 22.923 | 6.4113 |
| 2km/h, ac=0.98966 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.0067204 | 0.10753 | 3.172 | 15.141 | 26.714 | 20.255 | 34.603 |
| 3km/h, ac=0.97681 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.42339 | 15.094 | 65.793 | 18.32 | 0.36962 | 0 | 0 | 0 |
| 3km/h, ac=0.97681 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.36962 | 12.392 | 63.145 | 23.394 | 0.69892 | 0 | 0 | 0 |
| 3km/h, ac=0.97681 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.25538 | 8.7702 | 55.074 | 33.239 | 2.5672 | 0.087366 | 0.0067204 | 0 |
| 3km/h, ac=0.97681 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.23522 | 7.6411 | 39.133 | 36.445 | 10.974 | 3.0108 | 1.1626 | 1.3978 |
| 5km/h, ac=0.93626 | 20 | 0 | 0 | 0 | 0 | 0 | 2.8159 | 20.43 | 43.81 | 27.843 | 4.7782 | 0.32258 | 0 | 0 | 0 | 0 | 0 |
| 5km/h, ac=0.93626 | 10 | 0 | 0 | 0 | 0 | 0 | 2.668 | 19.745 | 43.199 | 28.703 | 5.3159 | 0.3629 | 0.0067204 | 0 | 0 | 0 | 0 |
| 5km/h, ac=0.93626 | 5 | 0 | 0 | 0 | 0 | 0 | 2.4059 | 18.246 | 42.238 | 29.94 | 6.6465 | 0.51075 | 0.013441 | 0 | 0 | 0 | 0 |
| 5km/h, ac=0.93626 | 0 | 0 | 0 | 0 | 0 | 0 | 2.1505 | 16.431 | 39.039 | 31.727 | 9.4422 | 1.082 | 0.10753 | 0.020161 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 20 | 0 | 0 | 0 | 0.18817 | 11.868 | 58.622 | 27.836 | 1.4852 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 10 | 0 | 0 | 0 | 0.17473 | 11.499 | 58.038 | 28.609 | 1.6801 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 5 | 0 | 0 | 0 | 0.20833 | 10.625 | 57.043 | 30.168 | 1.9556 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 0 | 0 | 0 | 0 | 0.14785 | 9.6169 | 52.863 | 34.22 | 3.1452 | 0.0067204 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 20 | 0 | 0.040323 | 9.2003 | 53.091 | 34.919 | 2.6949 | 0.053763 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 10 | 0 | 0.040323 | 9.0121 | 52.614 | 35.484 | 2.789 | 0.060484 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 5 | 0 | 0.040323 | 8.797 | 51.922 | 36.19 | 2.9906 | 0.060484 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 0 | 0 | 0.040323 | 8.0511 | 50.06 | 37.789 | 3.9852 | 0.073925 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 20 | 3.7366 | 62.796 | 32.312 | 1.1559 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 10 | 3.7097 | 62.52 | 32.507 | 1.2634 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 5 | 3.5887 | 62.056 | 33.051 | 1.3038 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 0 | 3.4812 | 60.249 | 34.516 | 1.7473 | 0.0067204 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 20 | 73.797 | 25.968 | 0.23522 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 10 | 73.636 | 26.129 | 0.23522 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 5 | 73.125 | 26.647 | 0.22849 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 0 | 71.082 | 28.595 | 0.32258 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 6 Autocorrelation amplitude estimation probability distribution in percent over reporting quantization levels averaging over 4 consecutive measurement occasions each separated by 40ms for the TDL-A channel with a delay spread of 30ns for 3.5GHz carrier frequency, 100MHz bandwidth and 30kHz subcarrier spacing. The correlation lag used was 10 slots.

| UE speed and autocorrelation | SNR [dB] | Q15 0.2929 | Q14 0.5000 | Q13 0.6464 | Q12 0.7500 | Q11 0.8232 | Q10 0.8750 | Q9 0.9116 | Q8 0.9375 | Q7 0.9558 | Q6 0.9688 | Q5 0.9779 | Q4 0.9844 | Q3 0.9890 | Q2 0.9922 | Q1 0.9945 | Q0 0.9961 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2km/h, ac=0.98966 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.32258 | 1.0148 | 9.5968 | 42.634 | 37.117 | 8.5013 | 0.81317 |
| 2km/h, ac=0.98966 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.31586 | 1.0013 | 9.5699 | 41.19 | 37.937 | 8.8978 | 1.0887 |
| 2km/h, ac=0.98966 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.013441 | 0.33602 | 1.0349 | 9.3884 | 38.999 | 38.233 | 10.444 | 1.5524 |
| 2km/h, ac=0.98966 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.020161 | 0.35618 | 1.1022 | 10.074 | 33.454 | 34.348 | 14.456 | 6.1895 |
| 3km/h, ac=0.97681 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.30914 | 2.6613 | 19.906 | 51.358 | 22.823 | 2.7554 | 0.18817 | 0 | 0 |
| 3km/h, ac=0.97681 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.30242 | 2.6613 | 19.845 | 50.638 | 23.448 | 2.9032 | 0.20161 | 0 | 0 |
| 3km/h, ac=0.97681 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.32258 | 2.6142 | 19.516 | 49.53 | 24.368 | 3.4476 | 0.18145 | 0.020161 | 0 |
| 3km/h, ac=0.97681 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.0067204 | 0.34274 | 2.6815 | 18.555 | 46.001 | 26.761 | 4.9664 | 0.57124 | 0.10081 | 0.013441 |
| 5km/h, ac=0.93626 | 20 | 0 | 0 | 0 | 0 | 0.91398 | 5.6989 | 20.054 | 33.985 | 27.016 | 10.397 | 1.6801 | 0.25538 | 0 | 0 | 0 | 0 |
| 5km/h, ac=0.93626 | 10 | 0 | 0 | 0 | 0 | 0.94758 | 5.6586 | 19.98 | 33.911 | 26.922 | 10.659 | 1.6532 | 0.26882 | 0 | 0 | 0 | 0 |
| 5km/h, ac=0.93626 | 5 | 0 | 0 | 0 | 0 | 0.93414 | 5.625 | 19.798 | 33.79 | 27.204 | 10.659 | 1.7204 | 0.26882 | 0 | 0 | 0 | 0 |
| 5km/h, ac=0.93626 | 0 | 0 | 0 | 0 | 0 | 0.9207 | 5.504 | 19.637 | 32.789 | 27.392 | 11.324 | 2.0901 | 0.34274 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 20 | 0 | 0 | 0.040323 | 0.93414 | 17.016 | 43.77 | 32.332 | 5.7661 | 0.12769 | 0.013441 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 10 | 0 | 0 | 0.040323 | 0.9543 | 16.888 | 43.642 | 32.406 | 5.9207 | 0.13441 | 0.013441 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 5 | 0 | 0 | 0.040323 | 0.97446 | 16.808 | 43.649 | 32.372 | 6.0081 | 0.14113 | 0.0067204 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 0 | 0 | 0 | 0.033602 | 1.0282 | 16.418 | 43.233 | 32.681 | 6.4315 | 0.14785 | 0.026882 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 20 | 0 | 0.5578 | 12.124 | 41.815 | 35.531 | 9.3347 | 0.63844 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 10 | 0 | 0.53091 | 12.11 | 41.781 | 35.511 | 9.3817 | 0.68548 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 5 | 0 | 0.55108 | 11.989 | 41.761 | 35.383 | 9.6237 | 0.6922 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 0 | 0 | 0.51075 | 12.016 | 41.055 | 35.948 | 9.7245 | 0.73925 | 0.0067204 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 20 | 3.0175 | 48.313 | 42.278 | 6.3306 | 0.060484 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 10 | 3.0108 | 48.165 | 42.379 | 6.371 | 0.073925 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 5 | 3.0175 | 47.95 | 42.608 | 6.3374 | 0.087366 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 0 | 3.004 | 47.359 | 42.675 | 6.8817 | 0.080645 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 20 | 42.09 | 53.884 | 3.9919 | 0.033602 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 10 | 41.902 | 53.999 | 4.0591 | 0.040323 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 5 | 41.727 | 54.133 | 4.0927 | 0.047043 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 0 | 41.035 | 54.637 | 4.2675 | 0.060484 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 7 Autocorrelation amplitude estimation probability distribution in percent over reporting quantization levels with no averaging over measurement occasions, for the TDL-A channel with a delay spread of 100ns for 3.5GHz carrier frequency, 100MHz bandwidth and 30kHz subcarrier spacing. The correlation lag used was 10 slots.

| UE speed[km/h] and autocorrelation | SNR [dB] | Q15 0.2929 | Q14 0.5000 | Q13 0.6464 | Q12 0.7500 | Q11 0.8232 | Q10 0.8750 | Q9 0.9116 | Q8 0.9375 | Q7 0.9558 | Q6 0.9688 | Q5 0.9779 | Q4 0.9844 | Q3 0.9890 | Q2 0.9922 | Q1 0.9945 | Q0 0.9961 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2km/h, ac=0.98966 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.19333 | 1.3467 | 3.9867 | 13.033 | 25.233 | 26.9 | 17.847 | 11.46 |
| 2km/h, ac=0.98966 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.18667 | 1.16 | 3.5733 | 10.56 | 22.807 | 26.287 | 19.213 | 16.213 |
| 2km/h, ac=0.98966 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.17333 | 0.97333 | 3.0667 | 8.4133 | 17.86 | 22.38 | 19.107 | 28.027 |
| 2km/h, ac=0.98966 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.0066667 | 0.046667 | 0.30667 | 1.3 | 4.24 | 9.1067 | 13.54 | 14.9 | 13.5 | 43.053 |
| 3km/h, ac=0.97681 | 20 | 0 | 0 | 0 | 0 | 0 | 0.053333 | 0.29333 | 1.98 | 6.6067 | 20.413 | 26.94 | 24.067 | 12.393 | 4.9667 | 1.82 | 0.46667 |
| 3km/h, ac=0.97681 | 10 | 0 | 0 | 0 | 0 | 0 | 0.033333 | 0.3 | 1.8267 | 5.98 | 18.847 | 26.567 | 24.04 | 13.787 | 5.72 | 2.1933 | 0.70667 |
| 3km/h, ac=0.97681 | 5 | 0 | 0 | 0 | 0 | 0 | 0.046667 | 0.25333 | 1.6333 | 5.3733 | 16.54 | 24.827 | 23.733 | 15.447 | 7.28 | 2.94 | 1.9267 |
| 3km/h, ac=0.97681 | 0 | 0 | 0 | 0 | 0 | 0 | 0.033333 | 0.28667 | 1.8867 | 5.2733 | 14.387 | 21.16 | 19.967 | 14.487 | 8.3733 | 4.6 | 9.5467 |
| 5km/h, ac=0.93626 | 20 | 0 | 0 | 0 | 0.19333 | 1.9867 | 7.2467 | 19.22 | 25.52 | 24.807 | 13.567 | 5.76 | 1.4267 | 0.27333 | 0 | 0 | 0 |
| 5km/h, ac=0.93626 | 10 | 0 | 0 | 0 | 0.18667 | 1.9467 | 7.0667 | 18.793 | 25.153 | 24.7 | 14.313 | 5.82 | 1.7267 | 0.28667 | 0.0066667 | 0 | 0 |
| 5km/h, ac=0.93626 | 5 | 0 | 0 | 0 | 0.18667 | 1.8467 | 6.8133 | 17.627 | 24.987 | 24.673 | 14.787 | 6.6733 | 1.9733 | 0.36667 | 0.066667 | 0 | 0 |
| 5km/h, ac=0.93626 | 0 | 0 | 0 | 0.0066667 | 0.20667 | 1.8 | 6.4533 | 16.64 | 23.62 | 23.233 | 15.747 | 7.6933 | 2.9933 | 0.96667 | 0.22 | 0.1 | 0.32 |
| 7km/h, ac=0.877 | 20 | 0 | 0.1 | 1.2333 | 6.12 | 17.707 | 26.473 | 26.467 | 14.553 | 5.5733 | 1.5667 | 0.20667 | 0 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 10 | 0 | 0.093333 | 1.2133 | 6.0133 | 17.487 | 26.473 | 26.267 | 14.773 | 5.8 | 1.66 | 0.22 | 0 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 5 | 0 | 0.1 | 1.1867 | 5.82 | 17.193 | 25.887 | 26.407 | 15.253 | 6.0067 | 1.84 | 0.28667 | 0.02 | 0 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 0 | 0 | 0.1 | 1.1733 | 5.8467 | 16.007 | 24.92 | 25.827 | 16.32 | 6.8933 | 2.3 | 0.51333 | 0.073333 | 0.013333 | 0.0066667 | 0 | 0.0066667 |
| 10km/h, ac=0.75718 | 20 | 0.39333 | 4.48 | 16.727 | 28.773 | 28.213 | 14.827 | 5.46 | 1.0267 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 10 | 0.38667 | 4.52 | 16.413 | 28.86 | 28.04 | 15.06 | 5.4933 | 1.1133 | 0.11333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 5 | 0.39333 | 4.4667 | 16.353 | 28.527 | 27.993 | 15.293 | 5.5933 | 1.2867 | 0.093333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 0 | 0.38 | 4.4133 | 15.873 | 27.58 | 27.76 | 15.88 | 6.38 | 1.5267 | 0.20667 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 20 | 19.607 | 34.753 | 29.047 | 12.66 | 3.56 | 0.37333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 10 | 19.507 | 34.78 | 29.007 | 12.72 | 3.6067 | 0.38 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 5 | 19.393 | 34.493 | 29.153 | 12.847 | 3.7 | 0.41333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 0 | 19.08 | 33.52 | 29.127 | 13.493 | 4.1933 | 0.57333 | 0.013333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 20 | 62.567 | 28.52 | 7.9267 | 0.97333 | 0.013333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 10 | 62.407 | 28.527 | 8.06 | 1 | 0.0066667 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 5 | 62.147 | 28.693 | 8.12 | 1.02 | 0.02 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 0 | 60.987 | 28.873 | 8.9133 | 1.1867 | 0.04 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 8 Autocorrelation amplitude estimation probability distribution in percent over reporting quantization levels with no averaging over measurement occasions, for the TDL-A channel with a delay spread of 30ns for 3.5GHz carrier frequency, 100MHz bandwidth and 30kHz subcarrier spacing. The correlation lag used was 10 slots.

| UE speed and autocorrelation | SNR [dB] | Q15 0.2929 | Q14 0.5000 | Q13 0.6464 | Q12 0.7500 | Q11 0.8232 | Q10 0.8750 | Q9 0.9116 | Q8 0.9375 | Q7 0.9558 | Q6 0.9688 | Q5 0.9779 | Q4 0.9844 | Q3 0.9890 | Q2 0.9922 | Q1 0.9945 | Q0 0.9961 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2km/h, ac=0.98966 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0.08 | 0.22 | 0.88667 | 2.5667 | 6.0667 | 13.007 | 17.98 | 19.92 | 16.993 | 22.28 |
| 2km/h, ac=0.98966 | 10 | 0 | 0 | 0 | 0 | 0 | 0.0066667 | 0.073333 | 0.21333 | 0.88667 | 2.5667 | 6.0133 | 12.813 | 17.893 | 19.46 | 17.087 | 22.987 |
| 2km/h, ac=0.98966 | 5 | 0 | 0 | 0 | 0 | 0 | 0.0066667 | 0.073333 | 0.19333 | 0.89333 | 2.72 | 5.9267 | 12.293 | 17.333 | 19.333 | 16.74 | 24.487 |
| 2km/h, ac=0.98966 | 0 | 0 | 0 | 0 | 0 | 0 | 0.013333 | 0.086667 | 0.24 | 0.88 | 2.7867 | 6.8333 | 11.627 | 16.033 | 16.427 | 14.173 | 30.9 |
| 3km/h, ac=0.97681 | 20 | 0 | 0 | 0 | 0.013333 | 0.093333 | 0.25333 | 1.5667 | 3.7667 | 8.3733 | 15.36 | 19.067 | 19.92 | 15.793 | 8.94 | 4.0533 | 2.8 |
| 3km/h, ac=0.97681 | 10 | 0 | 0 | 0 | 0.013333 | 0.08 | 0.28667 | 1.5267 | 3.7333 | 8.2733 | 15.253 | 19.26 | 19.92 | 15.487 | 9.0667 | 4.2333 | 2.8667 |
| 3km/h, ac=0.97681 | 5 | 0 | 0 | 0 | 0.0066667 | 0.073333 | 0.27333 | 1.5533 | 3.7133 | 8.38 | 14.947 | 19.007 | 19.727 | 15.3 | 9.2467 | 4.5133 | 3.26 |
| 3km/h, ac=0.97681 | 0 | 0 | 0 | 0 | 0.013333 | 0.086667 | 0.28667 | 1.3733 | 3.9533 | 8.4067 | 14.307 | 17.847 | 18.573 | 15.34 | 9.68 | 4.9867 | 5.1467 |
| 5km/h, ac=0.93626 | 20 | 0 | 0.026667 | 0.28667 | 1.2533 | 3.78 | 8.2267 | 14.74 | 19.173 | 20.193 | 16.353 | 9.3867 | 4.06 | 1.9533 | 0.49333 | 0.073333 | 0 |
| 5km/h, ac=0.93626 | 10 | 0 | 0.02 | 0.3 | 1.2467 | 3.7533 | 8.1667 | 14.7 | 19.113 | 20.18 | 16.44 | 9.3867 | 4.1733 | 1.94 | 0.51333 | 0.066667 | 0 |
| 5km/h, ac=0.93626 | 5 | 0 | 0.02 | 0.3 | 1.2133 | 3.92 | 8.02 | 14.78 | 18.673 | 20.28 | 16.46 | 9.56 | 4.02 | 2.1133 | 0.52667 | 0.11333 | 0 |
| 5km/h, ac=0.93626 | 0 | 0 | 0.026667 | 0.29333 | 1.3467 | 3.7867 | 7.8933 | 14.447 | 18.447 | 20.053 | 16.453 | 9.5933 | 4.5333 | 2.1933 | 0.7 | 0.15333 | 0.08 |
| 7km/h, ac=0.877 | 20 | 0.04 | 0.72667 | 3.1133 | 7.4133 | 14.34 | 20.047 | 21.74 | 16.42 | 9.7467 | 4.28 | 1.7933 | 0.31333 | 0.026667 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 10 | 0.053333 | 0.68 | 3.1667 | 7.4 | 14.26 | 20.087 | 21.513 | 16.6 | 9.7333 | 4.3467 | 1.8 | 0.32 | 0.04 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 5 | 0.053333 | 0.72 | 3.16 | 7.4533 | 14.093 | 19.807 | 21.727 | 16.593 | 9.7 | 4.5133 | 1.78 | 0.36 | 0.04 | 0 | 0 | 0 |
| 7km/h, ac=0.877 | 0 | 0.046667 | 0.72667 | 3.0733 | 7.4333 | 14.02 | 19.8 | 21.053 | 16.833 | 9.9333 | 4.6533 | 1.94 | 0.43333 | 0.046667 | 0.0066667 | 0 | 0 |
| 10km/h, ac=0.75718 | 20 | 1.7467 | 6.6867 | 14.453 | 21.573 | 22.973 | 17.84 | 9.48 | 3.8333 | 1.2533 | 0.16 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 10 | 1.7467 | 6.7067 | 14.347 | 21.553 | 22.927 | 17.967 | 9.44 | 3.9067 | 1.2333 | 0.17333 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 5 | 1.78 | 6.6867 | 14.2 | 21.547 | 22.787 | 17.907 | 9.7133 | 3.9267 | 1.2933 | 0.15333 | 0.0066667 | 0 | 0 | 0 | 0 | 0 |
| 10km/h, ac=0.75718 | 0 | 1.8333 | 6.7 | 13.987 | 21.267 | 22.653 | 18.247 | 9.7933 | 3.9133 | 1.42 | 0.15333 | 0.033333 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 20 | 18.987 | 27.16 | 25.7 | 17.253 | 8.0267 | 2.3933 | 0.46667 | 0.013333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 10 | 18.953 | 27.107 | 25.687 | 17.327 | 7.9667 | 2.4667 | 0.47333 | 0.02 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 5 | 18.987 | 26.967 | 25.84 | 17.02 | 8.26 | 2.4267 | 0.48667 | 0.013333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 15km/h, ac=0.49662 | 0 | 18.907 | 26.893 | 25.573 | 17.14 | 8.32 | 2.6267 | 0.51333 | 0.026667 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 20 | 45.76 | 32.473 | 16.14 | 4.72 | 0.84667 | 0.06 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 10 | 45.727 | 32.347 | 16.28 | 4.7467 | 0.84 | 0.06 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 5 | 45.66 | 32.4 | 16.193 | 4.78 | 0.89333 | 0.073333 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 20km/h, ac=0.20305 | 0 | 45.233 | 32.567 | 16.347 | 4.86 | 0.90667 | 0.086667 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Note that for spatial channel models fmax isn’t a useful measure of Doppler spread as should be obvious from Figure 4.