3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316868

# Xiamen, China, October 9th – October 13th, 2023

# Title:	Initial coexistence simulation results for above 10 GHz and related requirements

# Source:	THALES, Magister Solutions Ltd

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	5.26.2

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

At RAN4#105 the following WFs have been approved:

- R4-2220239 (source: THALES, “WF for above 10GHz band definition and system parameters”) with the latest references for Ka-Band frequency bands;

- R4-2220241 (source: Samsung, “Simulation assumptions for NTN co-existence study in bands above 10GHz”) with the latest discussions for the coexistence studies.

Agreements R4-2220239 1/ Companies are invited to study (general) ITU/national regulations and bring contributions/provide more information at this meeting (RAN#105) and next RAN4 meeting. 2/ Since it has been agreed to define several NTN Ka-bands to address the diversity of spectrum allocation (see agreement 15/11/2022 during online session), moderator proposes recommendation based on Option 2b. Option 2b (as starting point for next meeting discussion): RAN4 to consider defining/continue to discuss at next meeting (all options have same priority, companies need more time to check): - n511 with consideration of US/FCC regulations. - [n512 with consideration of CEPT regulations.] Note 1: All companies to further check impacts of latest revision of ECC Decision(05)01 on ECC Decision(13)01 cited in the current WF. - [n510 with consideration of US/FCC regulations.] Note 1: Provide the 3GPP definition that specifies the prevention of the use of a fixed terminal in the Fixed Service Spectrum (FSS). Note 2: Also provide information of a movable NTN user terminal in FSS spectrum in the US. with the following considerations: - DL: 17.7-20.2 GHz (n512, n511, n510); - UL: 27.5-30.0 GHz (n512), 28.35-30.0 GHz (n511), 27.5-28.35 GHz (n510).

Agreements R4-2220241 Table 1.1-1 Aggressor and victim combination No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; NTN UL; TN UL. 2; TN with NTN; TN UL; NTN UL. 3; TN with NTN; NTN UL; TN DL. 4; TN with NTN; TN DL; NTN UL. 5; TN with NTN; TN DL; NTN DL. 6; TN with NTN; NTN DL; TN DL. 7; TN with NTN; NTN DL; TN UL. 8; TN with NTN; TN UL; NTN DL. NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified..

Moreover, at RAN4#106 the following agreements have been made:

Agreement 1: Study NTN-TN coexistence by assuming a reference frequency of 17 GHz for NTN DL cases and 27 GHz NTN UL cases, as well as the consideration of ACLR and ACS assumptions as following:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Furthermore, for the assumption on TN ACLR/ACS for co-existence simulation, the values for 17 GHz in below table are considered as starting point for co-existence simulation purpose yet other options not precluded.

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Agreement 2: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands with the following figures:

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

The following WF documents shall be also taken into account:

- R4-2217468, WF on [313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #104-bis-e, Electronic Meeting, 10 - 19 October, 2022;

- R4-2220241, Way forward on [105][313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, November 14 – November 18, 2022;

- R4-2302878, WF for above 10GHz NTN-TN co-existence study (Samsung), 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023.

In previous contribution R4-2302535 (“NTN Simulation Parameters for above 10 GHz Coexistence Studies”, THALES), several NTN simulation parameters were proposed for above 10 GHz coexistence studies. The contribution R4-2305847 (THALES, Magister Solutions Ltd) provided material for discussion with respect to Ka-Band NTN-TN coexistence simulations and NTN UE terminal and Satellite antenna parameters. Parameters from R4-2302535 then were updated in R4-2305847, together with initial simulation results.

For other information, please also check (RAN4#106-bis-e):

- R4-2306002 (“WF on [311] NR_NTN_enh_Part3”, Samsung):

- R4-2306003 (“Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung).

And also (at RAN4#107):

- R4-2309700 (“Initial NTN calibration results for above 10 GHz Coexistence Studies”, THALES, Magister Solutions Ltd);

- R4-2309768 (“Collection table for NTN co-existence in above 10GHz calibration results”, Samsung);

- R4-2310449 (“Summary for [107][311] NR_NTN_enh_Part3”, Samsung);

- RAN4_107_BSRF_Demod_Test_Session Report_11_EOM.docx;

- R4-2309771 (“Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung);

- R4-2309767 (“WF for co-existence study of Above 10GHz NTN band”, Samsung);

The following agreements can be considered for reference only:

- R4-2309770 (“WF on above 10GHz SAN RF requirements“, CATT);

- R4-2309766 (“WF for system parameters on Ka band”, THALES).

With respect to RAN4#108 contributions and WFs, please find the following relevant contributions:

- R4-2312443 (“Updates on NTN calibration and coexistence simulation results for above 10 GHz”), THALES, Magister Solutions Ltd, RAN4#108, Toulouse, France, with a very complete set of results.

- R4-2313865 (“WF for NTN co-existence study”), Samsung, Cybercore, RAN4#108, Toulouse, France;

- R4-2313890 (“Simulation assumption for NTN co-existence study”), Samsung, RAN4#108, Toulouse, France.

The scope of this document is to provide initial simulation results for NTN-TN coexistence scenarios in above 10 GHz and related initial derived requirements.

# Few proposals for the WF/parameter update

Observation 1: It does not seem realistic (there is not such FR2 NR deployment to our knowledge) entirely covering an NTN beam, especially in GEO scenario.

- This is particularly important for instance for Scenario #4 & Scenario #8 (“All active TN cells in central NTN beam”) or

- Scenario #2 (with “Only the active TN cells in central NTN beam”) – consider the active TN cells from all clusters? or the active TN cells from only one cluster?

Observation 2: Especially at lower elevation angles (i.e. 25°) – value decided at RAN4#108 (as mean value between 20° and 30° elevation angle) the satellite beam footprint is much larger than the satellite beam footprint at 90°.

Observation 3: For this reason, the scaling factor applied for TN deployment for 25° NTN elevation angle is much higher than the scaling factor applied for TN deployment for 90° NTN elevation angle:

Scaling Factor

| Orbit | 90° | 25° |
| --- | --- | --- |
| LEO600 | 13.8 dB | 23.5 dB |
| LEO1200 | 19.6 dB | 28.6 dB |
| GEO | 29.1 dB | 33.4 dB |

Note: Please also note that for previous results, 20% TN activity was assumed as agreed.

Proposal 1: Consider a cellular Terrestrial Network (TN) not larger than 50 km diameter.

Proposal 2: Companies to check/compare scaling factor values.

Proposal 3: Companies to use same scaling factor values for lower NTN SAN elevation angle (e.g. 25°) and 90° NTN SAN elevation angle.

Scaling Factor

| Orbit | 90° | 25° |
| --- | --- | --- |
| LEO600 | 13.8 dB | 13.8 dB |
| LEO1200 | 19.6 dB | 19.6 dB |
| GEO | 29.1 dB | 29.1 dB |

Observation 4: Same as for FR1 TN-NTN coexistence simulations, simulations in above 10 GHz TN-NTN coexistence simulations show that NTN SAN and NTN UE are both victims. Therefore, once more, the NTN is a potential victim of the TN (acting as aggressor) and not vice-versa. This may be explained by the high density scenarios assumed by TN, but not only.

Proposal 4: Companies to focus on 90° elevation angle. If a second value is still required, companies are encouraged to decide use e.g. 45° elevation angle.

Proposal 5: RAN4 to increase hypothetical TN requirements (which are not currently specified by any TN specification since such TN deployment does not exist) at least with 3 more dBs:

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] + 3dB | [26] + 3dB | [19] + 3dB | [25] + 3dB | [23.8] +3dB | [18.2] +3dB |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Proposal 6: If + 3 dBs TN ACLR/ACS are not sufficient increase even more the TN requirements at 17 GHz (currently there is no TN specification on this frequency band).

Proposal 7: Alternatively, RAN4 could also decide to increase the guardband of NTN CBW.

Proposal 8: Alternatively, RAN4 could also decide to consider a different ACLR model from the fixed one.

Proposal 9: Other options shall not be precluded in order to have more realistic assumptions for the deployment.

Moreover, it seems that with the new RAN4#108 definition for VSAT UE antenna pattern (circular aperture) there is no definition for antenna pattern > 90° angle. We therefore propose to go back to previous definition (which was actually the same) but with larger defined antenna pattern.

Proposal 10: RAN4 to consider a Fall-Back to previous definition of VSAT UE antenna pattern (which was actually the same but with larger defined antenna pattern over angles >90°).

Unfortunately, this additional change will most probably affect scenario 8 in a negative manner. Therefore, it is very important to consider also other aspects and assumptions from different proposals – a combination of several proposals should be envisaged.

In order to recall the previous definition (please also refer to R4-2309771 “Simulation assumptions for NTN co-existence study in bands above 10GHz) from RAN4#107”), the following normalised antenna pattern corresponding to a theoretical circular aperture can be considered for coexistence analysis:

Fu=2J1uu

where:

- J1x  is the Bessel function of first type and first order with argument x,

-  is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ),

- D : antenna diameter,

- : wavelength.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

10 logFu2

With the linear form given by the following relation:

Fu2=2J1uu2

This is equivalent to (however previous equation from RAN4#107 is defined on the entire range and not only from -90° to +90°):

where:

- J1x is the Bessel function of the first kind and first order with argument x,

- a, is the radius of the antenna's circular aperture;

- k = 2πf/c is the wave number;

- f is the frequency of operation;

- c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that k x a equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

Proposal 11: RAN4 may also consider the NTN UL TxP increase as a potential way forward to improve LEO orbits in scenario 4.

Please also note that for all simulations in the next section the following assumptions are considered:

- 1.5 m NTN VSAT height;

- Option 1 for the Noise Figure values:

  - NTN UE with 2.5 dB NF;

  - NTN SAN with 3.5 dB NF.

Otherwise, everything else is as agreed in R4-2313865 and R4-2313890.

# Updated Simulation Results

Scenario 1 – NTN UL to TN UL

Scenario 1 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC1 | 27 | NTN UL | TN UL | NTN UE ACLR | TN gNB ACS 24 dB |

Observations Scenario 1:

- For this scenario, the coexistence requirements are relaxed

- 90° and 25° SAN elevation cases are most challenging but still < 1% throughput loss with 10 dB UE ACLR

- Low number of NTN UEs decreases ACI

Scenario 2 – TN UL to NTN UL

Scenario 2 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC2 | 27 | TN UL | NTN UL | NTN SAN ACS | TN UE ACLR 17 dB |

Observations Scenario 2:

- 10 dB SAN ACS provides < 3% throughput loss with 25° SAN elevation angle with scaling enabled

- With 25° SAN elevation case, about ~25% TN UEs have > 0 dBi ACI Tx antenna gain towards SAN

Scenario 3 – NTN UL to TN DL

Scenario 3 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC3 | 27 | NTN UL | TN DL | NTN UE ACLR | TN UE ACS 23 dB |

Observations Scenario 3:

- For this scenario, the coexistence requirements are relaxed

- 90° and 25° SAN elevation cases are most challenging but still < 1% throughput loss with 10 dB UE ACLR

- Low number of NTN UEs decrease the likelihood of NTN UE and TN UE being close

Scenario 4 – TN DL to NTN UL

Scenario 4 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC4 | 27 | TN DL | NTN UL | NTN SAN ACS | TN gNB ACLR 28 dB |

Observations Scenario 4:

- Very challenging ACI for NTN

- ~30 dB requirement SAN ACS for 90° SAN elevation case

- 25° SAN elevation case unfeasible with the current (unrealistic TN deployment) assumptions

Scenario 5 – TN DL to NTN DL

Scenario 5 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC5 | 17 | TN DL | NTN DL | NTN UE ACS | TN gNB ACLR 30 dB |

Observations Scenario 5:

- 25° and 90° SAN elevation cases give similar requirements

- ~20 dB NTN UE ACS requirement

- Assumed cluster orientation is not the worst case in 25° SAN elevation case: NTN UE and gNB boresight never pointing to each other.

Scenario 6 – NTN DL to TN DL

Scenario 6 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC6 | 17 | NTN DL | TN DL | NTN SAN ACLR | TN UE ACS 25 dB |

Observations Scenario 6:

- For this scenario, the coexistence requirements are (very) relaxed

- Very large ACI margin noticed

Scenario 7 – NTN DL to TN UL

Scenario 7 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC7 | 17 | NTN DL | TN UL | NTN SAN ACLR | TN gNB ACS 26 dB |

Observations Scenario 7:

- For this scenario, the coexistence requirements are relaxed

Scenario 8 – TN UL to NTN DL

Scenario 8 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC8 | 17 | TN UL | NTN DL | NTN UE ACS | TN UE ACLR 19 dB |

Observations Scenario 8:

- 570 TN UEs interfering 10 NTN UEs -> High change that TN UE(s) close to a NTN UE

- 19 dB TN UE ACLR is quite low. Not much to gain with 20+ dB NTN UE ACS.

- However, 17 GHz TN requirements are not specified by any kind of specification, so they could be increased to accommodate with NTN requirements.

# Results

Results: conclusions and sensitivity analysis from R4-2312443 (RAN4#108)

NTN ACLR / ACS requirements from co-existence simulations

| NTN SAN ACLR | < 10 dB |
| --- | --- |
| NTN SAN ACS | < 10 dB |
| NTN UE ACLR | ~10 dB |
| NTN UE ACS | ~20 dB |

However, this simulation campaign is the first iteration, and the work will continue in 3GPP. There are still many (on-going) assumptions that can change the results, including the density of the terrestrial network, and the UL power control scheme.

Updated Results: conclusions and sensitivity analysis at RAN4#108-bis

NTN ACLR / ACS requirements from co-existence simulations

| Parameter | LEO 90° | GEO 90° | LEO 25° | GEO 25° |
| --- | --- | --- | --- | --- |
| NTN SAN ACLR [dB] | 10 | 10 | 10 | 10 |
| NTN SAN ACS [dB] | 30 | 30 | TBD | TBD |
| NTN UE ACLR [dB] | 10 | 10 | 10 | 10 |
| NTN UE ACS [dB] | 22 | 20 | TBD | 30 |

The final requirements may slightly change depending on the latest updates decided at this meeting and also after computing average values between different companies.

Proposal 12: The proposed parameters for ACLR and ACS requirements resulted from coexistence analysis in above 10 GHz are:

| Parameter | LEO | GEO |
| --- | --- | --- |
| NTN SAN ACLR [dB] | 10 | 10 |
| NTN SAN ACS [dB] | 30 | 30 |
| NTN UE ACLR [dB] | 10 | 10 |
| NTN UE ACS [dB] | 30 | 30 |

# Conclusions

Observation 1: It does not seem realistic (there is not such FR2 NR deployment to our knowledge) entirely covering an NTN beam, especially in GEO scenario.

- This is particularly important for instance for Scenario #4 & Scenario #8 (“All active TN cells in central NTN beam”) or

- Scenario #2 (with “Only the active TN cells in central NTN beam”) – consider the active TN cells from all clusters? or the active TN cells from only one cluster?

Observation 2: Especially at lower elevation angles (i.e. 25°) – value decided at RAN4#108 (as mean value between 20° and 30° elevation angle) the satellite beam footprint is much larger than the satellite beam footprint at 90°.

Observation 3: For this reason, the scaling factor applied for TN deployment for 25° NTN elevation angle is much higher than the scaling factor applied for TN deployment for 90° NTN elevation angle:

Scaling Factor

| Orbit | 90° | 25° |
| --- | --- | --- |
| LEO600 | 13.8 dB | 23.5 dB |
| LEO1200 | 19.6 dB | 28.6 dB |
| GEO | 29.1 dB | 33.4 dB |

Proposal 1: Consider a cellular Terrestrial Network (TN) not larger than 50 km diameter.

Proposal 2: Companies to check/compare scaling factor values.

Proposal 3: Companies to use same scaling factor values for lower NTN SAN elevation angle (e.g. 25°) and 90° NTN SAN elevation angle.

Scaling Factor

| Orbit | 90° | 25° |
| --- | --- | --- |
| LEO600 | 13.8 dB | 13.8 dB |
| LEO1200 | 19.6 dB | 19.6 dB |
| GEO | 29.1 dB | 29.1 dB |

Observation 4: Same as for FR1 TN-NTN coexistence simulations, simulations in above 10 GHz TN-NTN coexistence simulations show that NTN SAN and NTN UE are both victims. Therefore, once more, the NTN is a potential victim of the TN (acting as aggressor) and not vice-versa. This may be explained by the high density scenarios assumed by TN, but not only.

Proposal 4: Companies to focus on 90° elevation angle. If a second value is still required, companies are encouraged to decide use e.g. 45° elevation angle.

Proposal 5: RAN4 to increase hypothetical TN requirements (which are not currently specified by any TN specification since such TN deployment does not exist) at least with 3 more dBs:

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] + 3dB | [26] + 3dB | [19] + 3dB | [25] + 3dB | [23.8] +3dB | [18.2] +3dB |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Proposal 6: If + 3 dBs TN ACLR/ACS are not sufficient increase even more the TN requirements at 17 GHz (currently there is no TN specification on this frequency band).

Proposal 7: Alternatively, RAN4 could also decide to increase the guardband of NTN CBW.

Proposal 8: Alternatively, RAN4 could also decide to consider a different ACLR model from the fixed one.

Proposal 9: Other options shall not be precluded in order to have more realistic assumptions for the deployment.

Proposal 10: RAN4 to consider a Fall-Back to previous definition of VSAT UE antenna pattern (which was actually the same but with larger defined antenna pattern over angles >90°).

The following normalised antenna pattern corresponding to a theoretical circular aperture can be considered for coexistence analysis:

Fu=2J1uu

where:

- J1x  is the Bessel function of first type and first order with argument x,

-  is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ),

- D : antenna diameter,

- : wavelength.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

10 logFu2

With the linear form given by the following relation:

Fu2=2J1uu2

This is equivalent to (however previous equation from RAN4#107 is defined on the entire range and not only from -90° to +90°):

where:

- J1x is the Bessel function of the first kind and first order with argument x,

- a, is the radius of the antenna's circular aperture;

- k = 2πf/c is the wave number;

- f is the frequency of operation;

- c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that k x a equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

Proposal 11: RAN4 may also consider the NTN UL TxP increase as a potential way forward to improve LEO orbits in scenario 4.

Observations Scenario 1:

- For this scenario, the coexistence requirements are relaxed

- 90° and 25° SAN elevation cases are most challenging but still < 1% throughput loss with 10 dB UE ACLR

- Low number of NTN UEs decreases ACI

Observations Scenario 2:

- 10 dB SAN ACS provides < 3% throughput loss with 25° elevation angle with scaling enabled

- With 25° SAN elevation case, about ~25% TN UEs have > 0 dBi ACI antenna gain

Observations Scenario 3:

- For this scenario, the coexistence requirements are relaxed

- 90° and 25° SAN elevation cases are most challenging but still < 1% throughput loss with 10 dB UE ACLR

- Low number of NTN UEs decrease the likelihood of NTN UE and TN UE being close

Observations Scenario 4:

- Very challenging ACI for NTN

- ~30 dB requirement SAN ACS for 90° SAN elevation case

- 25° SAN elevation case unfeasible with the current (unrealistic TN deployment) assumptions

Observations Scenario 5:

- 25° and 90° SAN elevation cases give similar requirements

- ~20 dB NTN UE ACS requirement

- Assumed cluster orientation is not the worst case in 25° SAN elevation case: NTN UE and gNB never pointing to each other

Observations Scenario 6:

- For this scenario, the coexistence requirements are (very) relaxed

- Very large margin noticed

Observations Scenario 7:

- For this scenario, the coexistence requirements are relaxed

Observations Scenario 8:

- 570 TN UEs interfering 10 NTN UEs -> High change that TN UE(s) close to a NTN UE

- 19 dB TN UE ACLR is quite low. Not much to gain with 20+ dB NTN UE ACS.

- However, 17 GHz TN requirements are not specified by any kind of specification, so they could be increased to accommodate with NTN requirements.

Proposal 12: The proposed parameters for ACLR and ACS requirements resulted from coexistence analysis in above 10 GHz are:

| Parameter | LEO | GEO |
| --- | --- | --- |
| NTN SAN ACLR [dB] | 10 | 10 |
| NTN SAN ACS [dB] | 30 | 30 |
| NTN UE ACLR [dB] | 10 | 10 |
| NTN UE ACS [dB] | 30 | 30 |

# References

- R4-2220239, “WF for above 10GHz band definition and system parameters”, THALES.

- R4-2220241, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”.

- TR 38.811.

- TR 38.821.

- Ka-Band Low Noise Amplifier Sub-System Module for Communication Satellite payload”, S. K. Jain and all.

- ITU-R Rec. P.618-12.

- ITU-R Rec. S.465-6.

- R.C. Hansen “Phased Arrays” in “Antenna Engineering Handbook” by John L. Volakis, editor. McGraw Hill. 4th edition. 2007.

- R4-2305847, “Updated NTN Simulation Parameters for above 10 GHz Coexistence Studies and Initial Simulation Results”, THALES, Magister Solutions Ltd.

- R4-2306002, “WF on [311] NR_NTN_enh_Part3”, Samsung.

- R4-2306003, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

-END-