3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316870

# Xiamen, China, October 9th – October 13th, 2023

# Title:	Calibration updates for above 10 GHz and related information

# Source:	THALES, Magister Solutions Ltd

# Type:	Discussion

# Document for:	Information

# Agenda Item:	5.26.2

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

At RAN4#105 the following WFs have been approved:

- R4-2220239 (source: THALES, “WF for above 10GHz band definition and system parameters”) with the latest references for Ka-Band frequency bands;

- R4-2220241 (source: Samsung, “Simulation assumptions for NTN co-existence study in bands above 10GHz”) with the latest discussions for the coexistence studies.

Agreements R4-2220239 1/ Companies are invited to study (general) ITU/national regulations and bring contributions/provide more information at this meeting (RAN#105) and next RAN4 meeting. 2/ Since it has been agreed to define several NTN Ka-bands to address the diversity of spectrum allocation (see agreement 15/11/2022 during online session), moderator proposes recommendation based on Option 2b. Option 2b (as starting point for next meeting discussion): RAN4 to consider defining/continue to discuss at next meeting (all options have same priority, companies need more time to check): - n511 with consideration of US/FCC regulations. - [n512 with consideration of CEPT regulations.] Note 1: All companies to further check impacts of latest revision of ECC Decision(05)01 on ECC Decision(13)01 cited in the current WF. - [n510 with consideration of US/FCC regulations.] Note 1: Provide the 3GPP definition that specifies the prevention of the use of a fixed terminal in the Fixed Service Spectrum (FSS). Note 2: Also provide information of a movable NTN user terminal in FSS spectrum in the US. with the following considerations: - DL: 17.7-20.2 GHz (n512, n511, n510); - UL: 27.5-30.0 GHz (n512), 28.35-30.0 GHz (n511), 27.5-28.35 GHz (n510).

Agreements R4-2220241 Table 1.1-1 Aggressor and victim combination No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; NTN UL; TN UL. 2; TN with NTN; TN UL; NTN UL. 3; TN with NTN; NTN UL; TN DL. 4; TN with NTN; TN DL; NTN UL. 5; TN with NTN; TN DL; NTN DL. 6; TN with NTN; NTN DL; TN DL. 7; TN with NTN; NTN DL; TN UL. 8; TN with NTN; TN UL; NTN DL. NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified..

Moreover, at RAN4#106 the following agreements have been made:

Agreement 1: Study NTN-TN coexistence by assuming a reference frequency of 17 GHz for NTN DL cases and 27 GHz NTN UL cases, as well as the consideration of ACLR and ACS assumptions as following:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Furthermore, for the assumption on TN ACLR/ACS for co-existence simulation, the values for 17 GHz in below table are considered as starting point for co-existence simulation purpose yet other options not precluded.

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Agreement 2: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands with the following figures:

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

The following WF documents shall be also taken into account:

- R4-2217468, WF on [313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #104-bis-e, Electronic Meeting, 10 - 19 October, 2022;

- R4-2220241, Way forward on [105][313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, November 14 – November 18, 2022;

- R4-2302878, WF for above 10GHz NTN-TN co-existence study (Samsung), 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023.

In previous contribution R4-2302535 (“NTN Simulation Parameters for above 10 GHz Coexistence Studies”, THALES), several NTN simulation parameters were proposed for above 10 GHz coexistence studies. The contribution R4-2305847 (THALES, Magister Solutions Ltd) provided material for discussion with respect to Ka-Band NTN-TN coexistence simulations and NTN UE terminal and Satellite antenna parameters. Parameters from R4-2302535 then were updated in R4-2305847, together with initial simulation results.

For other information, please also check (RAN4#106-bis-e):

- R4-2306002 (“WF on [311] NR_NTN_enh_Part3”, Samsung);

- R4-2306003 (“Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung).

And also (at RAN4#107):

- R4-2309700 (“Initial NTN calibration results for above 10 GHz Coexistence Studies”, THALES, Magister Solutions Ltd);

- R4-2309768 (“Collection table for NTN co-existence in above 10GHz calibration results”, Samsung);

- R4-2310449 (“Summary for [107][311] NR_NTN_enh_Part3”, Samsung);

- RAN4_107_BSRF_Demod_Test_Session Report_11_EOM.docx;

- R4-2309771 (“Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung);

- R4-2309767 (“WF for co-existence study of Above 10GHz NTN band”, Samsung);

The following agreements can be considered for reference only:

- R4-2309770 (“WF on above 10GHz SAN RF requirements“, CATT);

- R4-2309766 (“WF for system parameters on Ka band”, THALES).

With respect to RAN4#108 contributions and WFs, please find the following relevant contributions:

- R4-2312443 (“Updates on NTN calibration and coexistence simulation results for above 10 GHz”), THALES, Magister Solutions Ltd, RAN4#108, Toulouse, France, with a very complete set of results.

- R4-2313865 (“WF for NTN co-existence study”), Samsung, Cybercore, RAN4#108, Toulouse, France;

- R4-2313890 (“Simulation assumption for NTN co-existence study”), Samsung, RAN4#108, Toulouse, France.

The scope of this document is to provide calibration results and show latest divergences between different companies results for NTN in above 10 GHz WI (for information purpose only).

# 2 	Parameters for calibration phase

The following parameters have been used/updated for the calibration phase:

NTN Common Parameters

| Bandwidth | 200MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UEs per cell | 1 in DL, 10 in UL |
| UE height | 22.5 m |
| UE Tx power | 33 dBm |
| UE antenna gain | Tx = 43.2 dBi, Rx = 39.7 dBi |
| UE antenna diameter | 0.60 m |
| UE noise figure | 4 dB |
| Downlink allocation size | 132 PRBs |
| Uplink allocation size | 13 PRBs |
| NTN scenario | Urban (was Dense Urban) |
| Satellite antenna type | Parabolic (TR 38.811 Bessel) |
| UE antenna type | Parabolic (TR 38.811 Bessel) |
| Shadowing | Enabled |
| Atmospheric attenuation | Enabled |
| Scintillation | Disabled |
| Uplink Tx power control | Using R4-2313865, –SNR_target + Pmax – ThermalNoise – SANNoiseFigure - 10*log10(BW) = 113.38 dB with 5.9dB NF, 13 PRB allocation Computed as follows: -15 + 33 + 174 – 5.9 – 10*log10(18.72x10^6) |
| Cell selection | RSRP |
| Frequency re-use | 1 |

Observation 1: Sharing the exact UL Tx power control value between companies may be useful.

Proposal 1: Companies to share exact uplink Tx power control value used for calibration.

NTN Scenario Specific Parameters

|  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite antenna diameter | 5.9/3.66 m DL/UL | 0.6/0.36 m | 0.6/0.36 m |
| Satellite Tx power | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite antenna gain dBi | 58.5 | 38.5 | 38.5 |
| Noise figure | 4 dB | 4 dB | 4 dB |
| G/T dB/K | 27.976 (=5.9 dB NF) | 7.976 (=5.9 dB NF) | 7.976 (=5.9 dB NF) |
| Beamwidth deg | 0. 1763 | 1.7337 | 1.7337 |

TN Common Parameters

| UE height | 1.5 m |
| --- | --- |
| UE – gNB min distance (2D) | 35 m |
| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| gNB antenna element gain | 5.5 dBi |
| gNB antenna elements | 16x8 |
| gNB total TXP | 41.07 dBm (22 dBm per element /w 2dB ohmic loss) |
| gNB antenna beamwidth | Horizontal = 90.0°, Vertical = 90.0° |
| Shadowing | Enabled |
| UE Tx power | 23dBm (not 22.4 dBm) |
| BS & UE Noise Figure | 10 dB* *Excel File has 11/9 dB highlighted! R4-2313890 says 10/10 dB |
| Tx power control | Using R4-2313865, –SNR_target + Pmax – ThermalNoise – BSNoiseFigure - 10*log10(BW) = 89.21 dB with 10dB NF, 132 PRB allocation Computed as follows: -15 + 23 + 174 – 10 – 10*log10(190.08x10^6) |
| UE antenna type | 3GPP TR38.901, 2x2 antenna array with 5.0 dBi element gain 2 panels, front and back of terminal |
| UE rotation | Fully upright (top towards sky), random fixed 360° rotation on y-axis. |
| Cell selection | Best cell, both panels measured |
| gNB antenna type | 3GPP TR 38.901, 16x8 antenna array with RAN4 agreements |
| Downlink allocation size (PRB) | 132 PRB = Full bandwidth |
| Uplink allocation size (PRB) | 132 PRBs = Full bandwidth |
| UE height | 1.5 m |

1/ Calibration results NTN GEO DL VSAT 17 GHz:

2/ Calibration results NTN GEO UL VSAT 27 GHz:

3/ Calibration results NTN LEO@1200 DL VSAT 17 GHz:

4/ Calibration results NTN LEO@1200 UL VSAT 27 GHz:

5/ Calibration results NTN LEO@600 DL VSAT 17 GHz:

6/ Calibration results NTN LEO@600 UL VSAT 27 GHz:

Observation 2: All THALES, Huawei, Samsung and ZTE DL&UL coupling loss values similar to each other and 38.821 values.

Observation 3: Samsung GEO and LEO@600km DL SINR very different from others.

Observation 4: Qualcomm UL SINR quite different from others. Maybe antenna gain or power control?

Observation 5: Ericsson CL values are slightly different from other companies in almost all cases.

Observation 6: All THALES, Huawei and ZTE DL SINR values similar to each other and 38.821 values.

Observation 7: UL SINR values still differ a lot.

Observation 8: Another calibration may be required.

Observation 9: Two DL SINR groups. Wider SINR distribution could be explained by different shadowing between beams, i.e., shadowing correlation between beams is not 1. TR 38.821 calibration results align with the narrower DL SINR distribution from THALES, Huawei and ZTE.

Proposal 2: Assume shadowing correlation between beams of the same satellite as 1.

Moreover, Figure 3 below shows the NTN UE total transmission power distribution for LEO@600km, LEO@1200km and GEO cases.

Figure 3. NTN UE total UL transmission power distribution

Proposal 3: It makes sense to share UL TxP statistics and UL CLx-tile actual values between companies to align UL SINR

- Single beam UL SINR (no interference) should be 15 dB for all users in LEO! Moreover, some GEO users may be power limited.

# Conclusions

Observation 1: Sharing the exact UL Tx power control value between companies may be useful.

Proposal 1: Companies to share exact uplink Tx power control value used for calibration.

Observation 2: All THALES, Huawei, Samsung and ZTE DL&UL coupling loss values similar to each other and 38.821 values.

Observation 3: Samsung GEO and LEO@600km DL SINR very different from others.

Observation 4: Qualcomm UL SINR quite different from others. Maybe antenna gain or power control?

Observation 5: Ericsson CL values are slightly different from other companies in almost all cases.

Observation 6: All THALES, Huawei and ZTE DL SINR values similar to each other and 38.821 values.

Observation 7: UL SINR values still differ a lot.

Observation 8: Another calibration may be required.

Observation 9: Two DL SINR groups. Wider SINR distribution could be explained by different shadowing between beams, i.e., shadowing correlation between beams is not 1. TR 38.821 calibration results align with the narrower DL SINR distribution from THALES, Huawei and ZTE.

Proposal 2: Assume shadowing correlation between beams of the same satellite as 1.

Proposal 3: It makes sense to share UL TxP statistics and UL CLx-tile actual values between companies to align UL SINR

- Single beam UL SINR (no interference) should be 15 dB for all users in LEO! Moreover, some GEO users may be power limited.

# References

- R4-2220239, “WF for above 10GHz band definition and system parameters”, THALES.

- R4-2220241, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”.

- TR 38.811.

- TR 38.821.

- Ka-Band Low Noise Amplifier Sub-System Module for Communication Satellite payload”, S. K. Jain and all.

- ITU-R Rec. P.618-12.

- ITU-R Rec. S.465-6.

- R.C. Hansen “Phased Arrays” in “Antenna Engineering Handbook” by John L. Volakis, editor. McGraw Hill. 4th edition. 2007.

- R4-2305847, “Updated NTN Simulation Parameters for above 10 GHz Coexistence Studies and Initial Simulation Results”, THALES, Magister Solutions Ltd.

- R4-2306002, “WF on [311] NR_NTN_enh_Part3”, Samsung.

- R4-2306003, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

-END-