3GPP TSG-RAN WG4 Meeting #108bis	R4-2316903

Xiamen, China, October 09 – October 13, 2023

Title: 	Way Forward 256 QAM BS Demod

Agenda Item:	5.6.4.1

Source: 	Moderator (Nokia, Nokia Shanghai Bell)

Document for:	Agreement

# 1 PUSCH requirements

### Sub-topic 1-1: Tx EVM

Issue 1-1: Tx EVM

Agreement: Impact for TxEVM shall be included in impairment result.

### Sub-topic 1-2: SNR Limit

Issue 1-2: SNR Limit

Agreement: If impairment result for MCS 20 is over 20dB then check with TE vendors if this is testable. Request companies to include results with impairment for next meeting.

### Sub-topic 1-3: Phase Noise Model

Issue 1-3a: Tx Phase Noise Model

Agreement: Do not consider Tx phase noise model or impact for requirement derivation.

Issue 1-3b: Rx Phase Noise Model

Agreement: Do not specify Rx phase noise model but consider Rx phase noise impact for impaired results.

### Sub-topic 1-4: Channel

Issue 1-4: Channel

Agreement: TDLD 30-35

Note: Review channel if impaired results create testability issues.

### Sub-topic 1-5: Antenna Configuration

Issue 1-5: Antenna Configuration

Agreement: 1T2R

### Sub-topic 1-6: Carrier BW and SCS

Issue 1-6: 60 kHz SCS and corresponding carrier BW

Agreement: Companies to use 50 MHz for 60 kHz, however decision whether to include requirements for SCS of 60 kHz to be deferred to RAN4#109.

Issue 1-7: 120 kHz SCS and corresponding carrier BW

Agreement:

Companies to use 50 MHz initially for requirements derivation, with FFS on 100 MHz and 200 MHz.

Companies to provide simulations for 50, 100, 200 with a priority in order of 50, 200, 100, with a view on testable SNR.

Note: Could use existing applicability rule to extend 50 MHz to other CBW.

### Sub-topic 1-7: DMRS

Issue 1-8: DMRS

Agreement: Both 1+1 and 1+0, cases selected for tests to be based on BS Manufacture Declaration and applicability rule

### Sub-topic 1-8: PTRS

Issue 1-9: PTRS

Agreement: Define with both PTRS enabled and disabled, cases selected for tests to be based on BS Manufacturer Declaration and applicability rule to test requirements where a testable case is available.

For information below is a table of expected testable cases (with impairment, and testable being less than 20 dB SNR).

Table 1 : Expected Testable Cases with PTRS and DMRS for UL 256 BS Demod,

based on simulations for RAN4#108bis

|  |  | PTRS |  |
| --- | --- | --- | --- |
|  |  | Disabled | Enabled |
| DMRS | 1+0 | Likely not testable (based on simulations) | Likely testable (based on simulations) |
|  | 1+1 | Likely testable (based on simulations) | Likely testable (based on simulations) |

### Sub-topic 1-9: MCS

Issue 1-10: MCS

Agreement: MCS 20

# Topic #2: Work Split

## Open issues summary

### Sub-topic 2-1: Work Split

Issue 2-1: Work Plan

- Agreement:

| Task ID |  | Responsible Company |
| --- | --- | --- |
| Simulation result summary for 256 QAM BS Demodulation |  | Huawei |
| CR split |  |  |
| Specification | Content part | Responsible Company |
| 38.104 | PUSCH requirement | Nokia |
| 38.141-2 | PUSCH requirement | Ericsson |
| 38.104, 38.141-2, 38.141-1 (for alignment) | FRC tables | ZTE |
| 38.104 | Applicability rule, manufactory declaration, measurement uncertainty and test tolerance, delay profiles for FR2 | Huawei |
| 38.141-2 | Applicability rule, manufactory declaration, measurement uncertainty and test tolerance, delay profiles for FR2 | Samsung |

## A.4 Reference

[1] R4-2317946, Topic summary for [108][321] NR_RF_FR2_req_Ph3_Demod, Nokia, Nokia Shanghai Bell, RAN WG4 #108bis