3GPP TSG-RAN WG4 Meeting #108bis	R4-2316914

Xiamen, China, 9 – 13 Oct. 2023

Title: 	WF for 8Rx and 4Tx performance requirements

Agenda Item:	5.3.4

Source: 	Huawei, HiSilicon

Document for:	Approval

1 General

Issue 1-1-1: Test applicability rules for 8Rx CA demodulation requirements

- Proposals

  - Option 1

    - Extend Table 5.1.1.7.2-1 to include Tests cases in new Clause 5.2A.4.1.

  - Option 2

    - Make decision on the 8Rx CA test applicability rules together with or after the discussion on the test parameters, such as channel bandwidth aggregation, rank, UE capability and declaration, etc.,

Issue 1-1-2: Applicability rules for different number of RX antenna ports for CA demodulation requirements

- Proposals

  - Option 1

    - Within the CA configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, 4 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.3.1 are applied.

    - Within the CA configuration if any of the PCell and/or the SCells is a 2Rx supported RF band, 2 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.2.1 are applied.

    - Within the CA configuration if any of the PCell and/or the SCells is a 8Rx supported RF band, all 8Rx should be connected with data source from system simulator. Requirements from Clause5.2A.4.1 are applied.

    - For 8Rx capable UEs, the 2Rx supported RF bands, 4Rx supported RF bands and 8Rx supported RF bands are up to UE’s declaration.

2 CA requirements

Issue 2-2-1: CA capability

- Agreement

  - Intra band contiguous CA, Intra band non-contiguous CA, Inter band CA

Issue 2-2-2: CBW for PDSCH CA requirements

- Agreement

  - Define the performance requirements for the following channel bandwidths for each SCS:

    - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

    - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Issue 2-2-3: CA duplex model and SCS combination

- Agreement

  - Consider the following duplex model and SCS combination for 8Rx CA

    - FDD 15kHz + TDD 30kHz

    - FDD 15kHz + FDD 15kHz

    - TDD 30kHz + TDD 30kHz

Issue 1-1-2: Number of RX antenna ports for CA demodulation requirements

- Agreement

  - Consider Hybrid Rx for 8Rx CA perf requirements, such as 2Rx+8Rx, 4Rx+8Rx

Issue 2-2-4: Rank

- Agreement

  - Rank 8 for 8Rx+8Rx, Rank 2 for 2Rx+8Rx and 4Rx+8Rx

    - Rank 8 + Rank 8 for 8Rx+8Rx for UE supporting Rank 8 for 8Rx

    - Rank 2 + Rank 2 for 4Rx+8Rx

    - Rank 2 + Rank 2 for 2Rx+8Rx

Issue 2-2-5: HARQ-ACK configuration per PUCCH group

- Agreement

  - Spatial bundling of PUCCH HARQ ACKs is disabled

  - Companies are encouraged to double check this simulation assumptions in the simulation configuration, if needed, updated simulation results are welcome to be submitted for next meeting.

3 SDR CA requirements

SDR CA requirements

- Agreement

  - Reuse the existing methodology of Re-15 SDR CA test

4 CQI CA requirements

Issue 4-1-1: CA test for CQI reporting

- Agreement

  - Not consider CQI reporting for CA.

5 CRs

Draft CRs will be endorsed for this meeting if agreeable

No resubmission is needed for the next meeting if no updates to the draft CRs endorsed during this meeting

One draft big CR will capture all endorsed draft CRs for this meeting.

Annex A: Simulation assumptions (for information)

A.1 PDSCH

Other common parameters, refer to TS 38.101-4: Table 5.2-1: Common test parameters for PDSCH

Table 1: Simulation assumptions for PDSCH performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD, FDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30/40 FDD: 15/10 |
| MIMO layer and Antenna configuration |  |  | Rank 2: 2x8 Rank 4: 4x8 Rank 8: 8x8 |
| Propagation condition and antenna correlation |  |  | Rank 2: TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) Rank 4: TDLA30-10 Low Rank 8: TDLA30-10 Low |
| MCS |  |  | Rank2: MCS19 Rank 4: MCS17 Rank 8: MCS17 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| NZP-CSI-RS configuration |  |  | Rank 8: row 6 |
| Overhead |  |  | 0 |
| N1 and N2 configurations for 8Tx cases |  |  | Use (N1,N2) = (4,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 8 layers cases |
| Number of HARQ Processes |  |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |
| Test metric |  |  | SNR@70% max TP |

A.2 SDR

Other common parameters, refer to TS 38.101-4:

- Table 5.5A-1: Common test parameters for FDD and TDD component carriers

- Table 5.5A-2: Additional test parameters for FDD CC

- Table 5.5A-3: Additional test parameters for TDD CC

Table 2: Simulation assumptions for SDR performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

A.3 CQI test

Other common parameters, refer to TS 38.101-4:

- Table 6.2.2.1.1.1-1: CQI reporting definition test for FDD

- Table 6.2.2.2.1.1-1: CQI reporting definition test for TDD

Table 3: Simulation assumptions for CQI test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex Mode |  |  |  | TDD, FDD |  |  |  |
| Bandwidth/SCS |  |  | MHz | TDD: 40MHz/30kHz FDD: 10MHz/15kHz |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | 4 | 5 | 10 | 11 |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | TDD: 16 FDD: 8 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | TDD: 10/9 FDD: 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | 00000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | TDD: 9.5 FDD: 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Number of HARQ Processes |  |  |  | FDD: 4 TDD: 8 |  |  |  |
| Measurement channel |  |  |  | TDD: As specified in Table A.4-3, TBS.3-4 FDD: As specified in Table A.4-1, TBS.1-3 |  |  |  |

Annex B: CR split for TS 38.101-4

| Section | Test cases | Companies |
| --- | --- | --- |
| 5.1.1.2 Applicability of requirements for different number of RX antenna ports | Table PDSCH/PDCCH/PBCH test applicability rules for 8Rx | Nokia |
| 5.2.4 8RX requirements (New) | 5.2.4.1 FDD PDSCH performance requirements | Samsung |
|  | 5.2.4.2 TDD PDSCH performance requirements |  |
| 5.5A Sustained downlink data rate provided by lower layers | SDR tests | Ericsson |
| 6.1.1.2 Applicability of requirements for different number of RX antenna ports | Table 6.1.1.2-1: Requirements applicability for CSI applicabaility rules | Huawei |
| 6.2.4 8RX requirements (New) | 6.2.4.1 FDD CQI requirements | CTC |
|  | 6.2.4.2 TDD CQI requirements |  |
| A.3.2 Reference measurement channels for PDSCH performance requirements | A.3.2.1.1 Reference measurement channels for SCS 15 kHz FR1 | MediaTek |
|  | A.3.2.2.2 Reference measurement channels for SCS 30 kHz FR1 |  |
| B.1 Static propagation condition | B.1.2 UE Receiver with 8Rx (New) | ZTE |
| B.2.3 MIMO Channel Correlation Matrices | MIMO Correlation Matrices using Uniform Linear Array (ULA) | Apple |
| 5.2A.4 8Rx requirements | Demodulation requirements for 8Rx PDSCH CA | Huawei |

Annex C: Reference

[1] R4-2220613, WF on 8RX UE demodulation and CSI requirements, RAN4#105, Huawei, HiSilicon

[2] R4-2302942, WF for 8Rx UE performance requirements, RAN4#106, Huawei, HiSilicon

[3] R4-2305888, WF for 8Rx UE performance requirements, RAN4#106bis-e, Huawei, HiSilicon

[4] R4-2309806, WF for 8Rx UE performance requirements final, RAN4#107, Huawei, HiSilicon

[5] R4-2313877, WF for 8Rx UE performance requirements final, RAN4#108, Huawei, HiSilicon