3GPP TSG-RAN WG4 Meeting #108bis	R4-2316917

Xiamen, China, October 9 ‒ October 13, 2023

Title: 	WF on NonCol_intraB_ENDC_NR_CA_Demod

Agenda Item:	5.11.5

Source: 	Ericsson

Document for:	Approval

# Type 2 UE demodulation requirements

## Sub-topic 1-1	Test setup for Type 2 UE NR-CA PDSCH demodulation requirements

Way forward: Rank/MCS selection

- Interested companies are encouraged to bring the PDSCH simulation results with impairments in RAN4#109 to decide the MCS index pair and corresponding SNR test point pair to define PDSCH NR-CA demodulation requirements.

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | AWGN (TS38.101-4 B.1.1) |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Antenna configuration | 2x2 for both carriers |
| MCS table and MCS index | FFS |
| Test metric | 70% of the maximum throughput |

|  | Rank | MCS table | MCS index |
| --- | --- | --- | --- |
| Carrier with lower power | Rank 1 | MCS table 1 | Option 1: MCS4 (QPSK, 0.30) Option 2: MCS5 (QPSK, 0.37) |
| Carrier with higher power | Rank 2 | MCS table 2 | Option 1: MCS22 (256QAM, 0.74) Option 2: MCS23 (256QAM, 0.78) Option 3: MCS24 (256QAM, 0.82) Option 4: MCS25 (256QAM, 0.86) |

Note the required SNR difference between the carriers with lower power and higher power should not exceed 25dB.

Agreement: AWGN channel model

- Confirm that the AWGN channel model in the WF last meeting means the static channel model defined in B.1.1 in TS 38.101-4.

Agreement: CA combination

- Select single bandwidth of 40 MHz for the requirement.

Agreement: Test applicability

Send LS to RAN5 to apply the following test condition in the conformance test of PDSCH demodulation requirements as well as RF conclusion.

- Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC in R18.

Agreement: CR work split

Volunteer companies will submit draft CRs in RAN4#109.

| CR | Volunteer companies |
| --- | --- |
| Draft CR for Applicability rule | Huawei |
| Draft CR for PDSCH demodulation requirements | ZTE |
| Draft CR for FRC | Qualcomm |
| Big CR for Type 2 UE NR-CA PDSCH demodulation requirements | Nokia |