3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316921

# Xiamen, 9th - 13th Oct, 2023

Agenda item:			5.26.9

Source:	Moderator (Huawei, HiSilicon)

Title:	WF on NR_NTN_enh_SAN_UE_demod

Document for:	Approval

Introduction

This contribution is the way forward for NR NTN enhancement demod for both SAN and UE. Four topics are captured:

- Topic #1: Workplan

- Topic #2: General

- Topic #3: UE demodulation performance requirements

- Topic #4: SAN demodulation performance requirements

Topic #1: Workplan

Sub-topic 1-1: Workplan

Issue 1-1: Workplan

- Agreement

  - Workplan is agreed at RAN4#108-bis meeting.

| RAN4#108-bis Oct 2023 |  |
| --- | --- |
|  | Discussion and agreement on work plan. Initial round of discussion on performance requirements scope. Initial round of discussion on simulation assumptions. |
| RAN4#109 Nov 2023 |  |
|  | Second round of discussion on performance requirements scope. Second round of discussions on simulation assumptions. Initial round of simulation results collection and alignment. |
| RAN4#110 Feb 2024 |  |
|  | Finalize discussions on performance requirements scope. Discussions on CR split. Second round of simulation results collection and alignment. |
| RAN4#110-bis Apr 2024 |  |
|  | Finalize discussions on simulation assumptions. Third round of simulation results collection and alignment. Endorse draft CRs. |
| RAN4#111 May 2024 |  |
|  | Final round of simulation results collection and alignment. Agree formal CRs. |

Topic #2 General

Sub-topic 2-1: Scenario and channel model for above 10 GHz bands

Issue 2-1-1: Scenario

- Agreement

  - At least NGSO scenario to be considered for requirement definition, companies can check whether GSO can also be considered for NTN demod.

  - Focus on the mobility scenario assumed by RRM. (Mobility VSAT with LEO is not considered)

Issue 2-1-2: Channel model

- Agreement

  - For FR2-NTN, consider NTN-TDLA and/or NTN-TDL-C with down selection if necessary.

  - FFS delay and Doppler

  - For delay selection, consider the worst case based on typical angle selection, e.g. [30 degree].

  - How to derive the Doppler:

    - Option 1: Based on residual frequency error.

    - Option 2: Based on UE speed

      - FFS UE speed: [120km/h, 1000km/h], other Options are not precluded.

    - Interested companies are encourage to propose values for doppler and delay spread.

  - Other options are not precluded

Topic #3: UE demodulation performance requirements

Sub-topic 3-1: Test scope

Issue 3-1-1: Whether to define UE PDSCH demodulation performance requirements for above 10 GHz bands

- Agreement

  - Agree to define UE PDSCH demodulation performance requirements for above 10 GHz bands.

Issue 3-1-2: HARQ processes for above 10 GHz bands

- Agreement

  - FFS 16 HARQ process, 32 HARQ process, HARQ process disabled.

  - Interested Companies can provide further analysis for 16 HARQ process, 32 HARQ process, HARQ process disabled.

Issue 3-1-3: Whether to define UE PDCCH demodulation performance requirements for above 10 GHz bands

- Way forward

  - FFS to define new PDCCH requirements for NTN

  - FFS to reuse TN PDCCH requirements

Issue 3-1-4: Whether to define UE PBCH demodulation performance requirements for above 10 GHz bands?

- Way forward

  - FFS to define new PBCH requirements for NTN

  - FFS to reuse TN PBCH requirements

Issue 3-1-5: Whether to define UE CSI reporting requirements for above 10 GHz bands?

- Agreement

  - Do not consider PMI reporting and RI reporting.

  - Companies can provide feasibility analysis on CQI reporting under AWGN or fading channels.

Sub-topic 3-2: General issues for above 10 GHz bands

Issue 3-2-1: Frequency/timing drift

- Agreement

  - Assumption: UE is assumed to do compensation for frequency and timing drift , and is not part of baseband processing for demod.

  - No frequency and timing drift are modelled in Rel-18 NTN UE requirements.

Issue 3-2-2: SCS (except PBCH testing)

- Proposals

  - Option 1: 120kHz

Issue 3-2-3: Channel bandwidth

- Proposals

  - Option 1: 100MHz

  - Option 2: 200MHz

Issue 3-2-4: Antenna configuration

- Proposals

  - Option 1

    - Take 1Tx1Rx for parabolic VSAT antenna configuration for initial demodulation discussion and input from satellite companies is needed.

Issue 3-2-5: Receiver assumption

- Agreement

  - MMSE-IRC

Issue 3-2-6: Duplex

- Agreement

  - FDD

Issue 3-2-7: Payload

- Agreement

  - Transparent payload for both GSO and NGSO scenarios.

Issue 3-2-8: Beamforming and beam steering

- Proposals

  - Option 1: Discuss potential beamforming and beam steering mechanism for VSAT devices for NR NTN enhancements.

Issue 3-2-9: Rx phase noise

- Proposals

  - Option 1: Take Rx phase noise impact into impairment results and companies could give proper values based on preferred PN model.

  - Option 2: Do not consider PN impact.

Issue 3-2-10: Applicability rule

- Proposals

  - Option 1: Adding similar applicability rule for FR2 NTN UE optional capabilities as in Rel-17 FR1 NTN UE.

Sub-topic 3-3: Test setup for above 10 GHz bands

Issue 3-3-1: MCS for PDSCH

- Proposals

  - Option 1: 16QAM as baseline, FFS 64QAM based on link budget analysis.

  - Option 2: QPSK, 16QAM

  - Option 3: MCS4 (QPSK, 0.30) and MCS13 (16QAM, 0.48)

Issue 3-3-2: Rank for PDSCH

- Proposals

  - Option 1: Rank 1

Issue 3-3-3: PDSCH mapping type

- Proposals

  - Option 1: PDSCH mapping type A

Issue 3-3-4: PDCCH aggregation level (If agreed to be introduced)

- Proposals

  - Option 1:

    - Aggregation level: 4

Issue 3-3-5: Configuration for PBCH test (If agreed to be introduced)

- Proposals

  - Option 1:

    - SSB index is known and unknown.

    - SCS: 120kHz and 240kHz SCS

- Topic #4: SAN demodulaton requirements

Sub-topic 4-1: Test scope

Issue 4-1-1: Which feature can be considered for SAN PUSCH demodulation performance requirements for above 10 GHz bands?

- Agreement

  - Define PUSCH with DFT-S-OFDM and CP-OFDM demodulation performance requirements for above 10 GHz bands.

  - FFS PUSCH repetition Type A, companies can do some analysis on the link budget.

  - Do not consider UL timing adjustment

Issue 4-1-2: Which format can be considered for SAN PUCCH demodulation performance requirements for above 10 GHz bands?

- Agreement

  - Define PUCCH format 0/1/2/3/4 demodulation performance requirements for above 10 GHz bands.

Issue 4-1-3: Which format can be considered for SAN PRACH demodulation performance requirements for above 10 GHz bands?

- Agreement

  - Define PRACH burst format B4/C2 demodulation performance requirements for above 10 GHz bands.

  - Do not consider PRACH burst format A2.

Issue 4-1-4: Whether to define SAN PUSCH demodulation performance requirements for DMRS bundling?

- Agreement

  - Define SAN PUSCH demodulation performance requirements for DMRS bundling for FR1 only if RF confirm that phase continuous is feasible.

Issue 4-1-5: Whether to define SAN PUCCH demodulation performance requirements for Msg4 HARQ-ACK?

- Agreement

  - Do not define PUCCH requirements Msg4 HARQ-ACK for NLOS channel.

- Way forward

  - Option 1: Add new requirements under LOS channel for multi-slot PUCCH format 1.

  - Option 2: Do not define PUCCH requirements Msg4 HARQ-ACK for LOS channel.

Issue 4-1-6: Whether to define SAN demodulation performance requirements for network verified UE location?

- Agreement

  - Do not define SAN demodulation performance requirements for network verified UE location.

Issue 4-1-7: Whether to define SAN demodulation performance requirements for NTN-TN and NTN-NTN mobility and service continuity enhancement?

- Agreement

  - Do not define SAN demodulation performance for NTN-TN and NTN-NTN mobility and service continuity enhancement.

Issue 4-1-8: WID update

- Agreement

  - Update the impacted specification TS 38.108 for the performance part in the WID.

  - Update the objective of performance part WI to include coverage enhancement if necessary.

Sub-topic 4-2: General issues for above 10 GHz bands

Issue 4-2-1: Frequency and timing drift

- Agreement

  - Assumption: UE will do pre-compensation for frequency and timing drift.

  - No frequency and timing drift are modelled in Rel-18 NTN SAN requirements.

Issue 4-2-2: SCS and channel bandwidth

- Agreement

  - Start demod work with 120 kHz SCS for initial alignment.  If RRM concludes that 120 kHz SCS is not feasible, then further discussion is needed at that time on how to proceed

  - FFS channel bandwidth

Issue 4-2-3: Antenna configuration

- Agreement

  - For the SAN Rx, we need both 1Rx and 2Rx.  For UE Tx, more discussion is needed to cover 1 Tx only or also 2Tx.

Issue 4-2-4: MCS & rank

- Agreement

  - Postpone to discuss this issue after channel model is selected.

  - Companies can do some link budget analysis until the next meeting.

Issue 4-2-5: SAN type

- Agreement

  - Prioritize to define SAN demodulation requirement for Type 2-O. FFS to define BS demodulation requirement for Type 2-H, pending on conclusion of RF core requirement.

Issue 4-2-6: Duplex

- Agreement

  - FDD

Sub-topic 4-3: Test setup for above 10 GHz bands

Issue 4-3-1: Test setup for SAN PUSCH demodulation performance requirements

- Agreement

  - Option 1 as the starting point, other options are not precluded.

    - Option 1:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled, Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | [Pos2] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} [, {0, 1}] |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, |
| configuration | Time density (LPT-RS) | 1, |

Issue 4-3-2: Test setup for SAN PUCCH demodulation performance requirements

- Agreement

  - Option 1 as the starting point, other options are not precluded.

    - Option 1:

Table 3:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |
| Channel Model | [NTN-TDLA10-3000] |

Table 4:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 5   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

Issue 4-3-3: Test setup for SAN PRACH demodulation performance requirements

- Agreement

  - Option 1 as the starting point, other options are not precluded.

    - Option 1:

      - PRACH format: C2 and B4

      - Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

      - Frequency offset

        - [AWGN: 0Hz]

        - [Fading: 3000Hz]

      - Time error tolerance

        - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

      - Channel Model: [NTN-TDLA10-3000]

Sub-topic 4-4: Test setup for UL coverage enhancement

- Agreement

  - Postpone this topic discuss to the next meeting.

Issue 4-4-1: Frequency and timing drift

- Proposals

  - Option 1: Further discuss whether assumption of no timing drift modelling can be applied for DMRS bundling requirement pending on RF conclusion.

Issue 4-4-2: Test setup for SAN PUSCH demodulation performance requirements for DMRS bundling

- Proposals

  - Option 1

    - Demodulation performance for DMRS bundling depends on actual window length in which NTN UE can maintain it’s phase and power.

    - To define BS PUSCH demodulation requirements for DMRS bundling, with detailed parameters such as slot number, configured window length, frequency hopping, repetition type, waveform etc., RF discussion should be followed.

  - Option 2

    - The Channel model, Channel bandwidth & SCS and MCS & Rank can be reused from the existing FR1-NTN cases in TS 38.108, at the same time the other parameters can be reused from the existing PUSCH DMRS bundling cases in TS 38.104.

  - Option 3

    - FFS on introduce the timing drift model for specifying the PUSCH requirement with DMRS bundling in NTN scenario.

    - The following value for actual TDW and nominal TDW can be considered for PUSCH requirement with DMRS bundling in NTN scenario

      - Actual TDW and PUSCH-TimeDomainWindowLength = 2 or 4 for FDD

      - Actual TDW and PUSCH-TimeDomainWindowLength 2 for TDD

    - The following PRB allocation can be considered for PUSCH requirement with DMRS bundling in NTN scenario

      - PRB: 6RBs for both 15KHz and 30KHz

    - Introduce PUSCH requirement with mapping type A for NTN-specific UE with DMRS bundling

    - Introduce PUSCH requirement with CP-OFDM waveform for NTN-specific UE with DMRS bundling

    - Introduce PUSCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with DMRS bundling. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

    - Introduce PUSCH requirement for NTN-specific UE with 1+1 DMRS symbols.

    - Apply the existing test parameters for specifying PUSCH requirement with DMRS bundling for NTN-specific UE with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example UL-DL pattern [Note 1] |  | 15 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U 30 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 2] | 0, 3, 0, 3 for FDD 0, 3, 0, 3 for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | [n2] or [n8] for FDD n2 for TDD |
| pusch-TimeDomainWindowLength |  | [2] or [4] slots for FDD 2 slots for TDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Issue 4-4-3: Test setup for SAN PUSCH demodulation performance requirements for Msg4 HARQ-ACK (If agreed to be introduced)

- Proposals

  - Option 1

    - RAN4 shall define requirements for 4 and 8 PUCCH repetitions.

  - Option 2

    - Only PUCCH format 1 requirement should be specified

    - Introduce multi-slot PUCCH requirement with format 1 under 2 repetition for NTN-TDLC5-200 channel

    - Introduce PUCCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with repetition. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

    - Using the following test parameters for specifying PUCCH requirement for Msg4 HARQ-ACK

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |