3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316922

Xiamen, China, October 09 – October 13, 2023

Source:	China Telecom

Title:	WF on NR_cov_enh2_demod

Agenda Item:	5.27.3

Document for:	Approval

Introduction

This document provides the way forward on the performance part of Rel-18 further NR coverage enhancement WI.

<Topic #1: Multiple PRACH transmission requirements>

Issue 1-1-1: Whether to define BS performance requirements for Multiple PRACH transmission

  - RAN4 to define performance requirements for Multiple PRACH transmission with same preamble

Issue 1-1-2: Coverage of frequency range (FR) for Multiple PRACH transmission

  - Prioritize FR2-1.

  - Further discuss whether to cover FR1

Issue 1-1-3: Sequence length for BS performance requirements for Multiple PRACH transmission

  - Option 1: Only define PRACH requirements for normal mode and sequence length 139

  - Other options are not precluded

Issue 1-1-4: PRACH repetition number for BS performance requirements for Multiple PRACH transmission

  - Option 1: Test 8 times for PRACH repetition if the SNR value could be testable

  - Option 2: Define PRACH requirements for 2 PRACH transmissions

  - Other options are not precluded

Issue 1-1-5: Antenna configuration for BS performance requirements for Multiple PRACH transmission

- FR1 (if introduced)

  - Option 1: 1x2

  - Option 2: 1x2, 1x4, 1x8

- FR2-1:

  - Option 1: 1x2

Issue 1-1-6: Channel model for BS performance requirements for Multiple PRACH transmission

- FR1 (if introduced)

  - Option 1: TDLC 300-100 Low and AWGN channels

  - Option 2: Use CDL-A to define requirements for PRACH coverage enhancements to capture spatial gains

- FR2-1

  - Option 1: TDLA30-300 Low and AWGN channels

  - Option 2: Use CDL-A to define requirements for PRACH coverage enhancements to capture spatial gains

Issue 1-1-7: Frequency offset for BS performance requirements for Multiple PRACH transmission

  - For AWGN for both FR1 (if introduced) and FR2-1:

    - Option 1: 0 Hz

    - Other options are not precluded

  - For TDLC 300-100 fading channel for FR1 (if introduced):

    - Option 1: 400 Hz

    - Other options are not precluded

  - For TDLA30-300 fading channel for FR2-1:

    - Option 1: 4000 Hz

    - Other options are not precluded

Issue 1-1-8: Sub Carrier Spacing for BS performance requirements for PRACH repetitions

- FR1 (if introduced)

  - Option 1: 15kHz and 30kHz

  - Other options are not precluded

- FR2-1

  - Option 1: Use 60kHz SCS

  - Option 2: Cover 60kHz SCS and 120kHz SCS

  - Option 3: 120kHz SCS

Issue 1-1-9: PRACH preamble format for BS performance requirements for Multiple PRACH transmission

  - Option 1: Use PRACH format B4

  - Option 2: Cover PRACH preamble format A1, A2, A3, B4, C0 and C2

<Topic #2: Power domain enhancements>

Issue 1-2-1: Whether to define BS performance requirements for increased UE Tx power

  - Do not define BS performance requirements for increased UE Tx power

Issue 1-2-2: Whether to define BS performance requirements with Frequency Domain Spectrum Shaping (FDSS)

  - Option 1: Define BS performance requirements with FDSS

  - Option 2: Not to define BS performance requirements with FDSS

<Topic #3: Dynamic Waveform Switching>

Issue 1-3-1: Whether to define BS performance requirements for dynamic waveform switching

  - Not to define BS performance requirements for dynamic waveform switching