3GPP TSG-RAN WG4 Meeting #108bis	R4-2316923

Xiamen, China, Oct., 9 – 13, 2023

Title: 	WF on NCR-MT demodulation requirements

Agenda Item:	5.28.6

Source: 	ZTE Corporation

Document for:	Approval

Sub-topic 1-1 PDCCH requirements

Issue 1-1-1: Whether define new PDCCH requirements for DCI format 5_0/2_8

- Agreement:

    - No need to define new DCI format 5_0/2_8 requirements. Reuse same DCI type from IAB-MT and legacy UE requirements for NCR-MT PDCCH requirements. Following table used for simulation purpose on the requirements with legacy DCI.

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low |

Sub-topic 1-2 CSI requirements

Issue 1-2-1: Whether to define PMI requirements

- Agreement:

    - No need to define PMI requirements for NCR-MT.

Sub-topic 1-3 Others

Issue 1-3-1: Which FRC can be reused for NCR-MT

- Agreement:

    - FRCs need to be based on IAB-MT style testing.

      - For test cases in which the corresponding FRCs are available in IAB-MT specification, reuse the FRC from IAB-MT specification.

      - For the remaining test cases, companies are encouraged to provide FRCs.

Sub-topic 1-4 CR work split

38.106

| Section | Company |
| --- | --- |
| Section 12: Conducted performance requirements | ZTE |
| Section 13: Radiated performance requirements | Ericsson |
| Annex B: FRC | Huawei |
| Annex C: Propagation conditions | Nokia |

38.115-1

| Section | Company |
| --- | --- |
| Section 7: Conducted performance requirements(PDSCH,PDCCH) | Ericsson |
| Section 7: Conducted performance requirements(CSI) | Huawei |
| Annex F: FRC | ZTE |
| Annex G : Propagation conditions | Nokia |

38.115-2

| Section | Company |
| --- | --- |
| Section 7: Radiated performance requirements(PDSCH,PDCCH) | ZTE |
| Section 7: Radiated performance requirements(CSI) | Ericsson |
| Annex F: FRC | Nokia |
| Annex G: Propagation conditions | Huawei |

- Note: Those CR will be approved in RAN4 #110 meeting in Feb. 2024.