3GPP TSG-RAN WG4 Meeting #108bis	R4-2316941

Xiamen, China, 9th ‒ 13th October, 2023

Title: 	WF on SBFD BS requirements

Agenda Item:	5.19.4

Source: 	Ericsson

Document for:	Approval

# Testing in SBFD and non-SBFD slots

Some requirements might need to be tested in both SBFD and non-SBFD slots even when the requirement is the same. The reason would be if it could be expected that the BS operating condition may differ between SBFD and non-SBFD slots. Whether to apply a test in both SBFD and non-SBFD slots should be discussed on a requirement by requirement phase in the conformance part of a WI.

# RF requirement considering semi-static and dynamic condition

Agreement (for SI phase):

For SBFD-capable BS, RF requirement shall only be studied based on the semi-static configuration of subband time and frequency location, which is supported by SBFD-capable BS.

# Transition times between SBFD and non-SBFD slots

Agreement:  Between the non-SBFD slot and SBFD slot and vice versa, a transition period is needed.  If the SBFD configuration between adjacent SBFD slots is the same, then no transition period is needed.

# Total power dynamic range

The requirement limit for the total power dynamic range for SBFD slots is not as yet agreed. Contributions proposing a requirement limit are encouraged.

# Co-location and co-existence requirements

For co-location and coexistence requirement, further contributions are encouraged to decide on one of the following options:

    - Option 1: Co-location requirement can’t use 30 dB coupling loss as the coupling loss assumption for SBFD capable gNB co-location related requirement.

    - Option 2: No update on existing requirements, it’s declaration basis whether BS need to follow the requirements.

# Transmitter intermodulation

For transmitter intermodulation in SBFD slots, further contributions are encouraged to decide on one of the following options:

- TX IM is not applied in SBFD slots

- TX IM is applied in SBFD slots, but with a different interferer offset than 30dB

- TX IM is applied in SBFD slots with 30dB interferer offset. SBFD RX requirements are not applicable when the TX IM interferer is applied.

# In channel adjacent sub-band leakage ratio

Further contributions are welcomed taking into account the following options:

- Option 1: Do not create a new requirement for in-channel adjacent sub-band leakage ratio

- Option 2: Create a new requirement on in-channel adjacent sub-band leakage ratio, similar to ACLR

  - Requirement limit should also be proposed

# In-channel adjacent sub-band selectivity and blocking

Further contributions are welcomed taking into account the following options:

- Option 1: Do not create a new requirement for in-channel adjacent sub-band selectivity or blocking

- Option 2: Create a new requirement on in-channel adjacent sub-band selectivity (similar to ACS), but no blocking requirement

  - Requirement limit should also be proposed

- Option 3: Create a new requirement on in-channel adjacent sub-band blocking, but no selectivity requirement

  - Requirement limit should also be proposed

- Option 4: Create new requirements on in-channel adjacent sub-band selectivity (similar to ACS) and blocking

  - Requirement limits should also be proposed