3GPP TSG-RAN WG4 Meeting # 108bis   	R4-2316955

Xiamen, China, October 09 - October 13, 2023

Title: 	WF on NR_ATG_Demod

Agenda Item:	5.13.8

Source: 	CMCC

Document for:	Approval

- General

Issue 1-1: TDD pattern

Agreement:

- For new incremental ATG PDSCH, introduce test cases of new TDD pattern configuration 30D4S6U.

  - PDSCH requirements are optional based on UE capability.

- FFS on how to introduce new TDD pattern configuration 30D4S6U in ATG PUSCH requirements.

Issue 1-2: Applicability rule for TDD pattern

Agreement:

- For PDSCH

  - Legacy TDD pattern can be skipped if the test of the case with the new TDD pattern is passed.

- For PUSCH

  - Depending on outcome of issue 1-1.

Issue 1-3: Specification impact

Agreement:

- Consider the following as baseline for specification drafting:

  - Add applicability requirements for ATG UE in section 5.1.1

  - For PDSCH, introduce a new section for ATG demodulation requirements.

  - For PDCCH, PBCH and CQI reporting, do not introduce new section for ATG demodulation requirements.

Agreement:

- New section for ATG new incremental PUSCH requirements should be introduced in 38.104

- New section for ATG applicability of PUSCH requirements and [Applicability of requirements for TDD with different UL-DL patterns] should be introduced in 38.141-1 and 38.141-2

- New section for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

- New section for ATG applicability of PRACH requirements should be introduced in 38.141-1 and 38.141-2

Issue 1-4: Channel model

Agreement:

- Option 1 The values of the channel matrix used for demodulation requirements is the static channel defined in B.1 in TS 38.101-4

- UE demodulation

Issue 2-1-1: Test scope for PDSCH:

Agreement:

- For new incremental requirements:

  - introduces following test cases for new PDSCH requirements

| FDD 10 MHz 15kHz SCS | 2T2R, 2T4R | 16QAM (MCS 13 in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS 22 in table 1) |
|  |  | 256QAM (MCS 24 in table 2) |

| TDD 40MHz 30kHz SCS | 7D1S2U & New TDD pattern 30D4S6U | 2T2R, 2T4R | 16QAM (MCS 13 in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS 22 in table 1) |
|  |  |  | 256QAM (MCS 24 in table 2) |

Issue 2-1-2: Test scope for PDCCH

Agreement:

- To consider legacy PDCCH requirements for ATG PDCCH requirements to cover all AL.

- The following test cases are for down-selection: (if necessary)

  - 1T2R FDD: Test number 1, 3 and 5 in 5.3.2.1.1

  - 2T2R FDD: Test number 3 in 5.3.2.1.2

  - 1T2R TDD: All test cases in 5.3.2.2.1

  - 2T2R TDD: All test cases in 5.3.2.2.2

  - 1T4R FDD: Test number 1, 3 and 5 in 5.3.3.1.1

  - 2T4R FDD: Test number 3 in 5.3.3.1.2

  - 1T4R TDD: All test cases in 5.3.3.2.1

  - 2T4R TDD: All test cases in 5.3.3.2.2

Issue 2-1-3: Test scope for PBCH

Agreement:

- Legacy PBCH requirements are applicable for ATG UE.

- No specification impact of PBCH requirements other than applicability for ATG UE

Issue 2-1-4: SDR test cases

Agreement:

- Legacy SNR requirements of 1 MIMO layer are applicable for ATG UE with antenna configuration 1x2 and 1x4.

Issue 2-2-1: Test scope for CSI reporting

Agreement:

- Do not introduce PMI requirements for ATG UE

Issue 2-3-1: Applicability rule for UE demodulation requirements

Agreement:

- Directly check the CR in next meeting

- BS demodulation

Issue 3-1-1: Test Scope

Agreement:

- Introduce new dedicated test cases for 64QAM MCS 28 and 256QAM MCS 22

- If ATG BS declares support of 256QAM, new ATG test case for 256QAM is tested, new ATG test case for 64QAM is skipped, legacy 64QAM is tested.

- Separate FDD and TDD test cases

- CR split

| Spec | Section | Company |
| --- | --- | --- |
| 38101-4 | 3.3 Abbreviations 5.1.1.x Applicability of requirements for ATG | QC |
|  | 5.2.2.1.x FDD 2Rx 5.2.2.2.x FDD 4Rx 5.2.3.1.x TDD 2Rx 5.2.3.2.x TDD 4Rx | Huawei |
|  | A.3.2.1.x RMC for FDD A.3.2.1.x RMC for TDD | Ericsson |
| 38104 | 3.3 Abbreviations 4.6.1 Manufacturer declarations | CMCC |
|  | 8.2.x PUSCH requirements for ATG 11.2.1.x Requirements for PUSCH for ATG A.x FRC | Huawei |
| 38.141-1 | 3.3 Abbreviations 8.1.2.x Applicability of PUSCH for ATG performance requirements 8.1.2.x Applicability of PUCCH for ATG performance requirements 8.1.2.x Applicability of PRACH for ATG performance requirements | Samsung |
|  | 8.2.x Requirements for PUSCH for ATG A.x FRC | Ericsson |
| 38.141-2 | 3.3 Abbreviations 8.1.2.x Applicability of PUSCH for ATG performance requirements 8.1.2.x Applicability of PUCCH for ATG performance requirements 8.1.2.x Applicability of PRACH for ATG performance requirements | ZTE |
|  | 8.2.x Requirements for PUSCH for ATG A.x FRC | ZTE |