3GPP TSG-RAN WG4 Meeting # 108bis											R4-2316967

Xiamen, China, 9th – 13th October 2023

Title: 	WF on NTN RAN task

Agenda Item:	7.4

Source: 	MediaTek inc

Document for:	Approval

# Topic 1: NTN Testing

## Sub-topic 1-1: NGSO testing

### Issue 1-1: [NGSO][RF] Range of Doppler shift for RF frequency error requirements

Agreement: confirmed online

- For one of the cases in the freq error test, the range of absolute value of Doppler is greater than zero and less than or equal to 24ppm.

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The exact value(s) and the number of values (>1) to be selected are left to RAN5.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single frequency error measurement of the TE

### Issue 1-2: [NGSO][RF] Assumption of delay shift for RF frequency error requirements

Agreement:

- Constant value from the same ephemeris (i.e., orbit emulation) as the Doppler shift is derived

- UE is assumed to derive the amount of delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of delay

### Issue 1-3: [NGSO][RRM] Range of Delay shift for RRM UL timing accuracy test cases

Agreement: confirmed online

- The range of the one-way delay from UE to satellite is within 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest for LEO orbit 1200 km)

  - Assuming that the absolute elevation angle is larger than 30 degree

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The set of values is to be introduced by RAN4.  RAN5 to select from this set according to RAN4’s test case revision.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of time delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single timing accuracy measurement of the TE

### Issue 1-4: [NGSO][RRM] Assumption of Doppler shift for RRM UL timing accuracy test cases

Agreement:

- Constant value from the same ephemeris (i.e., orbit emulation) as the delay is derived

- UE is assumed to derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of Doppler

### Issue 1-5: [NGSO][RRM] Assumption of Doppler and timing shifts for RRM test cases other than UL timing accuracy

Agreement:

- Constant non-zero time delay, with constant non-zero Doppler value per satellite

## Sub-topic 1-2: GSO testing

Agreement:  Apply the same principles for GSO as agreed for NGSO

### Issue 2-1: [GSO][RF] Range of Doppler shift for RF frequency error requirements

Agreement:

- For one of the cases in the freq error test, the range of absolute value of Doppler is greater than zero and less than or equal to 0.93ppm.

### Issue 2-2: [GSO][RF] Assumption of delay shift for RF frequency error requirements

Agreement

- Constant value from the same ephemeris (i.e., orbit emulation) as the Doppler shift is derived

- UE is assumed to derive the amount of delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of delay

### Issue 2-3: [GSO][RRM] Range of Delay shift for RRM UL timing accuracy test cases

Agreement

- The range of the one-way delay from UE to satellite is within 119.375ms to 128.79ms

  - Assuming that the absolute elevation angle is larger than 30 degree

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The set of values is to be introduced by RAN4. RAN5 to select from this set according to RAN4’s test case revision.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of time delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single timing accuracy measurement of the TE

### Issue 2-4: [GSO][RRM] Assumption of Doppler shift for RRM UL timing accuracy test cases

Agreement

- Constant value from the same ephemeris (i.e., orbit emulation) as the delay is derived

- UE is assumed to derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of Doppler

### Issue 2-5: [GSO][RRM] Assumption of Doppler and timing shifts for RRM test cases other than UL timing accuracy

Agreement

- Constant non-zero time delay, with constant non-zero Doppler value per satellite

## Sub-topic 1-3: Testing methodology suggestion to RAN5

### Issue 3-1: [Methodology] Whether to use test mode and/or realistic ephemeris vs. artificial ephemeris

No agreement to this issue

### Issue 3-2: [Methodology] Random value Doppler/timing shift and ephemeris selection

No agreement to this issue

### Issue 3-3: [Methodology] Whether send an update LS to RAN5

No agreement to this issue

## Sub-topic 1-4: Issues realted to RAN4 spec update

### Issue 4-1: [Spec] Introduction of channel model

No agreement to this issue

### Issue 4-2: [Spec] Other notes to be added or modified

No agreement to this issue

# Topic 2: LS on applicability of the requirements in TS 36.101 Clause 8 and Clause 9 to IoT NTN UEs (R5-235817)

## Sub-topic 2-1: Answers to RAN5

### Issue 2-1: Answer to Q3: This table only captures the physical channels for eMTC and RAN4 will add a new table for NB1/NB2.

Agreement:

- Answer to Q3: This table only captures the physical channels for eMTC and RAN4 will add a new table for NB1/NB2

### Issue 2-2: For Q1 and Q2, to seek the common understanding on the following issue first

Agreement:

- All the Category M1 NTN UEs should support TN access.