3GPP TSG-RAN WG4 Meeting # 108-bis         	R4-2316985

# Xiamen, China, October 9 –13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | draftCR | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.106 with Clause 9: conducted receiver requirement for NCR-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces clause 9 to repeater core specification: Clause 9: conducted receiver requirement for NCR-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 9 introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 repeater specification would not include conducted receiver requirement for NCR-MT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 9. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.115-1, TS 38.115-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of CR to TS 38.106>

# 9	Conducted receiver characteristics for NCR-MT

## 9.1	General

Conducted receiver characteristics are specified at TAB connector for NCR type 1-C and NCR type 1-H, with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for conducted receiver characteristics requirements in clause 9:

-	Requirements apply during the receive period.

-	Requirements shall be met for any transmitter setting.

-	Throughput requirements defined for the conducted receiver characteristics do not assume HARQ retransmissions.

-	When NCR-MT is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS, blocking and intermodulation characteristics, the negative offsets of the interfering signal apply relative to the lower passband edge or inter-passband gap, and the positive offsets of the interfering signal apply relative to the upper passband edge or inter-passband gap.

NOTE 1:	In normal operating condition the NCR-MT in TDD operation are configured to TX OFF power during receive period.

## 9.2	Diversity characteristics

## [TBA]

## 9.3	Conducted reference sensitivity

### 9.3.1	NCR-MT reference sensitivity level

#### 9.3.1.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H at which a throughput requirement shall be met for a specified reference measurement channel.

#### 9.3.1.2	Minimum requirements for NCR-MT type 1-C and 1-H

The wide area NCR-MT reference sensitivity level is specified the same as the wide area IAB-MT reference sensitivity level requirement for IAB-MT type 1-H in TS 38.174 [xx], subclause X.X.X, where references to nominal channel bandwidth apply to NCR-MT passband.

The local area NCR-MT reference sensitivity level is specified the same as specified in TS 38.101-1 [xx] clause x.x.x.

Referenced requirements applying to NB IoT are not applicable to the NCR-MT.

## 9.4	Conducted maximum input level

## Maximum input level is defined as the maximum mean power received at the local area NCR-MT antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in 38.101-1 [xx] Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in TS 38.101-1 [xx] in Table 7.4-1.

## 9.5	Conducted adjacent channel selectivity

### 9.5.1	Adjacent Channel Selectivity (ACS)

#### 9.5.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 9.5.1.2	Minimum requirement for NCR-MT type 1-C

#### For Wide area NCR-MT type 1-C, minimum requirement is the same as specified for BS type 1-C in TS 38.104 [2], subclause 7.4.1.2.

#### For Local area NCR-MT type 1-C, minimum requirement is the same as specified in TS 38.101-1 [XX], subclause 7.3.2.

#### 9.5.1.3	Minimum requirement for NCR-MT type 1-H

#### For Wide area NCR-MT type 1-H, minimum requirement is the same as specified for IAB-MT type 1-H in TS 38.174 [2], subclause X.X.X.X.

#### For Local area NCR-MT type 1-H, minimum requirement is the same as specified in TS 38.101-1 [XX], subclause 7.3.2.

## 9.6	Conducted blocking characteristics

#### 9.6.2.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the TAB connector for NCR-MT type 1-C and NCR-MT type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 9.6.2.2	Minimum requirement for NCR-MT type 1-C

#### For wide area NCR-MT type 1-C, minimum requirement is the same as specified for BS type 1-C in TS 38.104 [2], subclause 7.4.2.2.

#### For local area NCR-MT type 1-C, minimum requirement is the same as specified in TS 38.101-1 clause x.x.x.

#### 9.6.2.3	Minimum requirement for NCR-MT type 1-H

#### For wide area NCR-MT type 1-H, minimum requirement at TAB connector is the same as specified for IAB-MT in TS 38.174 [2], subclause x.x.x.x.

#### For local area NCR-MT type 1-H, minimum requirement at TAB connector is the same as specified in TS 38.101-1 clause x.x.x.

## 9.7	Conducted spurious response

## 9.7.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

#### 9.7.2	Minimum requirements for NCR-MT type 1-C and NCR-MT type 1-H

#### For Local area NCR-MT type 1-C and NCR-MT type 1-H, minimum requirements are the same as specified in TS 38.101-1 [xx] clause 7.7.

## 9.8	Conducted intermodulation characteristics

#### 9.8.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

#### 9.8.2	Minimum requirement for NCR type 1-C

For Wide area NCR-MT type 1-C , minimum requirement at antenna connector is the same as specified for BS type 1-C in TS 38.104 [2], subclause 7.7.2.

For Local area NCR-MT type 1-C, minimum requirement at antenna connector is the same as specified in TS 38.101-1[2], subclause x.x.x.

#### 9.8.3	Minimum requirement for NCR type 1-H

For Wide area NCR-MT type 1-H, minimum requirement at TAB connector is the same as specified for IAB-MT type 1-H in TS 38.174 [2], subclause x.x.x.

For Local area NCR-MT type 1-H, minimum requirement at TAB connector is the same as specified  in TS 38.101-1 [2], subclause x.x.x.

## 9.9	Conducted spurious emissions

#### 9.9.1	General

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the antenna connector. The requirements only apply to NCR-MT type 1-C for TDD operation.

For each antenna connectors on BS-side and UE-side supporting both RX and TX in TDD, the requirements apply during the transmitter OFF state. For antenna connectors both BS-side and UE-side in FDD, the RX spurious emissions requirements are superseded by the TX spurious emissions requirements, as specified in clause 6.5.4.

For multi-band connectors that both transmit and receive in operating band supporting TDD, RX spurious emissions requirements are applicable during the TX OFF state, and are subject to exclusion zones in each supported operating band.

For Band n41 and n90 operation in Japan, the sum of receiver spurious emissions requirements over all antenna connectors for repeater type 1-C shall not exceed minimum requirements defined in clause 6.5.5.2.

#### 9.9.2	Minimum requirements for NCR-MT type 1-C and type 1-H

The receiver spurious emissions requirements for NCR-MT type 1-C at the antenna connector and NCR-MT type 1-H are that for each TAB connector, the power of emissions shall not exceed the value specified in table 9.9.2-1.

Table 9.9.2-1: NCR-MT type 1-C and type 1-H receiver spurious emissions minimum requirements

| Spurious frequency range | Minimum requirements | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 3, Note 5 |
| 12.75 GHz - 26 GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 6 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the UL operating band is reaching beyond 12.75 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the repeater transmitter operating band to ΔfOBUE above the highest frequency of the repeater transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.5.1. For multi-band connectors, the exclusion applies for all supported operating bands. NOTE 5: Does not apply for band n104. NOTE 6: Applies only for band n104. |  |  |  |

<End of CR to TS 38.106>