3GPP TSG-RAN WG4 Meeting 108-bis-e	R4-2317001

Xiamen, China, October 09 – October 13, 2023

Title: 	WF on demodulation and CSI requirements for NES

Agenda Item:	5.34.6

Source: 	Huawei, Hisilicon

Document for:	Approval

2-1 PDSCH requirements

Issue 2-1-1: Whether to define new requirements for SSB-less Scell operation

- Proposals

  - Option 1: Yes, RAN4 shall define demodulation minimum performance requirements for SSB-less SCell operation for inter-band CA for FR1 and co-located cells.

  - Option 2: No

Issue 2-1-2: Test setup for SSB-less Scell operation (If option1 is agreed in issue 2-1-1)

- Proposals

  - Option 1: RAN4 shall configure a PCell cell with SSB and TRS, and an SCell with only TRS (i.e., two RMCs are needed at the same time). A time offset, close to the CP length, shall be applied between PCell and SCell. Demodulation performance shall be measured on the SCell only.

  - Other options are not precluded

Issue 2-1-3: Whether to define new requirements for enhancement on cell DTX/DRX mechanism

Agreement: Don’t define new requirements for enhancement on cell DTX/DRX mechanism.

2-2 CSI requirements

Issue 2-2-1 Whether to define CSI requirements for spatial domain techniques

- Proposals

  - Option 1: Yes, RAN4 to define CQI and PMI requirements for type1 and 2 spatial domain adaption

  - Option 2: No

Issue 2-2-2 Whether to define CSI requirements for power domain techniques

- Proposals

  - Option 1: Yes, RAN4 to define CQI requirements with AWGN channel

  - Option 2: No

Issue 2-2-3: General test setup(If option 1 is agreed in issue 2-2-1 and 2-2-2)

- Proposals

  - Option 1:

    - Type 1 single-panel codebook

    - Two sub-configurations is configured, i.e. L=2, each sub-configuration is associated with one CSI-RS resource. The CSI-RS resource need further discussion.

    - FFS the N value. If N=1, use A-CSI reporting, TE can take turns triggering two sub-configurations and applying the CSI reported by UE.  If N=2, use P-CSI reporting, TE can take turns applying CSIs corresponding to two sub-configurations.

  - Other options are not precluded