3GPP TSG RAN WG4 Meeting #108bis		R4-2317005

Xiamen, China, 9 October – 13 October 2023

Title:	Reply LS on applicability of UE demodulation and CSI requirements for IoT-NTN UE

Response to:	R5-235817

Release:	Release 18

Work Item:	LTE_NBIOT_eMTC_NTN_req-UEConTest

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:

Contact person:	Licheng Lin

Licheng.lin@mediatek.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

RAN4 thanks RAN5 for raising these questions. After discussion, RAN4 would like to clarify applicability on demodulation and CSI requirements for Category M1 UE over NTN.

Based to TS36.306 Clause 4.3.38.1, Category M1 UE supporting NTN access (ntn-Connectivily-EPC-r17) shall also support the Rel-13 Category M1 CE Mode A UE capability (ce-ModeA-r13).

4.3.38.1 ntn-Connectivity-EPC-r17 This field indicates whether the UE supports NTN access. This field is only applicable if the UE supports ce-ModeA-r13 or any ue-Category-NB. […]

Therefore, according to RAN2 specification above, there are no Category M1 NTN UEs only supporting NTN access.

Answer to Q1 and Q2: There are no Category M1 NTN UEs only supporting NTN access. Demodulation and CSI testing of TN Category M1 in TS36.101 will suffice even if testing in a different frequency band.

Answer to Q3: As for the Table B.2-1 in TS 36.102, it is only applicable for Category M1 UEs and not applicable for Category NB1/NB2. RAN4 will add a new connection setup table for Category NB1/NB2 UEs in TS36.102 Clause B.2.

# 2	Actions

To RAN WG5

ACTION: 	RAN4 respectfully requests RAN5 to take the above information into account for their further work.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #109	November 13th - November 17th, 2023.	Chicago, IL, USA.

TSG RAN WG4 Meeting #110	February 26th - March 1st, 2024.		Athens, Greece.