3GPP TSG-RAN WG4 Meeting #108bis	R4-2317299

# Xiamen, China,  9th – 13th October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on interruption requirements for UE reporting NFG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New interruption requirements apply when a NeedforGapInfoNR capable UE measures inter-frequency MO-s without measurement gap but with interruptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the interruption length/ratio allowed for a UE measuring inter-frequency MO-s without measurement gap but with interruptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement is applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 8.2.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

##### 8.2.2.2.X	Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

When a UE supports [NeedForInterruptionInfoNR-R18] measurements and indicates [no-gap-with-interruption] on intra-frequency SSB-based or inter-frequency SSB-based measurements, the UE is allowed to cause interruptions while performing measurements on the frequency layers of the bands for which [no-gap-with-interruption] is indicated. Requirements in this section applies only when the UE is in SA operation mode.

The UE is allowed to cause interruption with interruption ratio no more than the requirements specified below upon UE measurements on a specific frequency layer that corresponds to the configured MO, where Tcycle,i is the measurement cycle on a certain frequency layer i according to the network configuration, specified in sub-clause [TBD].

UE is allowed to cause interruption on a certain frequency layer i:

-	up to [2.50%] probability of missed ACK/NACK when 80ms ≤ Tcycle,i < 160ms, or

-	up to [1.25%] probability of missed ACK/NACK when 160ms ≤ Tcycle,i < 320ms, or

-	up to [0.625%] probability of missed ACK/NACK when 320ms ≤ Tcycle,i.

If measurement gap is configured and the gap occasions are partially or fully overlapped with the SMTC occasions on a certain frequency layer that UE indicates [no-gap-with-interruption], no interruption from measurements on such layer is allowed.

The interruptions are allowed for all the active serving cells in the same FR as NR MO being measured if UE supports per-FR measurement gaps, and all the serving cells if UE does not support per-FR measurement gaps.

Table 8.2.2.2.X-1: Interruption length L in FR1

|  | SCS (kHz) | NR Slot | Interruption length L (slots) |
| --- | --- | --- | --- |
|  |  | length (ms) |  |
| 0 | 15 | 1 | [1] |
| 1 | 30 | 0.5 | [2] |
| 2 | 60 | 0.25 | [4] |

Table 8.2.2.2.X-2: Interruption length L in FR2

|  | SCS (kHz) | NR Slot | Interruption length L (slots) |
| --- | --- | --- | --- |
|  |  | length (ms) |  |
| 2 | 60 | 0.25 | [3] |
| 3 | 120 | 0.125 | [6] |

Editors’ note: Discussion is ongoing on cases where DRX or measurement gap is configured. Further update to this sub-clause subjects to the conclusions of those discussions.

Editors’ note2: Definition of Tcycle resembles measurement cycle definition in the measurement period requirements.

Editors’ note3: Total interruption ratio requirements will be updated subject to further conclusions.

### <<End of Change1>>