3GPP TSG-RAN WG4 Meeting #108bis	R4-2317310

Xiamen, China, October 9th ‒ October 13th, 2023

Title: 	WF on RRM requirements for NR support for dedicated spectrum less than 5MHz

Agenda Item:	5.14.7

Source: 	Nokia

Document for:	Approval

# Core Requirements

## Sub-topic 1-1 Applicability rules of RLM requirements

Way Forward: For the applicability rules of RLM requirements for different bands.

-	Further discuss the applicability rules of RLM requirements for different bands

- Option 1: For band n100, the PDCCH parameters with 12RB BW apply; for other bands, the PDCCH parameters with 15RB BW apply.

- Option 2: RAN4 to define SSB based RLM requirements for only 12 PRBs.

## Sub-topic 1-2 PDCCH transmission parameters for RLM and BFD requirements

PDCCH transmission parameters for RLM and BFD requirements.

Agreement:

Use aggregation level 2 in the transmission parameters for RLM IS with 12 PRB PDCCH transmission BW.

## Sub-topic 1-3 & 1-4 Cell-identification delay requirements

Cell-identification delay requirement conditions used for cell-identification delay requirements.

Agreement:

Use existing requirement side and channel conditions.

## Sub-topic 1-5 PBCH transmission BW

RAN1 agreement: PBCH transmission BW is always 12 PRB.

Way forward:

Specify PBCH decoding requirements based on 12PRB PBCH.

## Sub-topic 1-6 SSB Index reading and MIB reading

Delay requirements for SSB Index reading and MIB reading.

Way forward:

Define the delay requirements for SSB Index reading and MIB reading using existing requirement side and channel conditions.

## Sub-topic 1-7 Side condition for NR target cell detection for RRC connection re-establishment and RRC connection release with re-direction

Side condition for NR target cell detection for RRC connection re-establishment and RRC connection release with re-direction.

Agreement:

-	Inter-frequency RRC connection re-establishment:

- Side conditions for NR is target cell detection for RRC connection re-establishment is Es/Iot≥-4 dB.

-	Intra-frequency RRC connection re-establishment:

- Side conditions for NR is target cell detection for RRC connection re-establishment is Es/Iot≥-6 dB.

-	RRC connection release with re-direction:

- Side conditions for NR target cell detection for RRC connection release with re-direction is Es/Iot≥-4 dB.

## Sub-topic 1-8 Side conditions for HO requirements

Side conditions for defining Tsearch in HO requirements for 12PRB SSB and Tsearch in HO requirements for 12PRB SSB.

Agreement:

-	Unknown intra-frequency target cell: target cell Es/Iot≥-2 dB.

-	Unknown inter-frequency target cell: target cell Es/Iot≥-2 dB.

-	Unknown intra-frequency target cell: [3]*Trs ms (target cell Es/Iot≥-2 dB).

-	Unknown inter-frequency target cell: [5]*Trs ms (target cell Es/Iot≥-2 dB).

## Sub-topic 1-9 Applicability rule addition

Way forward:

For a UE which supports only less than 5MHz CBW, discuss whether to add applicability rule of existing requirements for each of the requirements applicable for the less than 5 MHz UE.

## Sub-topic 1-10 Assistance information for PBCH is 12 or 20 PRBs

RAN4 has had discussions whether it is necessary to include information regarding whether the PBCH is 12 or 20 PRBs in the measurement object or HO command.

Way forward:

Further check whether:

- the SSB should be on sync raster for the scenario concerned in the WI,

- the SSB can also be on the channel raster.

# Performance Requirements

## Sub-topic 2-1 Tests for different CBW

Way forward:

Further discuss whether a UEs supporting 12 RB bandwidth also shall support 15 RB bandwidth configuration, at least in rel-18.

## Sub-topic 2-2 L3 and L1 accuracy requirements

Way forward:

Further discuss if existing SSB based L3 and L1 accuracy requirements applies for less than 5MHz BW UE.

## Sub-topic 2-3 Test cases for UE operating in 5 MHz

Way forward:

Shall RAN4 specify additional test cases for UE operating in 5 MHz?

## Sub-topic 2-4 Use of DRX and non-DRX in test cases

Way forward:

For RLM/BFD, link recovery and SSB index reading test cases, should RAN4 specify the test cases of non-DRX with 12 PRBs, while test cases with DRX are specified with 15 PRBs?

## Sub-topic 2-5 HO test cases

Way forward:

Is RAN4 to introduce HO test cases?

## Sub-topic 2-6 RLM test cases

Way forward:

Is RAN4 to introduce RLM test cases?

## Sub-topic 2-7 BFD and CBD test cases

Way forward:

Is RAN4 to introduce BFD and CBD test cases?

## Sub-topic 2-8 Intra-frequency measurement delay with index reading test cases

Way forward:

Is RAN4 to introduce test cases for intra-frequency measurement delay with index reading?

## Sub-topic 2-9 12 PRB test cases

Way forward:

Should the 12 PRB test cases be included as a sub-test case instead of independent test cases?

## Sub-topic 2-10 New test configuration

Way forward:

RAN4 to discuss whether and how to introduce a new test configuration with 3MHz to existing test cases for testing UE that supports less than 5MHz BW.

## Sub-topic 2-11 Method for introducing test cases

Way forward:

RAN4 to discuss the method to be followed when introducing the identified test cases for UE supporting less than 5 MHz. Which among the below options should be followed for introducing the identified test cases for UE supporting only less than 5 MHz?

- Option 1: Define applicability rule for the identified test cases (e.g., saying these testes are applicable for UE supporting only less than 5 MHz)

- Option 2: Introduce new test cases in the new section for the UE supporting only less than 5 MHz CBW

## Sub-topic 2-12 List of test cases

In the following table most of the proposed test cases are listed as proposed by companies. As there were some overlap between the test cases, moderator tried to combine the test cases accordingly. Companies can:

- Indicate which test cases they see should be introduced.

- Indicate which test cases they see being the same/overlapping.

Proposed list of test cases:

| TC# | Description | Company |
| --- | --- | --- |
| Cell reselection |  |  |
| 1 | Intra-frequency cell reselection in IDLE mode | Nokia |
| RRC Re-establishment |  |  |
| 2 | RRC Re-establishment, Intra-frequency | Nokia |
| RLM |  |  |
| 3 | RLM OOS, 12RB PDCCH, non-DRX |  |
| 4 | RLM IS, 12RB PDCCH, DRX |  |
| 5 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode with PDCCH 12 PRBs UE |  |
| 6 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE |  |
| 7 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE |  |
| 8 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode PDCCH 12 PRBs UE |  |
| 9 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | Nokia |
| 10 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |  |
| 11 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | Nokia |
| 12 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |  |
| BFD |  |  |
| 13 | BFR, 12RB PDCCH, non-DRX |  |
| 14 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode with PDCCH 12 PRBs UE |  |
| 15 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRXmode with PDCCH 15 PRBs UE |  |
| 16 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | Nokia |
| 17 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode | Nokia |
| Event triggered reporting |  |  |
| 18 | Event triggered reporting with SBI reading, Intra-frequency, non-DRX | Nokia |
| 19 | Event triggered reporting with SBI reading, Inter-frequency, non-DRX | Nokia |
| 20 | SA event triggered reporting tests without gap under non-DRX with SSB index reading for 12 PRBs of PBCH UE |  |
| 21 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 12 PRBs PBCH UE |  |
| 22 | SA event triggered reporting tests without gap under non-DRX for < 5MHz BW |  |
| 23 | SA event triggered reporting tests without gap under DRX for < 5MHz BW |  |
| 24 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for < 5MHz BW |  |
| 25 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for < 5MHz BW |  |
| 26 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for < 5MHz BW |  |
| 27 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for < 5MHz BW |  |
| Handover |  |  |
| 28 | Intra-frequency handover from FR1 to FR1; known target cell for < 5MHz BW |  |
| 29 | Intra-frequency HO, unknown target cell |  |
| L1-RSRP reporting |  |  |
| 30 | SSB based L1-RSRP measurement when DRX is not used | Nokia |
| 31 | SSB based L1-RSRP measurement when DRX is used | Nokia |