3GPP TSG-RAN WG4 Meeting #108bis 	R4-2317325

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RRM requirements for TCI activation before cell switch in R18 LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce TCI state activation requirements for the activation of neighbor cell TCI before cell switch |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce TCI state activation requirements for the activation of neighbor cell TCI before cell switch |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements are defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

8.X	Downlink TCI state activation delay for unified TCI before LTM cell switch command

8.X.1	Introduction

The requirements in this clause apply for a UE configured with DLorJoint-TCIState configurations on a candidate cell. UE shall complete the activation of downlink TCI state within the delay defined in this clause.

When the target DL TCI state refers to a candidate cell and the activation of TCI is performed before cell switch, the requirements in this clause are applicable provided that following conditions are met:

-	The candidate cell is known to the UE.

-	At least one SSB is available for candidate cell L1-RSRP measurement in every 160ms.

-	Editor’s Note: TBD other conditions

Editor’s Note: TBD definition of known candidate cell

### 8.X.2	Known conditions for downlink TCI state

The downlink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	Downlink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command

-	The target downlink TCI state remains detectable during the downlink TCI state switching period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI switching period

-	SNR of the downlink TCI state ≥ -3dB

Otherwise, the downlink TCI state is unknown.

### 8.X.3	 MAC-CE based downlink TCI state activation delay before LTM cell switch

Editor’s note: The title of this section may be revised.

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework.

The requirements in this clause shall apply if the candidate cell satisfies the known cell condition defined in 8.X.1. If the known cell condition is not met, longer delay may be expected.

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to

- receive the cell switch command and perform cell switch with TΔ = 0 and Tmargin = 0, or

- receive the PDCCH-order indicating RACH-based early TA acquisition to target cell with TfirstSSB = 0

at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk* ([TL1-RSRP ]+ TSSB-proc)/ NR slot length, where

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

[-	TL1-RSRP is measurement period for SSB-based L1-RSRP after MAC CE command is decoded by the UE; The SSB shall be the associated to the activated TCI state]

-	TSSB-proc = 2 ms;

-	TOk = 1 if target TCI state is not in the active TCI state list for candidate cell, 0 otherwise.

8.X.4	Active Downlink TCI state list update delay

Editor’s Note: TBD

<End of Change #1>