3GPP TSG-RAN4 Meeting #108bis		 R4-2317354

# Xiamen, China, 9 – 13 October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR: RRM requirements for initiation/cease of SLSS Transmissions with CCA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR to introduce requirements for initiation/cease of SLSS as per worksplit in R4-2314349. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: Requirements to initiate/cease SLSS transmissions with CCA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will be missing when UE uses a SyncRef UE as synchronization source on a carrier subject to CCA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section: 12.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## 12.3A	Initiation/Cease of SLSS Transmissions with CCA

### 12.3A.1	Introduction

The requirements in this subclause are applicable to the UE capable of sidelink communication in unlicensed spectrum when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	EUTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source on a carrier frequency subject to CCA.

#### 12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source

The requirements defined in subclause 12.3.1.1 apply when the NR Cell is used as synchronization reference source and when the UE is

-	out of coverage on the NR sidelink carrier and in-coverage with a serving cell on a NR non- sidelink carrier, or

-	 in coverage with a serving cell on a NR sidelink carrier.

#### 12.3A.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source

The requirements defined in subclause 12.3.1.2 apply when the EUTRAN Cell is used as synchronization reference source and when the UE is

-	out of coverage on the NR sidelink carrier and in-coverage with a serving cell on a LTE non-sidelink carrier.

#### 12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source

The requirements defined in subclause 12.3.1.3 apply when GNSS is used as synchronization reference source and when the UE is

-	out of coverage on the sidelink carrier and in-coverage with a serving cell on a non-sidelink carrier, or

-	in coverage with a serving cell on a NR sidelink carrier.

#### 12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source

The requirements apply when SyncRef UE is used as synchronization reference source and when the UE is

-	in any cell selection state, or

-	out of coverage on the sidelink carrier and is associated with a serving cell on a non-sidelink carrier, or

-	in coverage with a serving cell on a NR sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331[2] are met and when SyncRef UE is used as synchronization reference source and if syncTxThreshOoC is included in the preconfigured sidelink parameters.

The UE shall be capable of measuring the PSBCH-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease SLSS transmissions within Tevaluate,SLSS, as shown in Table 12.3A.1.4-1 when the SyncRef UE is transmitting S-SSB on a carrier frequency subject to CCA.

Table 12.3A.1.4-1: Tevaluate,SLSS when SyncRef UE is transmitting S-SSB on a carrier subject to CCA and is used as synchronization reference source

| SL-DRX cycleNote 1 [ms] | Tevaluate,SLSS [ms] |
| --- | --- |
| No SL-DRX | (4 + L1) x S-SSB periods |
| SL-DRX cycle ≤ 160ms | (4 + L1) x S-SSB periods |
| SL-DRX cycle > 160ms | (4 + L1) x SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured for SL UE, the SL-DRX cycle in the requirement is the shortest of all the configured SL-DRX cycles. When the shortest SL-DRX cycle UE used changes, the requirements do not apply to the time of transition. Note 2: L1 is the number of unavailable S-SSB period during Tevaluate,SLSS_CCA due to the CCA failures; where L1 ≤ L1, max and L1, max=[4]. |  |

FFS: The UE shall initiate SLSS transmission if any of the following conditions is met:

- L1 exceeds L1,max

If higher layer filtering for PSBCH-RSRP measurements is pre-configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the selected SyncRef UE as defined in TS 38.331 [2] used to derive transmission timing for sidelink communication:

-	PSBCH-RSRP related side conditions given in Clause 12.4 for a corresponding Band are fulfilled,

-	sidelink S-SSB_RP and S-SSB Ês/Iot according to Annex B. 4 for a corresponding Band are fulfilled.

----------------------END OF CHANGE----------------------------