3GPP TSG-RAN WG4 Meeting #108bis	R4-2317359

Xiamen, China, October 09 – 13, 2023

Title:	LS on improvement on FR2 SCell/SCG setup delay

Response to:

Release:	Rel-18

Work Item:	Further Mobility Enhancements (NR_Mob_enh2)

Source:	RAN4 #108bis

To:	RAN2

Cc:

Contact person:	Jani-Pekka Kainulainen

jani-pekka.kainulainen@nokia.com,

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

Furthermore, RAN4 has been working on WID objective 7 on NR mobility enhancements, which (as per RP-231475) is defined as per the following:

- To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay [RAN4, RAN2], including: - Availability and validation of the IDLE/INACTIVE mode measurement results to be reported [RAN4]; and - Definition of corresponding RRM requirements [RAN4]; and - If necessary based on RAN4 outcome, definition of corresponding signalling support [RAN2]. Note 6: RAN4 will coordinate in due course with RAN2 to start the work. Note 7: R4-2220415 serves as baseline for future work in RAN4 Note 8: With exception of the above scenarios, enhancements on IDLE/INACTIVE mode measurements and on UE behavior in IDLE/INACTIVE mode are not in scope.

LS from RAN4 #108 (R4-2314466) includes RAN4 agreements from the previous meetings.

RAN4 has been discussing about validity condition related to existing measurement solution. The following agreement has been made:

RAN4 #108bis <Agreement>: - The measurements are considered valid if both of the following conditions are satisfied - A) the measurement are performed within the last [X] seconds before it is reported - X value is network configured. Signalling details are up to RAN2 - FFS on the X value(s) and will be decided by RAN4 - If X is not defined then no requirements will be introduced - B) the reported measurement results satisfy measurement accuracy [at the measurement instance] - FFS on side conditions

This validity condition is applicable to two cases:

- UE not supporting EMR but reporting rel-18 existing measurement results in idle/inactive mode.

- UE supporting EMR and only reporting EMR results.

# 2	Actions

To RAN2

ACTION: RAN4 respectfully asks RAN2 to take RAN4 agreements into account and provide signalling support.

# 3	Dates of the next TSG RAN4 meetings

RAN WG4 Meeting #109		Nov 13 – Nov 17, 2023,			Chicago, US

RAN WG4 Meeting #110		Feb 26 – Mar 01, 2024,			Athens, GR