3GPP TSG-RAN WG4 Meeting # 108bis		R4-2317375

# Xiamen, China, October 9 – October 13, 2023

Agenda item:			5.30.6

Source:	OPPO

Title:	WF on R18 NR SL RRM requirements (part 2)

Document for:	Approval

Topic #1: RRM Core requirements for SL CA and co-channel co-existence (5.30.3.1, 5.30.3.3)

- Sub-topic 1-1: RRM requirements for SL CA

- Issue 1-1: Constraints on interruptions to WAN at SL carrier addition/release due to paging and SIB reception

  - Option 1(Ericsson, LGE, OPPO, Nokia): Add the constraints on interruptions to WAN at SL carrier addition/release during WAN paging or system information reception

  - Option 2(Xiaomi, QC): Do not introduce constraints on interruptions to WAN at SL carrier addition/release during WAN paging or SIB reception.

Agreement:

FFS if interruptions to WAN at SL carrier addition/release is allowed during WAN paging or system information reception

FFS whether the scenario is valid that SL carrier addition/release can collide with WAN paging or SIB reception in RRC_IDLE/RRC_INACTIVE mode and whether LS is needed.

FFS if RLF and CBD should be protected.

- Issue 1-2: Other constraints on interruptions to WAN at SL carrier addition/release

FFS:

  - Option 1(Ericsson): For SL mode 2, interruptions due to SL component carriers addition/release shall not apply while:

    - T310 timer is running for RLF on PCell

    - performing candidate beam detection on PCell/serving cell as specfied in section 8.5.5. and 8.5.6

- Sub-topic 2-1: RRM requirements for co-channel coexistence for LTE SL and NR SL

- Issue 2-1: Applicability for LTE SL and NR SL co-channel co-existence

Agreement:

  - The requirements for the UE capable of V2X sidelink communication when the UE is out of coverage or in coverage on the carrier used for V2X sidelink operation in section 12.1 in TS 38.133 apply when the UE is:

- 	configured in co-channel coexistence for LTE SL and NR SL operation with same carrier

Topic #2: RRM performance requirements for R18 SL (5.30.3.4)

- Sub-topic 3-1: General and work plan

- Issue 3-1: Work plan for RRM performance part of SL

Agreement:

Agree on updated work plan of test case design for R18 SL RRM performance part.

(1) 3GPP RAN4 #108-bis meeting (Oct, 2023) • Discuss to achieve consensus on the updated work plan [RAN4] • Initial discussion on the work splitting for test case design [RAN4] (2) 3GPP RAN4 #109 meeting (Nov, 2023) • Discuss and agree on the work splitting for test case design [RAN4] • Initial discussion on configurations/parameters for test cases [RAN4] (3) 3GPP RAN4 #110 meeting (Feb, 2024) • Continue the discussion on the configurations/parameters for test cases [RAN4] • Continue the discussion on draft test cases [RAN4] • Draft CR for test cases for SL-CA and SL-U are expected (4) 3GPP RAN4 #110bis meeting (Apr, 2024) • Finalize the configurations/parameters for test cases [RAN4] • Discuss and endorse/agree on the CRs of test cases [RAN4] (5) 3GPP RAN4 #111 meeting (May, 2024) • Discuss and finalize the remaining issue of test cases [RAN4] • Endorse/agree on all CRs of test cases [RAN4]

- Issue 3-2: Co-channel coexistence for LTE SL and NR SL

Agreement:

- RAN4 not to define new test case for co-channel coexistence for LTE SL and NR SL in R18.

- Sub-topic 3-2: RRM performance requirements for SL-U

- Issue 3-3: Test case list for SL CA RRM

FFS:

Discuss and agree on the test case list and work splitting for SL CA RRM perf. part.

| Clause number | Core RRM Requirements | Need of test case | Title of test case | Volunteer company |
| --- | --- | --- | --- | --- |
| [A.9.1.x] | Interruptions to WAN due to Sidelink Carrier Aggregation | FFS | TC 1: Test for interruption on WAN due to SL carrier addition/release |  |
|  | Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation | FFS |  |  |
| [A.9.1.y] | Selection / Reselection of Synchronization Reference Source for Sidelink Carrier Aggregation | FFS | TC2: Test for V2X synchronization reference selection/reselection with GNSS as the highest priority under SL CA |  |
|  |  |  | [FFS: other test case with different configuration of sync reference ] |  |

- Sub-topic 3-3: RRM performance requirements for SL-U

- Issue 3-4: General of test case for SL-U

FFS:

- Proposal 5: As it is agreed to reuse the existing Te requirements of UE transmit timing of SyncRef UE. RAN4 can consider reusing the same test case for SL-U.

- Proposal 6: The number of S-SSB not available at the UE due to LBT failure at SyncRef UE should be considered in the initiation/cease of S-SSB transmissions delay at the UE.

- Proposal 7: The number of S-SSB not available at the UE due to LBT failure at SyncRef UE should be considered in the SyncRef UE selection delay.

- Proposal 9: The UE channel occupancy while performing congestion control measurement has no impact due to the LBT failure.

- Proposal 11: No impact due to LBT operation on SL Selection / Reselection of relay UE.

FFS:

- Proposal 8: RAN4 can study if it can reuse the test cases for L1-RSRP measurements, since No impact is expected due to LBT operation on sidelink.

- Proposal 10: RAN4 can study if interruption to ACK/NACK on the NR PCell needs to be updated based on the agreements on RRM core requirements.

- Issue 3-5: Accuracy requirements for SL-U

FFS:

Discuss whether the accuracy requirements for NR-U can be reused for SL-U and new accuracy requirement for channel occupancy measurements under CCA needs to be defined similar to NR-U in clause 10.1.35.

- Issue 3-6: Test case list for SL-U RRM

FFS:

Discuss and agree on the test case list and work splitting for SL-U RRM perf. part

| Core RRM Requirements | Need of test case | Title of test case | Note | Volunteer company |
| --- | --- | --- | --- | --- |
| UE Transmit timing | FFS | TC1: Test for V2X UE transmit timing with SyncRef UE as synchronization reference source under SL-U | New test needs to be defined to for SynRef UE as synchronization reference source when subject to CCA. |  |
| Initiation/Cease of SLSS transmission | FFS | TC2: Test for initiation/cease of SLSS transmission with SyncRef UE as synchronization reference source under SL-U | Test needed to verify the correct behaviour of initiation/cease of S-SSB transmissions when subject to CCA |  |
| Selection/Reselection of V2X synchronization Reference source | FSS | TC3: Test for V2X synchronization reference selection/reselection with GNSS as the highest priority under SL-U | New test cases are needed to verify the correct behaviour of selection/reselection of SyncRef UE when subject to CCA. |  |
|  |  | TC4: Test for V2X synchronization reference selection/reselection with gNB as the highest priority under SL-U |  |  |
| L1 SL-RSRP measurement | FFS |  | Whether new tests are needed for L1 SL-RSRP and congestion control measurement depends on the outcome of ongoing discussions. FFS: No impact is expected due to LBT operation on sidelink. FFS: The UE channel occupancy while performing congestion control measurement has no impact due to the LBT failure. |  |
| Congestion control measurement | FFS |  |  |  |
| Interruption | FFS |  | FFS: New test cases need to be defined for interruption to WAN during SL-U operation FFS: interruption to ACK/NACK on the NR PCell needs to be updated |  |