3GPP TSG-RAN WG4 Meeting # 108bis											R4-2317392

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.14.7

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on FR2 HST RRM core requirement maintenance

Document for:	Approval

# Topic#1: RRM Core requirement maintenance

## Sub-topic 1-1: Simultaneous multi-panel operation

Issue 1-1-1: Scheduling restriction requirements

Agreement:

- No enhancement on the scheduling/measurement restriction requirements for SSB based L1 measurements.

Issue 1-1-2: Sweeping factor N1

Agreement:

- Scaling factor N1 = 1.5 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N1 = 4 when highSpeedMeasFlagFR2-r17 is configured to set2 when UE supports [simultaneousReceptionFR2HST-r18] and highSpeedDeploymentTypeFR2-r17 is configured as bidirectional.

Issue 1-1-3: Multi-panel simultaneous reception in the feature list

Way forward:

Further discuss how to reflect UE capability for multi-panel simultaneous reception in the feature list:

- Option 1: To support Rel-18 enhanced FR2 operation, RAN4 either to define in Rel-18 a new feature group (as described in Table below)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22. NR_HST_FR2 | 22-1 | Support of FR2 HST operation | 1) Support of FR2 UE PC6 2) Support of enhanced RRM requirements for FR2 HST (except the requirement for one shot large UL timing adjustment) 3) Support of demodulation processing for FR2 HST |  | Yes | No | UE does not meet FR2 high speed train scenario | Per Band | NO | FR2 only | N/A | FR2 UE power class PC6 signalling is used to indicate support of feature group | Optional with capability signalling |
| 22. NR_HST_FR2 | 22-2 | Support of one shot large UL timing adjustment | 1) Support of one shot large UL timing adjustment | 22-1 | Yes | No | UE does not support one shot large UL timing adjustment | Per Band | NO | FR2 only | N/A |  | Optional with capability signaling |
| 22. NR_HST_FR2 | 22-x | Support of FR2 HST with simultaneous reception operation | 1) Support of enhanced RRM requirements for FR2 PC6 UE with two panel simultaneous reception. | 22-1 | Yes | No | UE does not meet FR2 high speed train scenario with two panel simultaneous reception | Per Band | NO | FR2-1 only | N/A |  | Optional with capability signaling |

- Option 2: Add a new component to FR2 HST feature group in Rel-18.

- Other options are not precluded

## Sub-topic 1-2: Intra-band carrier aggregation (CA) scenario

Issue 1-2-1: SCell activation delay requirement

Way forward:

Further discuss additional condition/capability needed for SCell activation delay for PC6 UE:

- Option 1: The SCell activation delay requirement does not need to be enhanced for Rel. 18 PC6 UEs by default, i.e., SCellwithoutSSB capability can be reused to indicate the support for “3 ms” SCell activation delay (revert the CR from RAN4#108)

- Option 2: Keep the Core requirement if a test for the 3ms activation delay in the HST FR2 Rel-18 performance part is introduced

- Option 3: Keep the Core requirements and FFS for test cases

Issue 1-2-2: CA related capabilities for the feature list

Way forward:

Further discuss whether need to distinguish UE feature and UE capabilities for inter-frequency idle mode and connected mode.

Further discuss how to reflect UE capability for inter-frequency measurements and CA in the feature list:

- Option 1:

  - Indicate the support for RRM requirements and inter-frequency measurements requirements for Connected mode as a single capability.

  - Indicate the support for inter-frequency measurements requirements for idle mode as two separate capabilities for cell re-selection measurements and idle inactive measurement reporting.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Enhanced RRM requirements specified for CA including inter-frequency measurement in connected mode for FR2 HST | Support of the enhanced RRM for requirements CA to support FR2 high speed as specified in TS 38.133 | Rel-17 RAN4 feature 22-1 | Yes | No | The performance of RRM for CA in FR2 HST scenario cannot be guaranteed | Per UE | NO | FR2 only | N/A |  | Optional with capability signalling |
| x-2 | Enhanced RRM requirements specified for inter-frequency measurement in Idle and Inactive mode for FR2 HST cell re-selection | Support of the enhanced RRM requirements for inter-frequency measurement for cell re-selection in idle and Inactive mode to support FR2 high as specified in TS 38.133 | Rel-17 RAN4 feature 22-1 | No | No | The performance of RRM for inter-frequency measurement in idle and Inactive mode for FR2 HST cannot be guaranteed | Per UE | NO | FR2 only | N/A |  | Optional without capability signalling |
| x-3 | Enhanced RRM requirements specified for inter-frequency measurement in Idle and Inactive mode for FR2 HST idle inactive measurement reporting | Support of the enhanced RRM requirements for inter-frequency measurement for early measurement reporting in idle and Inactive mode to support FR2 high speed as specified in TS 38.133 | Rel. 16 RAN4 feature 18-7 and Rel-17 RAN4 feature 22-1 | Yes | No | The performance of RRM for inter-frequency measurement in idle and Inactive mode for FR2 HST cannot be guaranteed | Per UE | NO | FR2 only | N/A |  | Optional with capability signalling |

  - Other options are not precluded.

### Sub-topic 1-3: UL timing adjustment solutions and Tunnel deployment

Issue 1-3-1: SCell activation delay requirement

Way forward:

Further discuss how to reflect the [highSpeedTCISwitchEnhMAC-CE-FR2-r18] UE capability in the feature list:

- Option 1: For Rel-18, extend the existing 22-1 feature group (Support of FR2 HST operation) defined in Rel-17 with the component corresponding to the support of enhanced MAC-CE indication.

- Option 2: The UE capability [highSpeedTCISwitchEnhMAC-CE-FR2-r18] to support of Rel-18 enhanced MAC-CE indication may be introduced as a new feature group: optional with capability signalling and with 22-1 Support of FR2 HST as a prerequisite.

- Other options are not precluded.

Issue 1-3-2: Timing adjustment and UL spatial relation switch

Way forward:

- Further check whether and how UL spatial relation switch shall be executed when corresponding DL TCI state is switched in Rel-18 bi-directional deployments with multi-panel reception when large one shot UL timing adjustment is used (other conditions FFS)

  - Option 1: UL spatial relation switch shall always be executed strictly when the corresponding DL TCI state switches

  - Other options are not precluded