3GPP TSG-RAN WG4 Meeting #108-bis                                                         R4-2317393

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.12.5

Source:	Samsung

Title:	WF on FR2 HST RRM performance requirements

Document for:	Approval

Introduction

For Rel-18 work item on enhanced NR support for high speed train scenario in frequency range 2, there are two threads  [210] and [211] organized for RRM discussion on RAN4#108-bis, and this document is used to capture the WF on topics in thread [211].

Topic #1: RRM Performance Requirements for Rel-18 FR2 HST

Sub-topic 1-1: Scope of RRM Performance Requirements for simultaneous multi-panel operation

Issue 1-1-1: Necessity of Test Cases for SSB based RLM/BFD

- Way Forward:

  - TC for SSB based RLM/BFD

    - Option 1: No new case needed

    - Option 2: Need new test case

      - New test case for A.7.5.1.X RLM IS and OOS tests with UE simultaneous multi-panel reception.

      - New test case for A.7.5.5.X BFD and LR test with UE simultaneous multi-panel reception

Issue 1-1-2: Necessity of Test Cases for SSB based L1-RSRP

- Way Forward:

  - TC for SSB based L1-RSRP

    - Option 1: Need new test case for A.7.6 Measurement procedure

      - Option 1-1: New test case for A.7.6.3.5X SSB based L1-RSRP measurement when DRX is used for Rel-18 FR2 HST PC6 UE with simultaneous multi-panel operation

      - Option 1-2: New test case for A.7.6.3.X SSB based L1-RSRP measurement when DRX is not used for Rel-18 FR2 HST PC6 UE with simultaneous multi-panel operation based on A.7.6.3.5 by modifying its parameters, e.g., change to set2 or configure P=3.

    - Option 2: Need new test case for A.7.7 Measurement Performance requirements

      - Option 2-1: New test case for A.7.7.4.X SSB based L1-RSRP measurement test for Rel-18 FR2 HST PC6 UE with simultaneous multi-panel operatio

Sub-topic 1-2: Scope of RRM Performance Requirements for intra-band CA

Issue 1-2-1: The principle of defining the event triggering test on SCell

- Way Forward:

  - FFS whether event triggered test on intra-band SCell where cell detection would not be considered and only enhanced measurement period is to be verified

Issue 1-2-2: Necessity of Test Cases for intra-frequency measurement

- Way Forward:

  - The necessity of defining new test case is FFS:

    - Option 1: No new test case needed

      - RAN4 do not introduce new test case for intra-frequency measurement requirements for Rel-18 FR2 HST PC6.

    - Option 2: Need new test case

      - RAN4 to define a new test alternative to A.6.6.1.8 that would verify intra-frequency cell search and A6 even reporting for switching in between two Scells.

      - New test case for A.7.6.1.X SA event triggered reporting tests without gap under non-DRX for PC UE supporting [measurementEnhancementCAInterFreqFR2-r18]

Issue 1-2-3: Necessity of Test Cases for Cell re-selection requirement: Inter-frequency measurement in Idle mode

- Way Forward:

  - FFS Test Cases for Cell re-selection requirement: Inter-frequency measurement in Idle mode

    - Need new test case

      - New test case for A.7.1.1.X Cell reselection to FR2 inter-frequency NR case for Rel-18 FR2 HST intra-band CA

      - FFS define two new test cases, one for UE configured with highSpeedMeasFlag-r17, the other for UE supporting [measurementEnhancementCAInterFreqFR2-r18]

Issue 1-2-4: Necessity of Test Cases for inter-frequency measurement in connected mode

- Agreement:

  - To define the new test case for enhancement on inter-frequency measurement, both with and without SSB time index detection

- Way Forward:

  - FFS whether to define test case for enhancement on inter-frequency measurement considering with and without gap

  - FFS whether to define test case for enhancement on inter-frequency measurement considering DRX and non-DRX

  - FFS the new test cases list for inter-frequency measurements with gaps requirements for Rel-18 FR2 HST PC6

Issue 1-2-5: Necessity of Test Cases for Scell activation delay

- Way Forward:

  - New test case (s) are needed, but whether need to define one or two new test cases is FFS:

    - Option 1: Need new test case (s)

      - Option 1-1: New test case for A.7.5.3.X SCell activation and deactivation for SCell in Rel-18 FR2 HST intra-band CA

      - Option 1-2: One or two new test cases. RAN4 to define the test for direct SCell activation with delay requirement of “3 ms” by enhancing the existing “A.7.5.3.4 and/or A.7.5.3.5” SCell activation delay requirement tests with the UEs supporting the optional capability of “SCellwithoutSSB”

Sub-topic 1-3: Scope of RRM Performance Requirements for UL timing adjustment

The following agreement was achieved in Wednesday session:

Agreement: - Define a new R18 TC combining with UL timing adjustment and TCI state switch. - Further discuss the MRTD aspect. - For UE passes the R18 test case, not perform the corresponding R17 test.

Sub-topic 1-4: RRM TC configuration

Issue 1-4-1: Channel model and propagation condition

- Way Forward:

  - FFS Channel model and propagation condition for the TCs corresponding to the requirements on simultaneous multi-panel operation for train roof-mounted FR2 high power devices

    - RAN4 adopts bi-directional assumption (i.e., AWGN (serving cell) and AWGN with 19444 Hz frequency offset (neighbour cell))

Issue 1-4-2: Test setup of AOA configuration

- Way Forward:

  - FFS Test setup of AOA configuration for L1 measurement requirement test case

  - FFS Test setup of AOA configuration for the test cases including: Cell re-selection; SCell Activation Delay for Deactivated SCell; SA NR inter-frequency measurement; TCI switch delay

    - The legacy test setup of AOA configuration can be reused for test case verification.

Sub-topic 1-5: Measurement Accuracy Requirement for Rel-18 FR2 HST UE

Issue 1-5-1: Measurement accuracy for Rel-18 enhanced FR2 CPE with simultaneous reception

- SS-RSRP accuracy performance requirement

- Agreement:

  - Measurement accuracy for Rel-18 enhanced FR2 CPE with simultaneous reception

    - For SS-RSRP measurement (FR2 intra-frequency RSRP measurement), the legacy absolute/relative accuracy performance requirements can be reused

Issue 1-5-2: Measurement accuracy for SCells in Rel-18 FR2 HST`

- Way Forward:

  - Measurement accuracy for SCells in Rel-18 FR2 HST

    - For SS-RSRP measurement, the legacy accuracy requirements can be reused.

      - FFS the requirements contain: Intra-frequency SS-RSRP accuracy requirements and intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements

Issue 1-5-3: Measurement accuracy for inter-frequency in Rel-18 FR2 HST

- Way Forward:

  - Measurement accuracy for inter-frequency in Rel-18 FR2 HST

    - For SS-RSRP measurement, the existing inter-frequency RSRP measurement accuracy requirements are applicable.

      - FFS the requirements contain: Inter-frequency SS-RSRP accuracy requirements and Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements