3GPP TSG-RAN WG4 Meeting # 108bis											R4-2317427

Xiamen, China, 09 Oct. – 13 Oct. 2023

Agenda item:			5.7.5

Source:	Ericsson

Title:	WF on FR2_multiRx_part2

Document for:	Approval

# Introduction

The AI to be discussed are:

- 5.7.2.2 L1-RSRP Measurements

- 5.7.2.5 TCI State switching

- 5.7.2.6 Receive Time Difference.

# Topic #1: L1-RSRP measurements

## Open issues summary

### Sub-topic 1-1: General aspects.

Issue 1-1-1: General proposals:

- Proposals

  - Option 1: Fast beam sweeping is always enabled if UE supports the UE capability.

  - Option 2: Fast beam sweeping is always enabled if UE supports the UE capability and when UE indicates preference of multi-Rx operation

### Sub-topic 1-2: Measurement period requirements

Issue 1-2-1: Measurement period for L1-RSRP configured for GBBR

Background:

For SSB based L1-RSRP measurements in FR2 the measurement period for GBBR defined as:

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: N is FFS |  |

- N is FFS

- Option 1: N = [reduceNumberRxBeam] for UE supporting faster beam sweeping under multi-Rx operations; otherwise N=8. (vivo, OPPO, QC, HW, ZTE, MTK)

- Option 2: N = 8 + K, where K is the number of SSBs in each CMR set (Apple)

- FFS: For CSI-RS based L1-RSRP measurements in FR2, the existing L1-RSRP measurement period is reused when configured for GBBR.

Issue 1-2-2 a: measurement period for SSB based L1-RSRP

- Way forward: Companies are requested to bring further analysis.

  - option 1:  N = [reduceNumberRxBeam] for UE supporting faster beam sweeping under multi-Rx operations; otherwise, N=8

  - Option 2: N = 8 + K, where K is the number of SSBs in each CMR set (Apple)

Issue 1-2-2 b: measurement period for CSI-RS based L1-RSRP

- Way forward: Companies are requested to bring further analysis.

  - Option 1: For CSI-RS based L1-RSRP measurements in FR2, the existing L1-RSRP measurement period is reused when configured for GBBR.

  - Option 2: N = ceil(maxNumberRxBeam / K) + 1, where K is the number of CSI-RSs in each CMR set

Issue 1-2-3: Measurement period for non-GBBR (i.e., measurement period of L1-RSRP not configured for GBBR)

- Way forward: Companies are requested to bring further analysis to next meeting

Issue 1-2-4: Conditions for enhanced requirements for L1-RSRP not configured with GBBR

- Way forward: Companies are requested to bring further analysis to next meeting

### Sub-topic 1-3: Others

Issue 1-3-1: Shall L1-SINR requirements be defined for the multi-RX UE

- Way forward: Companies are requested to bring further analysis to next meeting

Issue 1-3-2a: Measurement period for L1-SINR (based on conclusion of issue 1-3-1)

- Way forward: Companies are requested to bring further analysis to next meeting

Issue 1-3-2b: Other enhancements for L1-SINR (based on conclusion of issue 1-3-1)

- Way forward: Companies are requested to bring further analysis to next meeting

Issue 1-3-3: UE behaviour at transitions between single-RX and multi-RX operation modes

- Way forward: Companies are requested to bring further analysis to next meeting

# Topic #2: TCI state switch

Main technical topic overview. The structure can be done based on sub-agenda basis.

### Sub-topic 2-1: General principle for defining requirements

Issue 2-1-1: UE behaviour when dual TCI states are not supported

- Agreement: Following proposal is agreed to be captured in specification.

  - Proposal 1: It is proposed to capture in 38.133 the agreement that no UE behaviour is defined in Rel-18 multi-Rx WI when UE cannot receive dual TCI states simultaneously

### Sub-topic 2-2: DCI based TCI state switch

Issue 2-2-1: DCI based dual TCI state switch for sDCI scenario

- Agreements:

  - No additional delay on top of Rel-16 requirements is introduced for s-DCI based dual TCI states switch

Issue 2-2-2: For sDCI, for dual TCI to single TCI state switch, when the target TCI is one of the source TCI (e.g. [RS1,RS2] to [RS1]),

- Agreement:

  - There is no TCI switching delay when UE is configured with GBBR and is NOT configured with non-GBBR

Issue 2-2-3: DCI based dual TCI state switch for mDCI scenario

Issue 2-2-2-1: DCI based dual TCI state switch delay for mDCI:

  - Suggest waiting for RAN1 reply as CRs can be agreed in next meeting only.

Issue 2-2-2-2: Other proposals for mDCI

- Agreement:

  - In mDCI scenario, TCI switching with one CORESETpoolindex does not cause interruptions on TCI states with another CORESETpoolindex. However, there is no specification impact is there for this proposal

### Sub-topic 2-3: MAC CE based TCI state switch

Issue 2-3-1: MAC-CE based PDCCH TCI state switch for s-DCI PDCCH repetition

- Agreement

  - No additional delay is needed

  - For the following case, do not define test.

    - If target dual TCI states are NOT in the active TCI state list;

    - [Tfirst_SSB] arrives with [125]us after MAC CE processing time, where Tfirst_SSB is the shorter one between Tfirst-SSB1 and Tfirst_SSB2

Issue 2-3-2: MAC CE based PDCCH TCI state switch for m-DCI scenario

- Wait for RAN1 LS response

### Sub-topic 2-5: Known conditions

Issue 2-5-2: Definition of known condition

- Way forward: Companies are requested to bring further analysis if the previous agreement are identified to be not sufficient.

### Sub-topic 2-6: Active TCI state list update

Issue 2-6-1: Active TCI state list update delay requirement for mDCI

- Agreement:

  - the existing requirement for active TCI state list update can be reused with the update that it is for each new TCI state being added

# Topic #3: Receive time difference

### Sub-topic 3-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-1-1: Whether L1-RSRP GBBR requirements should be extended to RTD >CP if the UE supports RTD>CP capability

- Way forward: Companies are requested to bring further analysis

Issue 3-1-1: How to capture RTD applicability conditions in multi-RX WI

- Way forward: Further discuss in next meeting.

  - Option 1: This condition is mentioned for each relevant multi-RX RRM requirement such as scheduling/measurement restriction, dual TCI state switching, L1-RSRP for GBBR.

  - Option 2: In clause 3.6	“Applicability of requirements in this specification version,” it is stated that RTD < CP as a condition for the relevant requirements to apply