3GPP TSG-RAN WG4 Meeting #108bis                                   R4-2317572

(Revision of R4-2316660)

Xiamen, China, Oct 9th – 13th, 2023

Agenda Item:	6.4.3

Source:	MediaTek Inc.

Title: 	Further discussion on UE RF requirements for B254 for IoT NTN operation

Document for:	Discussion

# Introduction

RAN4#108 continued to discuss UE RF requirements for L+S IoT NTN band, and the following agreements have been reached [1]:

Issue 1-1-2: For B254 category M1, Is it agreeable to introduce the following three NS flags in total for FCC or ETSI? Agreement: NS value; Regulatory domain; Applicable range (MHz); In-band emissions; Out-of-band emissions. NS_03N; FCC; 1610-1626.5; FCC (1610-1626.5); FCC (1559-1610MHz). Issue 1-2-1: For configured Tx power, should the current TS 36.102 requirements be reused for band B254? - Proposals - Option 1: Yes - Option 2: No - Recommended WF - Option 1? Agreement: Option 1. Issue 1-2-2: For power control requirements, should the current TS 36.102 requirements be reused for band B254? - Proposals - Option 1: Yes - Option 2: No - Recommended WF - Option 1? Agreement: Option 1. Issue 1-2-3: For frequency error requirements, should the current TS 36.102 requirements be reused for band B254? - Proposals - Option 1: Yes - Option 2: No - Recommended WF - Option 1? Agreement: Option 1. Issue 1-2-4: For transmit modulation quality requirements, should the current TS 36.102 requirements be reused for band B254? - Proposals - Option 1: Yes - Option 2: No - Recommended WF - Option 1? Agreement: Option 1. Issue 2-1-1: For band B254 category M1, REFSENS should be specified as: - Proposals - Option 1: -103.7dBm (no insertion loss, i.e., -106.2 + 2.5) - Option 2: -102.2dBm (Similar to B255 with 0.5dB insertion loss) - Option 3: -103.1dBm for HD-FDD (0.6dBm insertion loss), -102.3dBm for FDD (additional 0.8dBm Tx desense) - Recommended WF - TBA Agreement: - -102.2dBm for FDD - -103.1dBm for HD-FDD Issue 2-1-2: For band B254 category NB1/NB2, REFSENS should be specified as: - Proposals - Option 1: -108.2dBm - Option 2: Others, please elaborate - Recommended WF - Option 1? Agreement: - Agree on Option 1 Issue 2-2-1: For band B254 category M1, should the general requirements for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, i.e., also for band B255, be re-used as Out-of-Band Blocking (OOBB)? - Proposals - Option 1: Yes - Option 2: No, please propose a new value. - Recommended WF - Option 1? Agreement: Agree on Option 1.

And in this contribution, we further discuss other UE RF requirements, e.g., A-MPR for IoT NTN category NB1/NB2 etc.

# Discussion

## 2.1 Transmitter characteristics

The table below illustrates an overview on the Tx RF requirements and the progress until RAN4#108, and these requirements highlighted in yellow will be discussed in this contribution:

| (sub) Section in TS 36.102 | Discussed or not for L+S IoT NTN band |
| --- | --- |
| 6.2A.1 Maximum output power for Cat. M1 | Yes, agreed |
| 6.2A2 MPR for Cat. M1 | Yes, re-using 36.102 as a starting point |
| 6.2A.3 A-MPR for Cat. M1 | No |
| 6.2A.4 Configured transmitted power for Cat. M1 | Yes, re-using 36.102. |
| 6.2B.1 Maximum output power for Cat. NB1/NB2 | Yes, agreed |
| 6.2B2 MPR for Cat. NB1/NB2 | Yes, re-using 36.102 as a starting point |
| 6.2B.3 A-MPR for Cat. NB1/NB2 | No |
| 6.2B.4 Configured transmitted power for Cat. NB1/NB2 | Yes, re-using 36.102 |
| 6.3A.1 Minimum output power for Cat. M1 | Yes, agreed to re-use the current 36.102 requirement. |
| 6.3A2 Transmit OFF power for Cat. M1 | Yes, agreed to re-use the current 36.102 requirement. |
| 6.3A.3 ON/OFF time mask for Cat. M1 | Yes, agreed to re-use the current 36.102 requirement. |
| 6.3A.4 Power control for Cat. M1 | Yes, re-using 36.102. |
| 6.3B.1 Minimum output power for Cat. NB1/NB2 | Yes, agreed to re-use the current 36.102 requirement. |
| 6.3B2 Transmit OFF power for Cat. NB1/NB2 | Yes, agreed to re-use the current 36.102 requirement. |
| 6.3B.3 ON/OFF time mask for Cat. NB1/NB2 | Yes, agreed to re-use the current 36.102 requirement. |
| 6.3B.4 Power control for Cat. NB1/NB2 | Yes, re-using 36.102. |
| 6.4A.1 Frequency error for UE category M1 | Yes, re-using 36.102. |
| 6.4A.2 Transmit modulation quality for category M1 | Yes, re-using 36.102. |
| 6.4B.1 Frequency error for UE category NB1 and NB2 | Yes, re-using 36.102. |
| 6.4B.2 Transmit modulation quality for Category NB1 and NB2 | Yes, re-using 36.102. |
| 6.5A.1 General | No discussion needed |
| 6.5A.2 Occupied bandwidth for category M1 | No |
| 6.5A.3 Out of band emission for category M1 | No |
| 6.5A.4 Spurious emission for category M1 | No |
| 6.5B.1 General | No discussion needed. |
| 6.5B.2 Occupied bandwidth for category NB1 and NB2 | No |
| 6.5B.3 Out of band emission for category NB1 and NB2 | No |
| 6.5B.4 Spurious emission for category NB1 and NB2 | Yes, agreed to re-use the current 36.102 requirement, and draft CR for UE co-exist agreed. |
| 6.6A Transmit intermodulation for category M1 | No |
| 6.6B Transmit intermodulation for category NB1 and NB2 | No |

A-MPR for category M1

As stated in [2], for category M1, no A-MPR is needed to meet out-of-band emission requirements for both FCC and ETSI and no A-MPR is needed to meet ETSI in-band emission requirements for 1610-1618.25MHz, and no A-MPR is needed to meet ETSI in-band emission requirements for 1618.25-1626.5MHz either.

Proposal 1: No A-MPR for category M1 is introduced for B254.

A-MPR for category NB1/NB2

For A-MPR for category NB1/NB2, we conduct similar simulations where a full 12 tones allocation QPSK signal is generated and passed to a typical PC3 power amplifier. And LO is set to the center of the carrier. The simulation results are shown in the following figures.

- 3GPP SEM

- FCC

- ETSI, Fc = 1618.25 MHz – 1626.5 MHz

- ETSI, Fc = 1610 MHz – 1618.25 MHz

Fig. 1, MPR simulation results for 3GPP, FCC and ETSI requirements.

As a summary on the simulation, A-MPR to meet ETSI requirements are proposed as following Table - 1, and a new NS flag NS_04N is also proposed accordingly.

Table – 1, A-MPR for B254 PC3

| NS flag | Carrier frequency range | A-MPR (dB) |
| --- | --- | --- |
| NS_04N | 1610 MHz – 1618.25 MHz | 0 |
|  | 1618.25 MHz – 1626.5 MHz | 0 |

Proposal 2: No A-MPR for category NB1/NB2 for B254 to meet corresponding ETSI requirements.

Occupied bandwidth

For occupied bandwidth for category M1, the same requirements can be reused corresponding to 99% of the total integrated mean power of the transmitted spectrum on the assigned channel. For occupied bandwidth for category NB1 and NB2, similarly the current requirement in TS 36.102 should be re-used.

Proposal 3: Re-use the current 36.102 requirement for occupied bandwidth for band B254 for IoT NTN operation.

Out of band emission

Out of band emission requirements (spectrum emission mask) specified in TS 36.102 can be reused as general requirements for B254.

Proposal 4: Re-use the current 36.102 requirement for out of band emission for band B254 for IoT NTN operation.

Spurious emission for category M1

Spurious emission for category M1 specified in TS 36.102 can also be reused as general requirements for B254.

Proposal 5: Re-use the current 36.102 requirement for spurious emission for category M1 for band B254 for IoT NTN operation.

Tx intermodulation

There is no Tx intermodulation requirement for category M1 in TS 36.102, however, Tx intermodulation requirements in clause 6.7.1F of TS 36.101 is applied to category NB1 and NB2. Similarly, B254 can follow the same principle.

Proposal 6: Re-use the current 36.102 requirement for Tx intermodulation including both category M1 and NB1/NB2 for band B254 for IoT NTN operation.

## 2.2 Receiver characteristics

The table below illustrates an overview on the Rx RF requirements and the progress until RAN4#107, and these requirements highlighted in yellow will be discussed in this contribution:

| (sub)Section in TS 36.102 | Discussed or not for L+S IoT NTN band |
| --- | --- |
| 7.3A Reference sensitivity power level for UE category M1 | Yes agreed |
| 7.3B Reference sensitivity power level for UE category NB1 and NB2 | Yes agreed |
| 7.4A Maximum input level for category M1 | Yes, agreed to re-use the current 36.102 requirement. |
| 7.4B Maximum input level for category NB1 and NB2 | Yes, agreed to re-use the current 36.102 requirement. |
| 7.5A Adjacent Channel Selectivity for category M1 | No |
| 7.5B Adjacent Channel Selectivity for category NB1 and NB2 | No |
| 7.6A Blocking characteristics for category M1 | Yes, agree to re-use 36.102 requirements |
| 7.6B Blocking characteristics for category NB1 and NB2 | Yes, agree to re-use 36.102 requirements |
| 7.7A Spurious response for category M1 | Yes, agree to re-use 36.102 requirements |
| 7.7B Spurious response for category NB1 and NB2 | Yes, agree to re-use 36.102 requirements |
| 7.8A Intermodulation characteristics for category M1 | No |
| 7.8B Intermodulation characteristics for category NB1 and NB2 | No |

ACS

For ACS, the current requirements in TS 36.102 can be reused for B254.

Proposal 7: Re-use existing ACS requirements for B254 for IoT NTN operation.

Rx Intermodulation

For Rx intermodulation, similarly the existing requirements in TS 36.102 can be reused for B254 as well.

Proposal 8: Re-use existing Rx intermodulation requirements for B254 for IoT NTN operation.

# Conclusion

In this contribution we have the following proposals for UE RF requirements for L+S IoT NTN band:

Proposal 1: No A-MPR for category M1 is introduced for B254.

Proposal 2: No A-MPR for category NB1/NB2 for B254 to meet corresponding ETSI requirements. Proposal 3: Re-use the current 36.102 requirement for occupied bandwidth for band B254 for IoT NTN operation.

Proposal 4: Re-use the current 36.102 requirement for out of band emission for band B254 for IoT NTN operation.

Proposal 5: Re-use the current 36.102 requirement for spurious emission for category M1 for band B254 for IoT NTN operation.

Proposal 6: Re-use the current 36.102 requirement for Tx intermodulation including both category M1 and NB1/NB2 for band B254 for IoT NTN operation.

Proposal 7: Re-use existing ACS requirements for B254 for IoT NTN operation.

Proposal 8: Re-use existing Rx intermodulation requirements for B254 for IoT NTN operation.

# References

- R4-2314726 WF on IoT_NTN_FDD_LS_band, MediaTek Inc.

- R4-2311222 Power back-off for IoT in the NTN L-/S-band,	Apple