3GPP TSG-RAN WG4 Meeting #108bis	draft R4-2317594

Xiamen, China, Oct 9 -13, 2023

Title: 	WF on Beam correspondence requirements for RRC_INACTIVE and initial access

Agenda Item:	5.6.5

Source: 	Nokia

Document for:	Approval

# <Topic #1> Beam correspondence requirement

Agreement

- Define the core requirement (spherical coverage EIRP for msg1) with 2 dB relaxation from the connected mode as mandatory.

  - Capture “Tolerance of 2 dB applied for PRACH msg1” as note in Table 6.2.1.3-3 of TS 38.101-2.

# Informative: Implementation of the note to TS 38.101-2

Table 6.2.1.3-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| n262 | 2.9 |
| n263 | 2.3 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 4: Tolerance of 2 dB applied for PRACH msg1. |  |