3GPP TSG-RAN WG4 Meeting # 108-bis								             R4-2317603

Xiamen, China, 9th October – 13th October, 2023

Title: 	PC5 NR-U CA_n96B A-MPR for NS_53 NS_54

Agenda Item:	4.1.1.2

Source: 	Skyworks Solutions, Inc.,

Document for:	Approval

# 1	Introduction

During the RAN4 meeting #108, Power Class 5 (PC5) A-MPR for uplink (UL) carrier aggregation (CA) class B in Band n96 (also known as CA_n96B), was evaluated using simulation results in [1, 2]. In this contribution, we present our perspective based on both measurements and results. This document is a revision of R4-23167777 due to late measurement data collection.

# Discussion

## 2.1 Background

With regards to the NS_53 A-MPR levels: for aggregated bandwidth of up to 100 MHz, we observe similar A-MPR performance between [1] vs [2]. Evaluations for 120, 140 and 160 MHz are absent in [2]. Our conclusions are as follows:

- For cases where the A-MPR is dominated by the NS_53 in-band PSD requirement of -1 dBm/MHz, we propose that the PC5 A-MPR for UL CA be identical to that of the PC5 single UL carrier requirement.

- For cases where the A-MPR is driven by the NS_53 out-of-band (OOB) requirement of -27 dBm/MHz, it would be advisable to ensure that these OOB A-MPR requirements only affect the lower edge and the upper edge channels of Band n96. The inner channels would consequently be limited by the in-band PSD requirements.

## 2.2 Measurement results

#### Power Amplifier (PA) calibration

- Power Class: PC5

- Calibration point: 1 dB MPR for QPSK DFT-s-OFDM, 20 MHz 100RB3 waveform at 27 dB ACLR

- Post PA losses of 4 dB

- ACLR: 27 dB

- IQ image: -28 dB

- Carrier leakage: -28 dBc

Measurement results are presented as raw PA output back-off (OBO).

#### 2.2.1 NS_53 Measurement results

The calculated and the measured OBO to meet the NS_53 -1dBm/MHz in-band requirements are shown in Figure 1 and Figure 2 for DFT-s-OFDM, CP-OFDM, Full and Interlaced allocations respectively (DFT_F, DFT_I, CP_F,CP_I). Figure 3 shows the OBO to meet the NS_53 out-of-band -27dBm/MHz requirements.

Figure 1: PC5 DFT-s-OFDM OBO to meet NS_53 -1dBm/MHz in-band PSD requirements.

Figure 2: PC5 CP-OFDM OBO to meet NS_53 -1dBm/MHz in-band PSD requirements.

Figure 3: PC5 OBO to meet NS_53 -27dBm/MHz out-of-band PSD requirements.

Observation 1: For QPSK, the following A-MPR seem sufficient.

- DFT-s-OFDM full allocations:

  - 20MHz, A-MPR ≤ [9.0]dB

  - 40MHz, A-MPR ≤ [6.0]dB

  - 60MHz, A-MPR ≤ [4.0]dB

  - 80MHz, A-MPR ≤ [3.0]dB

  - 100,120,140,160MHz, A-MPR ≤ [3.0]dB

- DFT-s-OFDM partial allocations:

  - 20MHz, A-MPR ≤ [11.5]dB

  - 40MHz, A-MPR ≤ [8.5]dB

  - 60MHz, A-MPR ≤ [7.0]dB

  - 80MHz, A-MPR ≤ [5.5]dB

  - 100MHz, A-MPR ≤ [4.5]dB

  - 120,140,160MHz, A-MPR ≤ [3.5]dB

- CP-OFDM full allocations:

  - 20MHz, A-MPR ≤ [9.0]dB

  - 40MHz, A-MPR ≤ [5.5]dB

  - 60MHz, A-MPR ≤ [5.0]dB

  - 80MHz, A-MPR ≤ [5.0]dB

  - 100,120,140,160MHz, A-MPR ≤ [4.5]dB

- CP-OFDM partial allocations:

  - 20MHz, A-MPR ≤ [11.0]dB

  - 40MHz, A-MPR ≤ [8.5]dB

  - 60MHz, A-MPR ≤ [6.5]dB

  - 80MHz, A-MPR ≤ [5.5]dB

  - 100MHz, A-MPR ≤ [5.0]dB

  - 120,140,160MHz, A-MPR ≤ [4.5]dB

#### 2.2.2 NS_54 Measurement results

No A-MPR is needed to meet the NS_54 +17dBm/MHz in-band PSD requirements. NS_54 A-MPR is driven by the -27dBm/MHz out-of-band PSD requirements for f ≤ 5925MHz. This out-of-band requirement impact only the lowermost aggregated PCC/SCC channels of band n96. Thanks to the 20MHz guard-band separating the lower most channels from the NS_54 out-of-band frequency range, the OBO is expected to be lower than the NS_53 out-of-band OBO. The measured OBO are shown in Figure 4 for DFT-s-OFDM and CP-OFDM.

Figure 4: PC5 OBO to meet NS_54 -27dBm/MHz out-of-band requirements

Observation 2: For NS_54, the A-MPR is driven by the -27dBm/MHz out-of-band rqeuirement since no OBO is needed to meet the in-band +17dBm/MHz requirement. This means that A-MPR is only needed for the lower most PCC/SCC NR-ARFCN channels, i.e. for the channels closest to the frequency range <= 5925MHz. For QPSK,

- DFT-s-OFDM full allocations: the MPR seems sufficient to cover all cases,

- DFT-s-OFDM partial allocations and only for lowermost channels

  - 40,60MHz, A-MPR ≤ [4.0]dB

  - 80,100,120,140,160MHz, A-MPR ≤ [3.5]dB

- CP-OFDM full allocations: the MPR seems sufficient to cover all cases,

- CP-OFDM partial allocations

  - 40,60MHz, A-MPR ≤ [4.5]dB for lower most channels.

  - 100,120,140,160MHz, MPR seems sufficient.

# Conclusion

In this contribution we present PA output back-off measurement results for NR-U UL-CA n96B / n96C to meet the NS_53 and NS_54 requirements and make the following observations.

Observation 1: For QPSK, the following A-MPR seem sufficient.

- DFT-s-OFDM full allocations:

  - 20MHz, A-MPR ≤ [9.0]dB

  - 40MHz, A-MPR ≤ [6.0]dB

  - 60MHz, A-MPR ≤ [4.0]dB

  - 80MHz, A-MPR ≤ [3.0]dB

  - 100,120,140,160MHz, A-MPR ≤ [3.0]dB

- DFT-s-OFDM partial allocations:

  - 20MHz, A-MPR ≤ [11.5]dB

  - 40MHz, A-MPR ≤ [8.5]dB

  - 60MHz, A-MPR ≤ [7.0]dB

  - 80MHz, A-MPR ≤ [5.5]dB

  - 100MHz, A-MPR ≤ [4.5]dB

  - 120,140,160MHz, A-MPR ≤ [3.5]dB

- CP-OFDM full allocations:

  - 20MHz, A-MPR ≤ [9.0]dB

  - 40MHz, A-MPR ≤ [5.5]dB

  - 60MHz, A-MPR ≤ [5.0]dB

  - 80MHz, A-MPR ≤ [5.0]dB

  - 100,120,140,160MHz, A-MPR ≤ [4.5]dB

- CP-OFDM partial allocations:

  - 20MHz, A-MPR ≤ [11.0]dB

  - 40MHz, A-MPR ≤ [8.5]dB

  - 60MHz, A-MPR ≤ [6.5]dB

  - 80MHz, A-MPR ≤ [5.5]dB

  - 100MHz, A-MPR ≤ [5.0]dB

  - 120,140,160MHz, A-MPR ≤ [4.5]dB

Observation 2: For NS_54, the A-MPR is driven by the -27dBm/MHz out-of-band rqeuirement since no OBO is needed to meet the in-band +17dBm/MHz requirement. This means that A-MPR is only needed for the lower most PCC/SCC NR-ARFCN channels, i.e. for the channels closest to the frequency range <= 5925MHz. For QPSK,

- DFT-s-OFDM full allocations: the MPR seems sufficient to cover all cases,

- DFT-s-OFDM partial allocations and only for lowermost channels

  - 40,60MHz, A-MPR ≤ [4.0]dB

  - 80,100,120,140,160MHz, A-MPR ≤ [3.5]dB

- CP-OFDM full allocations: the MPR seems sufficient to cover all cases,

- CP-OFDM partial allocations

  - 40,60MHz, A-MPR ≤ [4.5]dB for lower most channels.

  - 100,120,140,160MHz, MPR seems sufficient.

# References

- R4-2311250 NR-U ULCA PC5 AMPR, 3GPP TSG-RAN WG4 #108 Toulouse, France, Apple.

- R4-2313823 Simulation results on UE RF NR-U UL CA A-MPR for PC5, 3GPP TSG-RAN WG4 #108 Toulouse, France, Charter.