3GPP TSG-RAN WG4 Meeting #108bis      	R4-2317633

Xiamen, China, 9 – 13 Oct, 2023

Title:	WF on interband 2UL CA co-ex simplification

Agenda item:	5.1.3

Source:	Nokia

Document for:	Approval

# 1	Background

This is a WF on interband 2UL CA co-ex simplification.

# 2 	Discussion

During RAN4#108bis [1] 2UL CA co-ex simplification was discussed and WF was assigned.

RAN4 agrees to clarify the intersection requirement and amend the informative note so that recommendation to RAN5 is not to test intersection requirement even it is a core requirement as single band testing guarantees necessary UE performance. This WF applies to LTE, NR CA and EN-DC.

Proposal for a CR for 38.101-1 NR CA section 6.5A.3.2.3	Spurious emissions for UE co-existence for Inter-band CA:

This clause specifies the additional requirements for inter-band uplink carrier aggregation configurations with the single CC uplink assigned to two NR bands for coexistence with protected bands for the specified uplink carrier aggregation configurations in Table 6.5A.3.2.3-1. The intersection of the requirements for the individual bands specified in clause 6.5.3.2 shall also apply for the specified uplink carrier aggregation configurations. Intersection of a requirement means that both UL constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply.

For inter-band carrier aggregation with two contiguous carriers assigned to one NR band, the requirements in subclause 6.5A.3.2.1 apply for that band.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the spurious emissions for UE co-existence requirements in subclause 6.5A.3.2.2 apply for that band.

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.3-1 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands the requirements in Table 6.5A.3.2.3-1  could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5A.3.2.3-1 and in clause 6.5.3.2 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

# 3	Conclusion

This WF has a proposal for a CR for 38.101-1 NR CA section 6.5A.3.2.3	Spurious emissions for UE co-existence for Inter-band CA. If this proposal is acceptable similar changes are being proposed for LTE UL interband CA in 36.101 and EN-DC UL DC in 38.101-3. CRs would be provided for next meeting for all thee specifications.

# 4	References

[1] R4-2317630, Discussions on interband 2UL CA co-ex simplification, RAN4#108bis