# 3GPP TSG-RAN WG4 Meeting #108bis	R4-2317648

# Xiamen, China, Oct 09 – Oct 13 , 2023

Title: 	WF on NR_NTN_enh_UERF

Agenda Item:	5.26.9

Source: 	ZTE

Document for:	Approval

Sub-topic 1  General

Issue 1-3: Antenna types for NTN UE

Agreement:

- Define the UE RF requirements based on the envelope of mathematical antenna model to cover both UE with mechanical steering antenna and UE with electronic steering antenna.

- FFS whether and how to capture the information in the TS.

Issue 1-4: UE types

Agreement:

- RAN4 to consider the following UE Types:

1/ Terminal Type 1 (Electronic steering antenna) with the following NFs:

- 2.5 dB NF (Class 1) - for both LEO and GEO;

- 6 dB NF (Class 2) - for LEO only.

2/ Terminal Type 2 (Mechanical steering antenna) with the following NFs:

- 2.5 dB NF (Class 1) – for both LEO and GEO.

Issue 2-4: EVM requirement

Agreement:

- Reuse the existing RF requirement with Modulation limited up to 16QAM and optional to support 64QAM

Issue 2-5: Beam switching requirement

Agreement/WF:

Take following information as working assumption for beam switching time:

Ø  For mechanical steering: the typical values can be 22 degree/second, 6~8 seconds (if 120 degrees steering is assumed) for inter-satellite beam switching.

Ø  For electronic steering: A typical value is FFS for beam steering.

Work split

| Sub-clause | Volunteer company |
| --- | --- |
| Clause 5 | Ericsson |
| Clause 9.1~9.2 | Ericsson |
| Clause 9.3 | ZTE |
| Clause 9.4 | THALES |
| Clause 9.5.1, 9.5.2, 9.5.3, 9.5.4 | THALES |
| Clause 9.5.5 | THALES (can be shared/joint-work) |
| Clause 9.6 | THALES (can be shared/joint-work) |
| Clause 10.1-10.2 | Huawei |
| Clause 10.3 | THALES, Huawei |
| Clause 10.4 | ZTE |
| Clause 10.5 | THALES (if no other volunteers) |
| Clause 10.6 | ZTE |
| Clause 10.7 | Ericsson |
| Annex | ZTE |