3GPP TSG-RAN WG4 Meeting #xxx	R4-2317657

<Meeting Location>, <Date> ‒ <Date>, 2023

Title: 	WF on CA_n34-n40 MSD

Agenda Item:	4.1.1.1

Source: 	Skyworks Solutions Inc, Huawei

Document for:	Approval

# Cross-band MSD for CA_n34-n40

## Background

Cross band MSD test point and values are provided in [1] related to CBW change:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n40 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2302.5 | 5 | 3.2 | >ACLR2 |
| n40 | n34 | 2350 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 19.5 | >ACLR2 |

It was commented that the first test point of n34 UL in band n40 correspond to a case that was already valid since the introduction of the band combination in Release 17. Also, some companies would prefer to further evaluate the values.

<Way forward/Agreement>:

- The proposed test points above can be used for further evaluation of the MSD values at next meeting.

- Based on the evaluation results:

  - The introduction of the first test point (n34 UL into n40 DL) can be discussed

  - The MSD value of the second test point (n40 UL into n34 DL) can be discussed and specified

- If the same combination is requested for PC2, companies are encouraged to provide MSD data for both test points.

# Reference:

[1] R4-2316769 CA_n34-n40 MSD	Skyworks Solutions Inc. RAN4#108bis