3GPP TSG-RAN WG4 Meeting # 108bis														R4-2317722

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.30.2.1.2 Tx requirements

Source:					LG Electronics

Title:	TP for TR 38.786 on the SL-U MPR and A-MPR

Document for:	Approval

- Background

This contribution provides text proposal on the PSSCH/PSCCH MPR/A-MPR for SL-U operation in unlicensed band based on the proposals in [1].

- Text Proposal

<<<<<<<<<<< Start of changes in section 6 >>>>>>>>>>

6.1.2	UE maximum output power reduction for NR SL-U

The following assumption can serve as a starting point for MPR simulation assumptions that are considered to derive MPR requirements for SL-U operation.

- Basic RF parameters for MPR of single carrier SL-U UE

  - Waveforms: CP-OFDM for SL-U

  - Supported CBW: 20/40/60/80/100MHz.

  - Modulation: QPSK/16-QAM/64-QAM/256-QAM

  - SCS: 15/30kHz

  - C-IMD: 45dBc or 60dBc

  - EVM and impairments

| Modulation | SystemEVM | PA only | Image |  |
| --- | --- | --- | --- | --- |
| QPSK | 17.5% | 10% | 34dB |  |
| 16QAM | 12.5% | 8% | 34dB |  |
| 64QAM | 8% | 4% | 34dB |  |
| 256QAM | 3.5% | 1.8% | 34dB |  |

  - IBE and impairment exceptions are same as NR-U

- Reuse IBE mask from TS38.10101 Table 6.4F.2.3-1

- Image/carrier exceptions position and specific test according to interlaces.

  - ACLR: 27dBc for both PC3/PC5 UE with NR MBW

  - SEM according to 38.101-1 clause 6.5F.2.2 as well as Table 6.1.2-4

  - PA calibration & PA configuration:

- Power class 5 (single PA): 1dB MPR and DFT-s-OFDM QPSK 100RB3 20MHz waveform with 27dB ACLR

  - MPR table format can be reused the MPR format of NR-U according to each transmitted channel e.g.PSSCH/PSCCH transmission, PSFCH transmission and PSBCH transmission if any issue is not seen..

  - The MPR will be applied to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel.

  - Follow the RB interlaced allocation based on Table 6.2F.2-1 and Table 6.2F.2-2 in TS38.101-1. Also refer the interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. FFS on the inter-laced RB allocation for PSFCH, and on the repeated RB allocation for S-SSB by RAN1

For SL-U in single CC operation, the simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported and the following constraints in Table 6.1.2-1 can be assumed.

Table 6.1.2-1: SL-U operation’s MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,125,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216,220,225,228,230,240,250,252,255,260,264,270 The Allowed LCRB will be decided by the supported SCS. |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-2: SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - Power per RB is same in PSFCH for all users - Total power is 20dBm - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB-set, RAN4 consider interlacing RBs for PSFCH For multiple RB-sets, RAN4 considers both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of interlaced transmission should meet at least 80% of channel bandwidth in a single RB-set [, Ngap = RBend – RBstart ]. - RB allocation method as NR SL legacy RB allocation is also considered. |

For S-SSB transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-3: SL-U UE’s MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, RAN4 consider both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of repeated S-SSB transmission should meet at least 80% of channel bandwidth.in a single RB-set [,Ngap = RBend – RBstart]. |

For MPR and A-MPR for SL-U wideband operation, RAN4 assumed in-carrier SEM as follows.

In the case of non-transmitted 20 MHz channel(s) on the edges of an assigned channel bandwidth the spectrum emission mask for operation with shared spectrum channel access, specified in Table 6.5F.2.2.0-1, is applied by using the total bandwidth of the remaining transmitted channels. The spectrum emission mask for non-transmitted channels is floored at -28dBr inside the assigned channel bandwidth and continues at same level until the channel bandwidth mask is intercepted, i.e. until ΔfOOB, as defined in 38.101-1 Clause 6.5F.2.2.0, equals BWChannel / 2. When ΔfOOB > BWChannel / 2 requirements in clause 6.5F.2.2.0 apply.

The In-Carrier SEM in gap between transmitted channels is defined in Table 6.1.2-4 and some examples are shown in Figure 6.1.2-1.

The relative power of any UE emission shall not exceed the most stringent levels given by the spectrum emission mask for operation with shared spectrum channel access with full channel bandwidth and the spectrum emission mask for non-transmitted channels with the channel bandwidth of the transmitted channels in the case of non-transmitted channels at the edge of an assigned channel bandwidth or spectrum emission mask defined for gaps between the transmitted channels.

An exception to the spectrum emission mask for non-transmitted channels allows a single [2] MHz bandwidth to extend to [-28] dBc relative to total transmit power, or [-20] dBm, whichever is the greatest.

Table 6.1.2-4: In-Carrier SEM in gap between transmitted channels for SL-U wideband operation

| Carrier BW (MHz) | TX bitmap | TX BW (MHz) | Gap (MHz) | ¦Frequency¦ in gap from transmitted channel and mask levels |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 10 | 20 | 30 | MHz |
| 60 | 101 | 40 | 20 | 0 | -20 | -23 |  |  | dBr |
| 80 | 1101 1011 | 60 | 20 | 0 | -20 | -23 |  |  | dBr |
|  | 1001 | 40 | 40 | 0 | -20 | -25 | -25 |  | dBr |
| 100 | 10100 01010 00101 | 40 | 20 | 0 | -20 | -23 |  |  | dBr |
|  | 10110 11010 01101 01011 | 60 | 20 | 0 | -20 | -23 |  |  | dBr |
|  | 10111 11011 11101 | 80 | 20 | 0 | -20 | -23 |  |  | dBr |
|  | 11001 10011 | 60 | 40 (adjacent from ’11’) | 0 | -20 |  | -25 |  | dBr |
|  |  | 60 | 40 (adjacent from’1’) | 0 | -20 | -25 |  |  | dBr |
|  | 10001 | 40 | 60 | 0 | -20 | -25 | -25 | -25 | dBr |

Figure 6.1.2-1: In-Carrier SEM in gap between transmitted channels for SL-U wideband operation

6.1.2.1	MPR for SL-U operation in single CC

6.1.2.1.1	MPR for simultaneous PSSCH/PSCCH transmission

6.1.2.1.1.1	LG Electronics’ simulation results (R4-2315542)

The following simulation scenarios are considered as Table 6.1.2.1.1.1-1.

Table 6.1.2.1.1.1-1: SL-U PSSCH/PSCCH MPR/A-MPR simulation scenarios

|  | Scenario | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 5 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 6 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation (Interlace_0) | 7 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 8 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 9 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 10 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 11 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band Full operation | 13 | CP-OFDM | 40 | Bitmap 10 | 15 |
|  | 14 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 15 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 16 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 17 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 18 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 21 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 22 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 23 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 00100 | 30 |
| Wide band Interlaced Operation (Interlace_0) | 30 | CP-OFDM | 40 | Bitmap 10 | 15 |
|  | 31 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 32 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 33 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 34 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 35 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 36 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 37 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 38 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 39 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 40 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 41 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 42 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 43 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 44 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 45 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 46 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Here, for RB set, ‘105RB0’ means that RB starts at RB index ‘0’ and transmitted RB size is 105 RB. And, Bitmap ‘1’ means the corresponding RB set is transmitted, and ‘0’ means the corresponding RB set is not transmitted for wide band operation.

Table 6.1.2.1.1.1-2 shows the all possible bitmap of sub-band configuration for wide band operation. The wide band operation can be aggregated with multiple of 20MHz based sub-band. For the SL-U PSSCH/PSCCH MPR simulation, only contiguous RB set bitmaps are considered.

Table 6.1.2.1.1.1-2: All possible RB set Bitmap of sub-band configuration

|  | Contiguous RB sets Bit map | Non-contiguous RB sets Bit map |
| --- | --- | --- |
| 20MHz | N/A | N/A |
| 40MHz | 11, 10, 01 | N/A |
| 60MHz | 111, 110, 011, 100, 001, 010 | 101 |
| 80MHz | 1111, 1110, 0111, 1100, 0011, 1000, 0001, 0110, 0100, 0010 | 1001, 1101, 1011, 1010, 0101 |
| 100MHz | 11111, 11110, 01111, 11100, 00111, 11000, 00011, 10000, 00001, 01110, 01100, 00110, 01000, 00010, 00100 | 10001, 11011, 11010, 01011, 11001, 10011, 10101, 10110, 01101, 10100, 00101, 10010, 01001, 01010, 11101, 10111 |

Table 6.1.2.1.1.1-3 and Figure 6.1.2.1.1.1-1 show the MPR simulation results for the scenarios.

Table 6.1.2.1.1.1-3: PSSCH/PSCCH MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.83 | 2.15 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.83 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.83 | 3.18 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.88 | 5.08 | 5.89 | 5.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 2.81 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.47 | 2.13 | 0.00 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.47 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.17 | 2.47 | 3.16 | 2.47 | 3.16 | 2.45 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.88 | 5.07 | 5.47 | 5.07 | 5.88 | 5.07 | 5.88 | 5.46 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.15 | 2.47 | 2.47 | 2.47 | 2.47 | 2.46 | 0.00 | 2.13 | 0.00 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.15 | 2.47 | 2.47 | 2.47 | 2.47 | 2.46 | 0.10 | 2.13 | 0.63 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.15 | 2.81 | 2.48 | 3.16 | 2.48 | 3.15 | 2.12 | 3.16 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.07 | 5.46 | 5.05 | 5.88 | 5.06 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.48 | 2.47 | 2.47 | 2.47 | 2.47 | 2.13 | 0.00 | 2.46 | 0.00 | 2.46 | 0.13 | 2.13 | 0.00 |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.48 | 2.47 | 2.47 | 2.47 | 2.47 | 2.13 | 0.35 | 2.46 | 0.63 | 2.47 | 0.64 | 2.13 | 0.10 |
|  | '64QAM' | 3.16 | 2.48 | 3.17 | 2.48 | 3.17 | 2.47 | 3.16 | 2.47 | 2.81 | 2.47 | 3.16 | 2.13 | 3.16 | 2.46 | 3.16 | 2.47 | 3.15 | 2.12 |
|  | '256QAM' | 5.88 | 5.06 | 5.47 | 5.08 | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.06 | 5.47 | 5.06 | 5.47 | 5.06 | 5.46 | 5.06 |

Figure 6.1.2.1.1.1-1: PSSCH/PSCCH MPR simulation results for SL-U power class 5

Considering the inner RB set bitmaps and the outer RB set bitmaps on top of the Full/Partial RB allocation, the PSSCH/PSCCH MPR for SL-U power class 5 can be proposed as Table 6.1.2.1.1.1-4 based on the simulation results.

Table 6.1.2.1.1.1-4 Maximum power reduction (MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full2 (dB) | Partial3 (dB) | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full4 (dB) |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 1.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 3.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1.1-5 apply. |  |  |  |  |  |  |

Table 6.1.2.1.1.1-5: Outer/Inner sub-band configuration for SL-U wideband operation

| Wideband operation channel bandwidth (MHz) | Contiguous sub-band configuration |  | Non-contiguous sub-band configuration |  |
| --- | --- | --- | --- | --- |
|  | Outer | Inner | Outer | Inner |
| 40 | 11, 10, 01 | N/A | N/A | N/A |
| 60 | 111, 110, 011, 100, 001 | 010 | 101 | N/A |
| 80 | 1111, 1110, 0111, 1100, 0011, 1000, 0001 | 0110, 0100, 0010 | 1101, 1011, 1010, 0101, 1001 | N/A |
| 100 | 11111, 11110, 01111, 11100, 00111, 11000, 00011, 10000, 00001 | 01110, 01100, 00110, 01000, 00010, 00100 | 11011, 11010, 01011, 11001, 10011, 10101, 10110, 01101, 10100, 00101, 10010, 01001, 11101, 10111, 10001 | 01010 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |  |  |

6.1.2.1.1.2	OPPO’s simulation results (R4-2316119)

With the agreement above, the simulation cases are updated with 60kHz deleted. For each case, the QPSK, 16QAM, 64QAM and 256QAM will be simulated.

Table 6.1.2.1.1.2-1 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 40 | 105RB0 | 30 |
|  | 5 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 6 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 8 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 9 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 10 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 11 | CP-OFDM | 40 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 13 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 14 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation contiguous RB allocation | 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 18 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 21 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 23 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 30 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 31 | CP-OFDM | 100 | Bitmap 00100 | 30 |
| Wide band operation Interlaced RB allocation | 32 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 33 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 34 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 35 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 36 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 37 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 38 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 39 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 40 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 41 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 42 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 43 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 44 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 45 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 46 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 47 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 48 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Corresponding MPR results are shown as below for case 1 to 14:

Table 6.1.2.1.1.2-2 MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.4 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 3.4 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.1 | 4.3 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.2.1.1.2-3 MPR for Wideband operation

|  | Bitmap | 10 | 100 | 1000 | 10000 | 010 | 0100 | 01000 | 00100 | 110 | 1100 | 11000 | 0110 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Figure 6.1.2.1.1.2-1, Contiguous and Interlaced Single CC MPR

Figure 6.1.2.1.1.2-2, Contiguous and Interlaced Wideband MPR

The case 15, 16, 17, 19, 20, 22, 24, 25, 26, 28, 29 can apply 8dB limit while the rest of the cases should apply 9dB limit.

So the exception bitmap can be listed as below:

Table 6.1.2.1.1.2-4 exception cases mapping to bitmap

| 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
| --- | --- | --- | --- | --- |
| 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
| 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
| 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
| 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
| 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
| 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
| 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
| 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
| 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
| 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |

The Exception MPR mapping for wideband operation is proposed as below:

Table 6.1.2.1.1.2-5: Exception bitmap mapping

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | 110, 011 |
| 80 | 1000, 1100, 0100, 0010, 0011, 0001 |
| 100 | 10000, 11000, 11100, 01000, 01100, 00110, 00010, 00111, 00011, 00001 |

6.1.2.1.1.3	Huawei’s simulation results (R4-2315226)

Referring to the simulation cases listed in Table 6.1.2.1.1.3-1, the MPR value are shown in Table 6.1.2.1.1.3-2 and Table 6.1.2.1.1.3-3 for below.

Table 6.1.2.1.1.3-1 Simulation cases for SL-U

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 5 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 6 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 7 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 8 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 9 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 10 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 11 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation | 13 | CP-OFDM | 40 | Bitmap 10 | 15 |
|  | 14 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 15 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 16 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 17 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 18 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 21 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 22 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 23 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Table 6.1.2.1.1.3-2  Maximum power reduction (MPR) for SL-U with Full Allocation

| Modulation | Case: Bandwidth/SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz/15k | 20MHz/30k | 40MHz/15k | 60MHz/30k | 80MHz/30k | 100MHz/30k |
| QPSK | 4.0 | 3.8 | 4.1 | 4.0 | 4.0 | 4.1 |
| 16 QAM | 4.1 | 3.9 | 4.1 | 4.1 | 3.8 | 4.2 |
| 64 QAM | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| 256 QAM | 7.5 | 7.7 | 7.6 | 7.8 | 7.5 | 7.9 |

Table 6.1.2.1.1.3-3  Maximum power reduction (MPR) for SL-U with Interlaced Allocation

| Modulation | Case: Bandwidth/SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz/15k | 20MHz/30k | 40MHz/15k | 60MHz/30k | 80MHz/30k | 100MHz/30k |
| QPSK | 4.0 | 4.1 | 4.0 | 4.1 | 4.1 | 4.1 |
| 16 QAM | 4.1 | 4.2 | 4.2 | 4.1 | 4.2 | 4.2 |
| 64 QAM | 5.8 | 5.9 | 5.7 | 5.6 | 5.9 | 6.1 |
| 256 QAM | 7.0 | 7.3 | 7.1 | 7.4 | 7.7 | 7.4 |

Table 6.1.2.1.1.3-4  Maximum power reduction (MPR) for SL-U with Wide band operation

| Modulation | Case: Bandwidth/Bitmap pattern |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 40MHz | 60MHz |  | 80MHz |  |  | 100MHz |  |  |  |  |  |
|  | 10 | 100 | 110 | 1000 | 1110 | 0110 | 10000 | 11110 | 01000 | 01100 | 01110 | 00100 |
| QPSK | 4.0 | 3.7 | 3.9 | 3.8 | 4.0 | 3.8 | 3.9 | 4.1 | 3.0 | 3.2 | 3.9 | 3.4 |
| 16 QAM | 4.0 | 3.9 | 4.1 | 3.9 | 4.1 | 3.9 | 3.9 | 4.2 | 3.7 | 3.8 | 4.0 | 3.7 |
| 64 QAM | 5.1 | 5.1 | 5.5 | 5.2 | 5.7 | 5.7 | 5.1 | 5.8 | 5.4 | 6.0 | 5.9 | 5.5 |
| 256 QAM | 6.4 | 6.8 | 6.9 | 6.3 | 7.0 | 7.1 | 6.5 | 7.3 | 6.7 | 7.4 | 7.3 | 7.0 |

Table 6.1.2.1.1.3-4: Proposed MPR for SL-U with single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.1 | ≤ 4.1 |
|  | 16 QAM | ≤ 4.2 | ≤ 4.2 |
|  | 64 QAM | ≤ 6.1 | ≤ 6.1 |
|  | 256 QAM | ≤ 8.0 | ≤ 7.7 |

6.1.2.1.1.4	Qualcomm’s simulation results (R4-2316791)

Table 6.1.2.1.1.4-1: Power class 5 PSSCH/PSCCH MPR simulation results

|  | case | Precoding | BW |  | RB Setup | SCS | QPSK and 16QAM MPR | 64 QAM MPR | 256 QAM MPR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 |  | 105RB0 | 15 | 3.5 | 3.8 | 6.2 |
|  | 2 | CP-OFDM | 20 |  | 50RB0 | 30 | 3.5 | 3.8 | 6.0 |
|  | 3 | CP-OFDM | 40 |  | 216RB0 | 15 | 3.6 | 3.7 | 6.0 |
|  | 4 | CP-OFDM | 60 |  | 160RB0 | 30 | 3.5 | 3.6 | 6.0 |
|  | 5 | CP-OFDM | 80 |  | 216RB0 | 30 | 3.6 | 3.7 | 6.2 |
|  | 6 | CP-OFDM | 100 |  | 270RB0 | 30 |  |  |  |
| Interlaced Allocation Single CC | 7 | CP-OFDM | 20 |  | 1 RB0 every 10RBs | 15 | 3.5 | 3.7 | 6.2 |
|  | 8 | CP-OFDM | 20 |  | 1RB0 every 5RBs | 30 | 3.6 | 3.7 | 6.1 |
|  | 9 | CP-OFDM | 40 |  | 1RB0 every 10RBs | 15 | 3.6 | 3.9 | 6.4 |
|  | 10 | CP-OFDM | 60 |  | 1RB0 every 5RBs | 30 | 3.5 | 3.7 | 6.3 |
|  | 11 | CP-OFDM | 80 |  | 1RB0 every 5RBs | 30 | 3.7 | 3.9 | 6.5 |
|  | 12 | CP-OFDM | 100 |  | 1RB0 every 5RBs | 30 | 3.6 | 4.1 | 6.6 |
| Wide band Full operation | 13 | CP-OFDM | 40 |  | Bitmap 10 | 15 | 3.6 | 3.7 | 6.0 |
|  | 14 | CP-OFDM | 60 |  | Bitmap 100 | 30 | 3.1 | 3.6 | 6.0 |
|  | 15 | CP-OFDM | 60 |  | Bitmap 110 | 30 | 3.3 | 3.7 | 6.0 |
|  | 16 | CP-OFDM | 80 |  | Bitmap 1000 | 30 | 3.1 | 3.6 | 6.0 |
|  | 17 | CP-OFDM | 80 |  | Bitmap 1100 | 30 | 3.3 | 3.7 | 6.0 |
|  | 18 | CP-OFDM | 80 |  | Bitmap 1110 | 30 | 3.4 | 3.6 | 6.0 |
|  | 19 | CP-OFDM | 80 |  | Bitmap 0100 | 30 | 1.3 | 3.6 | 6.0 |
|  | 20 | CP-OFDM | 80 |  | Bitmap 0110 | 30 | 1.3 | 3.5 | 5.9 |
|  | 21 | CP-OFDM | 100 |  | Bitmap 10000 | 30 | 3.1 | 3.6 | 6.0 |
|  | 22 | CP-OFDM | 100 |  | Bitmap 11000 | 30 | 3.3 | 3.7 | 6.0 |
|  | 23 | CP-OFDM | 100 |  | Bitmap 11100 | 30 | 3.4 | 3.6 | 6.0 |
|  | 24 | CP-OFDM | 100 |  | Bitmap 11110 | 30 | 3.4 | 3.7 | 6.2 |
|  | 25 | CP-OFDM | 100 |  | Bitmap 01000 | 30 |  |  |  |
|  | 26 | CP-OFDM | 100 |  | Bitmap 01100 | 30 | 1.4 | 3.5 | 5.9 |
|  | 27 | CP-OFDM | 100 |  | Bitmap 01110 | 30 | 1.5 | 3.8 | 6.5 |
|  | 28 | CP-OFDM | 100 |  | Bitmap 00100 | 30 |  |  |  |
| Wide band Interlaced operation | 29 | CP-OFDM | 40 |  | Bitmap 10 | 15 | 3.5 | 3.7 | 6.0 |
|  | 30 | CP-OFDM | 60 |  | Bitmap 100 | 30 | 3.2 | 3.8 | 6.1 |
|  | 31 | CP-OFDM | 60 |  | Bitmap 110 | 30 | 3.5 | 4.0 | 6.6 |
|  | 32 | CP-OFDM | 80 |  | Bitmap 1000 | 30 | 3.2 | 3.8 | 6.1 |
|  | 33 | CP-OFDM | 80 |  | Bitmap 1100 | 30 | 3.5 | 4.0 | 6.6 |
|  | 34 | CP-OFDM | 80 |  | Bitmap 1110 | 30 | 3.5 | 4.0 | 6.6 |
|  | 35 | CP-OFDM | 80 |  | Bitmap 0100 | 30 | 1.4 | 3.8 | 6.1 |
|  | 36 | CP-OFDM | 80 |  | Bitmap 0110 | 30 | 1.6 | 4.0 | 6.5 |
|  | 37 | CP-OFDM | 100 |  | Bitmap 10000 | 30 | 3.2 | 3.8 | 6.1 |
|  | 38 | CP-OFDM | 100 |  | Bitmap 11000 | 30 | 3.5 | 4.1 | 6.6 |
|  | 39 | CP-OFDM | 100 |  | Bitmap 11100 | 30 | 3.5 | 4.0 | 6.6 |
|  | 40 | CP-OFDM | 100 |  | Bitmap 11110 | 30 | 3.6 | 3.9 | 6.5 |
|  | 41 | CP-OFDM | 100 |  | Bitmap 01000 | 30 | 1.4 | 3.8 | 6.1 |
|  | 42 | CP-OFDM | 100 |  | Bitmap 01100 | 30 | 1.6 | 4.0 | 6.5 |
|  | 43 | CP-OFDM | 100 |  | Bitmap 01110 | 30 | 1.6 | 4.0 | 6.6 |
|  | 44 | CP-OFDM | 100 |  | Bitmap 00100 | 30 |  |  |  |

The power class 5 PSSCH/PSCCH simulation results in the table should be considered in the discussion of MPR requirements.

6.1.2.1.1.5	Xiaomi’s simulation results (R4-2315440)

Table 6.1.2.1.1.5-1 and Figure 6.1.2.1.1.5-1 show the MPR values for different modulations for PSCCH and PSSCH in single CC operation with full allocation:

Table 6.1.2.1.1.5-1 MPR for PSCCH and PSSCH in single CC operation

|  | case | Waveform | BW | RB Setup | SCS | MPR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | QPSK | 16QAM | 64QAM | 256QAM |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 | 3.2 | 3.4 | 5.1 | 14 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 | 3.2 | 3.5 | 5.5 | 14.5 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 | 3.5 | 3.5 | 5.1 | 14 |
|  | 4 | CP-OFDM | 60 | 160RB0 | 30 | 3.3 | 3.5 | 5.5 | 14.2 |
|  | 5 | CP-OFDM | 80 | 216RB0 | 30 | 3.2 | 3.5 | 5.1 | 14 |
|  | 6 | CP-OFDM | 100 | 270RB0 | 30 | 3.5 | 3.5 | 5.5 | 14.2 |

Figure 6.1.2.1.1.5-1 MPR for PSCCH and PSSCH in single CC operation with full allocation

Based on above simulation results for full allocation in single CC, we proposed:

The MPR of full allocation for PC5 SL-U shall be as below table 6.1.2.1.1.5-2.

Table 6.1.2.1.1.5-2 Proposed MPR for PSCCH and PSSCH in single CC operation.

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full (dB) |
| CP-OFDM | QPSK | 4 |
|  | 16 QAM | 4 |
|  | 64 QAM | 6 |
|  | 256 QAM | 14 |

6.1.2.1.2	MPR for S-SSB transmission

6.1.2.1.2.1	LG Electronics’ simulation results (R4-2315542)

Tx Power for multiple S-SSBs

- The equal power per each S-SSB is assumed for MPR and A-MPR simulation.

For S-SSB MPR/A-MPR, the following simulation scenarios of Table 6.1.2.1.2.1-1 are considered.

Table 6.1.2.1.2.1-1: SL-U S-SSB MPR/A-MPR simulation scenarios

| Sub-band RB sets | Scenario | Bitmap | S-SSB {11RBs}xN repeated RB location index | N (Repeated#) | SCS(kHz) |
| --- | --- | --- | --- | --- | --- |
| 1 (20MHz) | 1 | 1 | {0 11 22 33 44 55 66 77 88} | 9 | 15 |
|  | 2 | 1 | {0 23 46 69 92} | 5 | 15 |
|  | 3 | 1 | {0 95} | 2 | 15 |
|  | 4 | 1 | {0 11 22 33} | 4 | 30 |
|  | 5 | 1 | {0 39} | 2 | 30 |
| 2 (40MHz) | 6 | 11 | {0 11 22 33 44 55 66 77 88 111 122 133 144 155 166 177 188 199} | 18 | 15 |
|  | 7 | 11 | {0 23 46 69 92 111 134 157 180 203} | 10 | 15 |
|  | 8 | 11 | {0 94 111 205} | 4 | 15 |
|  | 9 | 10 | {0 11 22 33 44 55 66 77 88} | 9 | 15 |
|  | 10 | 10 | {0 23 46 69 92 } | 5 | 15 |
|  | 11 | 10 | {0 94} | 2 | 15 |
| 3 (60MHz) | 12 | 111 | {0 11 22 33 56 67 78 89 112 123 134 145} | 12 | 30 |
|  | 13 | 111 | {0 38 56 94 112 150} | 6 | 30 |
|  | 14 | 110 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 15 | 110 | {0 38 56 94 } | 4 | 30 |
|  | 16 | 100 | {0 11 22 33} | 4 | 30 |
|  | 17 | 100 | {0 38} | 2 | 30 |
|  | 18 | 010 | {56 67 78 89 } | 4 | 30 |
|  | 19 | 010 | {56 94} | 2 | 30 |
|  | 20 | 101 | {0 11 22 33 112 123 134 145} | 8 | 30 |
|  | 21 | 101 | {0 38 112 150} | 4 | 30 |
| 4 (80MHz) | 22 | 1111 | {0 11 22 33 56 67 78 89 111 122 133 144 167 178 189 200} | 16 | 30 |
|  | 23 | 1111 | {0 38 56 94 111 149 167 205} | 8 | 30 |
|  | 24 | 1110 | {0 11 22 33 56 67 78 89 111 122 133 144} | 12 | 30 |
|  | 25 | 1110 | {0 38 56 94 111 149 } | 6 | 30 |
|  | 26 | 1100 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 27 | 1100 | {0 38 56 94} | 4 |  |
|  | 28 | 1000 | {0 11 22 33} | 4 | 30 |
|  | 29 | 1000 | {0 38} | 2 | 30 |
|  | 30 | 0110 | {56 67 78 89 111 122 133 144} | 8 | 30 |
|  | 31 | 0110 | {56 94 111 49} | 4 | 30 |
|  | 32 | 0100 | {56 67 78 89} | 4 | 30 |
|  | 33 | 0100 | {56 94} | 2 | 30 |
|  | 34 | 1101 | {0 11 22 33 56 67 78 89 167 178 189 200} | 12 | 30 |
|  | 35 | 1101 | {0 38 56 94 167 205} | 6 | 30 |
|  | 36 | 1010 | {0 11 22 33 111 122 133 144} | 8 | 30 |
|  | 37 | 1010 | {0 38 111 149} | 4 | 30 |
|  | 38 | 1001 | {0 11 22 33 167 178 189 200} | 8 | 30 |
|  | 39 | 1001 | {0 38 167 205} | 4 | 30 |
| 5 (100MHz) | 40 | 11111 | {0 11 22 33 56 67 78 89 112 123 134 145 167 178 189 200 223 234 245 256} | 20 | 30 |
|  | 41 | 11111 | {0 38 56 94 112 148 167 205 223 261} | 10 | 30 |
|  | 42 | 11110 | {0 11 22 33 56 67 78 89 112 123 134 145 167 178 189 200} | 16 | 30 |
|  | 43 | 11110 | {0 38 56 94 112 148 167 205} | 8 | 30 |
|  | 44 | 11100 | {0 11 22 33 56 67 78 89 112 123 134 145} | 12 | 30 |
|  | 45 | 11100 | {0 38 56 94 112 148} | 6 | 30 |
|  | 46 | 11000 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 47 | 11000 | {0 38 56 94} | 4 | 30 |
|  | 48 | 10000 | {0 11 22 33} | 4 | 30 |
|  | 49 | 10000 | {0 38} | 2 | 30 |
|  | 50 | 01110 | {56 67 78 89 112 123 134 145 167 178 189 200} | 12 | 30 |
|  | 51 | 01110 | {56 94 112 148 167 205} | 6 | 30 |
|  | 52 | 01100 | {56 67 78 89 112 123 134 145} | 8 | 30 |
|  | 53 | 01100 | {56 94 112 148} | 4 | 30 |
|  | 54 | 01000 | {56 67 78 89} | 4 | 30 |
|  | 55 | 01000 | {56 94} | 2 | 30 |
|  | 56 | 00100 | {112 123 134 145} | 4 | 30 |
|  | 57 | 00100 | {112 148} | 2 | 40 |
|  | 58 | 11011 | {0 11 22 33 56 67 78 89 167 178 189 200 223 234 245 256} | 16 | 30 |
|  | 59 | 11011 | {0 38 56 94 167 205 223 261} | 8 | 30 |
|  | 60 | 11010 | {0 11 22 33 56 67 78 89 167 178 189 200} | 12 | 30 |
|  | 61 | 11010 | {0 38 56 94 167 205} | 6 | 30 |
|  | 62 | 11001 | {0 11 22 33 56 67 78 89 223 234 245 256} | 12 | 30 |
|  | 63 | 11001 | {0 38 56 94 223 261} | 6 | 30 |
|  | 64 | 10101 | {0 11 22 33 112 123 134 145 223 234 245 256} | 12 | 30 |
|  | 65 | 10101 | {0 38 112 148 223 261} | 6 | 30 |
|  | 66 | 10110 | {0 11 22 33 112 123 134 145 167 178 189 200} | 12 | 30 |
|  | 67 | 10110 | {0 38 112 148 167 205} | 6 | 30 |
|  | 68 | 10100 | {0 11 22 33 112 123 134 145} | 8 | 30 |
|  | 69 | 10100 | {0 38 112 148} | 4 | 30 |
|  | 70 | 10010 | {0 11 22 33 167 178 189 200} | 8 | 30 |
|  | 71 | 10010 | {0 38 167 205} | 4 | 30 |
|  | 72 | 10001 | {0 11 22 33 223 234 245 256} | 8 | 30 |
|  | 73 | 10001 | {0 38 223 261} | 4 | 30 |
|  | 74 | 01010 | {56 67 78 89 167 178 189 200} | 8 | 30 |
|  | 75 | 01010 | {56 94 167 205} | 4 | 30 |
|  | 76 | 11101 | {0 11 22 33 56 67 78 89 112 123 134 145 223 234 245 256} | 16 | 30 |
|  | 77 | 11101 | {0 38 56 94 112 148 223 261} | 8 | 30 |

Here, for S-SSB {11RBs}xN repeated RB location index, ‘0’ means that {11RB} start RB index = 0. And, Bitmap ‘1’ means the corresponding RB set is transmitted, and ‘0’ means the corresponding RB set is not transmitted for wide band operation.

Table 6.1.2.1.2.1-2 and Figure 6.1.2.1.2.1-1 show the MPR simulation results for the scenarios.

Table 6.1.2.1.2.1-2: S-SSB MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.24 | 6.62 | 3.98 | 5.69 | 3.92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.17 | 8.63 | 5.56 | 9.02 | 6.95 | 3.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.29 | 6.72 | 7.87 | 5.31 | 5.57 | 3.94 | 4.54 | 3.52 | 7.67 | 5.49 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 10.34 | 6.86 | 8.81 | 5.60 | 7.67 | 5.38 | 5.68 | 4.06 | 6.11 | 4.12 | 4.42 | 3.52 | 8.92 | 5.97 | 7.55 | 5.21 | 7.63 | 5.31 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 11.15 | 6.75 | 10.18 | 5.56 | 9.44 | 5.91 | 7.75 | 5.22 | 5.79 | 3.91 | 6.02 | 3.44 | 5.60 | 3.35 | 4.43 | 3.53 | 4.43 | 3.53 | 10.18 | 7.03 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.18 | 6.32 | 8.95 | 6.04 | 8.61 | 4.98 | 9.08 | 4.99 | 7.67 | 5.10 | 7.87 | 5.45 | 7.62 | 5.41 | 6.90 | 4.60 | 10.68 | 5.79 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.2.1.2.1-2: S-SSB MPR simulation results for SL-U power class 5

Table 6.1.2.1.2.1-3 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and the number of S-SSB repetition per RB set.

Table 6.1.2.1.2.1-3 : S-SSB MPR simulation results for SL-U power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | 11.17 | 6.86 | 6.11 | 4.12 |
| Non-contiguous sub-band RB sets | 10.68 | 7.03 | 6.90 | 4.60 |

The S-SSB MPR for SL-U power class 5 can be proposed as Table 6.1.2.1.2.1-4 or Table 6.1.2.1.2.1-5 based on the simulation results when considering implementation margin.

Table 6.1.2.1.2.1-4 : S-SSB MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 8.5 | ≤ 6.5 |
| Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 9.5 | ≤ 7.0 |
| NOTE 1: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |  |  |

Table 6.1.2.1.2.1-5 :  S-SSB MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 9.5 | ≤ 7.0 |
| NOTE 1: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |  |  |

6.1.2.1.3	MPR for PSFCH transmission

6.1.2.1.3.1	LG Electronics’ simulation results (R4-2315542)

For PSFCH MPR/A-MPR, the following simulation scenarios of Table 6.1.2.1.3.1-1 are considered based on Alt 1-1b.

Table 6.1.2.1.3.1-1: SL-U PSFCH MPR/A-MPR simulation scenarios

| Sub-band RB sets | Scenario | Bitmap | PSFCH RB location index | SCS(kHz) |
| --- | --- | --- | --- | --- |
| 1 (20MHz) | 1 | 1 | {0 10 20 30 40 50 60 70 80 90 100 104} | 15 |
|  | 2 | 1 | {0 5 10 15 20 25 30 35 40 45 49} | 30 |
| 2 (40MHz) | 3 | 11 | {0 10 20 30 40 50 60 70 80 90 100 104 111 121 131 141 151 161 171 181 191 201 211 215} | 15 |
|  | 4 | 10 | {0 10 20 30 40 50 60 70 80 90 100 104} | 15 |
| 3 (60MHz) | 5 | 111 | {0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 6 | 110 | {0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 7 | 100 | {0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 8 | 010 | {56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 9 | 101 | {0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161} | 30 |
| 4 (80MHz) | 10 | 1111 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 11 | 1110 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 12 | 1100 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 13 | 1000 | { 0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 14 | 0110 | { 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 15 | 0100 | { 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 16 | 1101 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 17 | 1010 | { 0 5 10 15 20 25 30 35 40 45 49 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 18 | 1001 | { 0 5 10 15 20 25 30 35 40 45 49 167 172 177 182 187 192 197 202 207 212 216} | 30 |
| 5 (100MHz) | 19 | 11111 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 20 | 11110 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 21 | 11100 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 22 | 11000 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 23 | 10000 | { 0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 24 | 01110 | { 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 25 | 01100 | { 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 26 | 01000 | { 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 27 | 00100 | { 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 28 | 11011 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 29 | 11010 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 30 | 11001 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 31 | 10101 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 32 | 10110 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 33 | 10100 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 34 | 10010 | { 0 5 10 15 20 25 30 35 40 45 49 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 35 | 10001 | { 0 5 10 15 20 25 30 35 40 45 49 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 36 | 01010 | { 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 37 | 11101 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 223 228 233 238 243 248 253 258 263 268 272} | 30 |

Here, for PSFCH RB location index, ‘0’ means that RB index = 0. And, Bitmap ‘1’ means the corresponding RB set is transmitted, and ‘0’ means the corresponding RB set is not transmitted for wide band operation.

Table 6.1.2.1.3.1-2 and Figure 6.1.2.1.3.1-1 show the MPR simulation results for the scenarios.

Table 6.1.2.1.3.1-2: PSFCH MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 0.92 | 1.16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 1.76 | 0.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 2.55 | 1.81 | 1.14 | 0.79 | 2.59 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 2.24 | 2.12 | 1.79 | 1.05 | 1.63 | 0.83 | 2.83 | 2.60 | 2.74 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 1.85 | 2.31 | 2.54 | 1.82 | 1.10 | 1.45 | 1.60 | 0.63 | 0.93 | 2.68 | 2.86 | 2.98 | 2.87 | 2.65 | 2.68 | 2.53 | 2.75 | 2.40 | 3.06 |

Figure 6.1.2.1.3.1-1: PSFCH MPR simulation results for SL-U power class 5

Table 6.1.2.1.3.1-3 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.2.1.3.1-3 : PSFCH MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration5 | Inner RB set configuration5 |
| Contiguous sub-band RB sets | 2.55 | 1.63 |
| Non-contiguous sub-band RB sets | 3.06 | 2.40 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH MPR for SL-U power class 5 can be proposed as Table 6.1.2.1.3.1-4 or Table 6.1.2.1.3.1-5 based on the simulation results when considering implementation margin.

Table 6.1.2.1.3.1-5 PSFCH MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.5 |
| Non-contiguous sub-band RB sets | ≤ 5.0 | ≤ 4.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |

Table 6.1.2.1.3.1-6 PSFCH MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 5.0 | ≤ 4.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |

6.1.3	UE additional maximum output power reduction for NR SL-U

Based on the NR-U agreement for the additional spectrum emission mask in clause 6.5F.2.3 in TS38.101-1, RAN4 do not consider the additional spectrum emission mask.

For the additional spurious emission requirements for SL-U, RAN4 will consider the NR-U additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1.

To derive A-MPR requirements for SL-U operation in single CC, RAN4 will assume the basic simulation parameters and assumptions in section 6.1.2 and the additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1 will be considered.

6.1.3.1	A-MPR for SL-U with NS_28

6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.1.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.1.1.1-1: Simulation results for NS_28 Allocation Full and partial allocations.

| NS_28 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤8.0 | ≤8.1 | ≤7.7 | ≤8.0 | ≤7.1 | ≤7.8 | ≤6.9 | ≤7.8 |
| 16 QAM | ≤8.0 | ≤8.3 | ≤7.4 | ≤8.3 | ≤7.3 | ≤8.1 | ≤6.9 | ≤8.1 |
| 64 QAM | ≤8.1 | ≤8.7 | ≤7.4 | ≤8.7 | ≤7.3 | ≤8.2 | ≤7.0 | ≤8.2 |
| 256 QAM | ≤8.2 | ≤8.8 | ≤7.4 | ≤8.8 | ≤7.3 | ≤8.2 | ≤7.1 | ≤8.1 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

6.1.3.1.2	A-MPR for S-SSB transmission

6.1.3.1.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.2	A-MPR for SL-U with NS_29

6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.2.2	A-MPR for S-SSB transmission

6.1.3.2.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.3	A-MPR for SL-U with NS_30

6.1.3.3.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.3.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.3.1.1-1: Simulation results for NS_30 Allocation Note 2 Full and partial allocations.

| NS_30 A-MPR Allocation Note2 | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤10.7 | ≤10.9 | ≤10.4 | ≤11.0 | ≤9.3 | ≤10.3 | ≤9.1 | ≤10.4 |
| 16 QAM | ≤10.8 | ≤9.0 | ≤10.1 | ≤11.1 | ≤9.6 | ≤10.5 | ≤9.2 | ≤10.5 |
| 64 QAM | ≤10.9 | ≤11.4 | ≤10.1 | ≤11.4 | ≤9.6 | ≤10.6 | ≤9.3 | ≤10.6 |
| 256 QAM | ≤11.0 | ≤11.5 | ≤10.0 | ≤11.5 | ≤9.7 | ≤10.6 | ≤9.4 | ≤10.5 |

Table 6.1.3.3.1.1-2: Simulation results for NS_30 Allocation Note 3 Full and partial allocations.

| NS_30 A-MPR Allocation Note3 | 20M | 20M | 40M | 40M |
| --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial |
| QPSK | ≤8.3 | ≤8.4 | ≤5.9 | ≤6.2 |
| 16 QAM | ≤8.1 | ≤8.7 | ≤5.8 | ≤6.3 |
| 64 QAM | ≤8.3 | ≤9.0 | ≤5.8 | ≤6.3 |
| 256 QAM | ≤8.4 | ≤8.9 | ≤5.8 | ≤6.3 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

6.1.3.3.2	A-MPR for S-SSB transmission

6.1.3.3.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.4	A-MPR for SL-U with NS_31

6.1.3.4.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.4.1.1	LG Electronics’ simulation results (R4-2315542)

Table 6.1.3.4.1.1-6 and Figure 6.1.3.4.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.4.1.1-1: NS_31-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 4.67 | 5.08 | 4.67 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 4.67 | 5.08 | 4.67 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 4.67 | 5.07 | 4.67 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.89 | 5.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 4.28 | 4.67 | 4.66 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 4.28 | 4.67 | 4.67 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.67 | 4.67 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.28 | 4.67 | 5.89 | 4.28 | 4.67 | 3.16 | 5.88 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 4.28 | 4.67 | 5.89 | 4.28 | 4.67 | 3.16 | 5.88 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.29 | 4.66 | 5.89 | 4.27 | 4.67 | 3.15 | 5.88 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.88 | 5.07 | 5.47 | 5.89 | 5.88 | 5.07 | 5.88 | 5.87 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 3.53 | 4.28 | 4.67 | 5.89 | 4.28 | 4.67 | 3.89 | 4.28 | 3.16 | 5.88 | 1.81 | 2.81 |  |  |  |  |  |  |
|  | '16QAM' | 3.53 | 4.28 | 4.67 | 5.90 | 4.28 | 4.67 | 3.90 | 4.28 | 3.16 | 5.87 | 1.81 | 2.80 |  |  |  |  |  |  |
|  | '64QAM' | 3.53 | 4.28 | 4.67 | 5.89 | 4.28 | 4.67 | 3.90 | 4.28 | 3.16 | 5.88 | 3.16 | 2.80 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.90 | 5.47 | 5.07 | 5.47 | 5.07 | 5.46 | 5.87 | 5.88 | 5.06 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 3.17 | 3.90 | 4.66 | 5.89 | 4.27 | 4.67 | 3.89 | 4.28 | 3.53 | 4.28 | 3.16 | 5.88 | 1.81 | 2.80 | 2.47 | 3.16 | 3.16 | 5.88 |
|  | '16QAM' | 3.17 | 3.90 | 4.66 | 5.89 | 4.28 | 4.67 | 3.90 | 4.28 | 3.52 | 4.28 | 3.15 | 5.88 | 1.81 | 2.80 | 2.47 | 3.16 | 3.51 | 5.88 |
|  | '64QAM' | 3.17 | 3.90 | 4.67 | 5.89 | 4.27 | 4.67 | 3.89 | 4.28 | 3.53 | 4.28 | 3.52 | 5.88 | 3.15 | 2.80 | 3.16 | 3.16 | 3.52 | 5.88 |
|  | '256QAM' | 5.88 | 5.06 | 5.47 | 5.88 | 5.47 | 5.07 | 5.88 | 5.06 | 5.88 | 5.06 | 5.46 | 5.88 | 5.88 | 5.06 | 5.88 | 5.06 | 5.88 | 5.88 |

Figure 6.1.3.4.1.1-1: NS_31-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.4.1.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.4.1.1-2 : NS_31-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full(dB) | Partial (dB) |
| CP-OFDM | QPSK | 4.67 | 6.32 | 3.52 | 6.30 |
|  | 16 QAM | 4.68 | 6.32 | 3.52 | 6.30 |
|  | 64 QAM | 4.67 | 6.32 | 3.52 | 6.30 |
|  | 256 QAM | 5.48 | 6.32 | 5.88 | 6.30 |

The inner RB set configuration needs to considered when defining the SL-U A-MPR requirements.

Based on the results, Table 6.1.3.4.1.1-3 can be considered for NS_31 A-MPR for SL-U UE power class 5.

Table 6.1.3.4.1.1-3. NS_31 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 4.5 | ≤ 6.5 | Follow SL-U MPR table |
|  | 16 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1.1-5 apply. |  |  |  |  |  |  |

6.1.3.4.1.2	OPPO’s simulation results (R4-2316119)

For PSD = 10dBm/MHz：

Table 6.1.3.4.1.2-1 A-MPR for single CC PSD=10 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.4.1.2-2 A-MPR for Wideband operation PSD=10 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 2.8 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

For PSD = 7dBm/MHz：

Table 6.1.3.4.1.2-3 A-MPR for single CC PSD=7 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.4.1.2-4 A-MPR for Wideband operation PSD=7 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 3.0 | 2.7 | 2.9 | 2.6 | 2.9 | 2.6 | 2.9 | 2.7 | 2.7 | 2.7 | 2.9 | 2.7 | 2.8 | 3.0 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 2.8 | 3.0 | 3.0 | 3.0 | 3.0 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

For PSD = 4dBm/MHz：

Table 6.1.3.4.1.2-5 A-MPR for single CC PSD=4 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 5.6 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 5.7 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 5.7 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.4.1.2-6 A-MPR for NS_02S Wideband operation PSD=4 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.5 | 3.0 | 3.6 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 2.8 | 2.8 | 3.5 | 2.8 | 2.8 | 3.6 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.6 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 5.9 | 5.9 | 3.1 | 6.0 | 2.9 | 5.9 | 2.6 | 5.9 | 2.9 | 5.9 | 2.9 | 2.7 | 2.7 | 5.9 | 3.0 | 2.8 | 6.0 |
|  | 16QAM | 6.0 | 6.0 | 3.1 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 | 5.9 | 3.0 | 3.0 | 2.8 | 6.0 | 3.0 | 3.0 | 6.0 |
|  | 64QAM | 6.0 | 6.0 | 4.2 | 6.0 | 4.2 | 5.9 | 4.1 | 5.9 | 4.2 | 5.9 | 4.2 | 4.1 | 4.0 | 6.0 | 4.2 | 4.1 | 5.9 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

From the table 6.1.3.4.1.2-1 to table 6.1.3.4.1.2-6 above, it can be found that even though the PSD requirement is different from 10 to 7 to 4 dBm/MHz, the A-MPR simulation result for both single CC and wide-band operation are the same. The reason here is that the PSD requirement is not the dominate factor and it is the SEM and EVM for some of the 256QAM. The figure 6.1.3.4.1.2-1 and figure 6.1.3.4.1.2-2 below show the A-MPR.

Figure 6.1.3.4.1.2-1 Single CC A-MPR

Figure 6.1.3.4.1.2-2 Wideband A-MPR

Note : NS_02S is equal to NS_31.

6.1.3.4.1.3	Qualcomm’s simulation results (R4-2316791)

We performed simulations for the full and partial allocations and computed the A-MPR for NS_31. The results are shown in the tables below.

Table 6.1.3.4.1.3: Simulation results for NS_31 Full and partial allocations.

| NS_31 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤ 5.2 | ≤ 5.7 | ≤ 8.3 | ≤ 8.6 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.1 |
| 16 QAM | ≤ 5.6 | ≤ 5.8 | ≤ 8.0 | ≤ 9.0 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.3 |
| 64 QAM | ≤ 5.5 | ≤ 6.0 | ≤ 8.0 | ≤ 9.3 | ≤ 8.1 | NA | ≤ 8.1 | ≤ 8.5 |
| 256 QAM | ≤ 5.6 | ≤ 5.9 | ≤ 8.0 | ≤ 9.5 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.5 |

The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

6.1.3.4.2	A-MPR for S-SSB transmission

6.1.3.4.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_31, Table 6.1.3.4.2.1-1 and Figure 6.1.3.4.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.4.2.1-1: NS_31-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 7.90 | 6.82 | 6.81 | 6.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 7.45 | 9.20 | 8.17 | 7.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 6.46 | 6.69 | 7.21 | 4.76 | 7.20 | 8.18 | 6.79 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.41 | 8.95 | 6.78 | 7.97 | 6.48 | 6.89 | 7.19 | 5.98 | 4.31 | 4.61 | 7.12 | 9.04 | 6.62 | 7.85 | 6.61 | 7.79 | 6.47 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.91 | 7.19 | 9.59 | 6.61 | 9.54 | 6.67 | 7.99 | 6.53 | 6.80 | 7.20 | 6.12 | 4.95 | 5.66 | 4.09 | 4.70 | 7.28 | 4.56 | 7.16 | 9.97 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 8.85 | 6.62 | 9.14 | 6.54 | 8.62 | 6.08 | 8.80 | 5.83 | 8.04 | 6.48 | 7.86 | 6.46 | 8.13 | 6.49 | 7.49 | 6.35 | 10.15 | 6.31 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.4.2.1-13: NS_31-S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.4.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.4.2.1-2 : NS_31 S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | 11.05 | 7.58 | 6.12 | 7.28 |
| Non-contiguous sub-band RB sets | 10.15 | 7.47 | 7.49 | 6.35 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.4.2.1-3 or Table 6.1.3.4.2.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.4.2.1-3: NS_31 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 | ≤ 10.0 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

Table 6.1.3.4.2.1-4: NS_31 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

6.1.3.4.3	A-MPR for PSFCH transmission

6.1.3.4.3.1	LG Electronics’ simulation results (R4-2315542)

For NS_31, Table 6.1.3.4.3.1-1 and Figure 6.1.3.4.3.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.4.3.1-1: NS_31-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 2.97 | 3.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 3.40 | 3.03 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 3.89 | 3.62 | 5.63 | 5.59 | 3.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 3.54 | 3.62 | 3.58 | 5.64 | 2.67 | 5.64 | 3.27 | 3.19 | 3.18 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 2.95 | 3.63 | 3.88 | 3.62 | 5.65 | 2.31 | 2.60 | 5.62 | 5.51 | 3.04 | 3.23 | 3.22 | 3.40 | 3.48 | 3.12 | 3.14 | 3.13 | 3.19 | 3.55 |

Figure 6.1.3.4.3.1-1: NS_31-PSFCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.4.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.4.3.1-2 : NS_31 PSFCH A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous sub-band RB sets | 5.65 | 5.64 |
| Non-contiguous sub-band RB sets | 3.55 | 3.19 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.4.3.1-3 or Table 6.1.3.4.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.4.3.1-3 NS_31 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 7.5 | ≤ 7.5 |
| Non-contiguous sub-band RB sets | ≤ 5.5 | ≤ 5.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

Table 6.1.3.4.3.1-4 NS_31 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

6.1.3.5	A-MPR for SL-U with NS_53

6.1.3.5.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.5.1.1	LG Electronics’ simulation results (R4-2315542)

For NS_53, Table 6.1.3.5.1.1-9 and Figure 6.1.3.5.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.5.1.1-1: NS_53-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 8.50 | 10.84 | 8.51 | 10.85 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 8.50 | 10.84 | 8.51 | 10.84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 8.50 | 10.84 | 8.51 | 10.84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 8.50 | 10.83 | 8.51 | 10.84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.47 | 7.61 | 8.50 | 10.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.47 | 8.06 | 8.50 | 10.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.47 | 8.06 | 8.50 | 10.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 8.05 | 8.50 | 10.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 5.89 | 8.51 | 11.31 | 5.47 | 8.06 | 8.50 | 11.29 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 5.89 | 8.51 | 11.32 | 5.47 | 8.06 | 8.50 | 11.30 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 6.31 | 8.96 | 11.30 | 5.47 | 8.06 | 8.50 | 11.29 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 6.30 | 8.96 | 11.31 | 5.47 | 8.06 | 8.50 | 11.29 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.82 | 4.67 | 8.50 | 11.30 | 5.47 | 8.06 | 3.53 | 6.31 | 8.50 | 11.29 | 5.47 | 8.05 |  |  |  |  |  |  |
|  | '16QAM' | 2.82 | 4.67 | 8.50 | 11.31 | 5.47 | 8.05 | 3.53 | 6.31 | 8.50 | 11.29 | 5.47 | 8.05 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 4.67 | 8.51 | 11.30 | 5.47 | 8.06 | 3.53 | 6.30 | 8.96 | 11.29 | 5.47 | 8.05 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 8.96 | 11.31 | 5.47 | 8.06 | 5.47 | 6.31 | 8.50 | 11.29 | 5.47 | 8.05 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 3.17 | 3.90 | 8.50 | 11.30 | 5.47 | 8.06 | 3.52 | 6.30 | 2.47 | 4.67 | 8.50 | 11.30 | 5.47 | 8.05 | 3.52 | 5.88 | 8.50 | 11.30 |
|  | '16QAM' | 3.17 | 3.90 | 8.50 | 11.32 | 5.47 | 8.06 | 3.52 | 6.31 | 2.47 | 5.06 | 8.50 | 11.29 | 5.47 | 8.05 | 3.52 | 6.30 | 8.96 | 11.30 |
|  | '64QAM' | 3.17 | 3.90 | 8.96 | 11.31 | 5.47 | 8.06 | 3.53 | 6.31 | 2.81 | 4.67 | 8.50 | 11.29 | 5.47 | 8.05 | 3.52 | 5.88 | 8.96 | 11.29 |
|  | '256QAM' | 5.47 | 5.07 | 8.51 | 11.32 | 5.47 | 8.06 | 5.47 | 6.30 | 5.47 | 5.06 | 8.50 | 11.30 | 5.46 | 8.05 | 5.47 | 6.30 | 8.96 | 11.29 |

Figure 6.1.3.5.1.1-1: NS_53-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.5.1.1-2 shows the maximum value of simulation results considering Full/Partial RB allocation.

Table 6.1.3.5.1.1-2 : NS_53-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 8.51 | 11.31 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 4.67 | 3.17 | 3.90 |
|  | 16 QAM | 8.96 | 11.32 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 5.06 | 3.17 | 3.90 |
|  | 64 QAM | 8.96 | 11.31 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 4.67 | 3.17 | 3.90 |
|  | 256 QAM | 8.96 | 11.32 | 5.47 | 8.06 | 5.47 | 6.31 | 5.47 | 5.07 | 5.47 | 5.07 |

Table 6.1.3.5.1.1-3 can be considered for NS_53 A-MPR for SL-U UE power class 5.

Table 6.1.3.5.1.1-3. NS_53 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

6.1.3.5.1.2	OPPO’s simulation results (R4-2316119)

Case 1,2,8,9 are for 20MHz; 3,4,10,11 are for 40MHz; 5,12 are for 60MHz; 6,13 are for 80MHz and 7,14 are for 100MHz.

Table 6.1.3.5.1.2-1 A-MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 8.4 | 8.6 | 5.4 | 5.5 | 3.8 | 3.2 | 3.2 | 9.8 | 10.9 | 6.6 | 7.8 | 6.0 | 4.5 | 3.9 |
| 16QAM | 8.4 | 8.6 | 5.4 | 5.6 | 3.9 | 3.9 | 3.8 | 9.8 | 10.9 | 6.6 | 7.8 | 6.0 | 4.5 | 3.9 |
| 64QAM | 8.3 | 8.5 | 5.5 | 5.4 | 5.6 | 5.5 | 5.5 | 9.8 | 10.9 | 6.7 | 7.8 | 6.1 | 4.6 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 9.8 | 10.9 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.5.1.2-2 A-MPR for Wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.6 | 5.8 | 5.5 | 5.5 | 5.5 | 5.8 | 4.0 | 4.0 | 4.0 | 2.8 |
|  | 16QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.6 | 5.8 | 5.6 | 5.6 | 5.6 | 5.7 | 4.0 | 4.0 | 4.0 | 3.6 |
|  | 64QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 5.8 | 5.5 | 5.5 | 5.5 | 5.7 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.6 | 8.6 | 8.5 | 8.6 | 7.7 | 7.3 | 7.3 | 7.3 | 8.5 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 10.9 | 10.9 | 11.0 | 10.9 | 10.9 | 10.9 | 10.9 | 11.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.2 | 6.4 | 6.4 | 5.1 |
|  | 16QAM | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.3 | 6.5 | 6.3 | 5.1 |
|  | 64QAM | 11.0 | 11.0 | 11.0 | 10.9 | 11.0 | 10.9 | 11.0 | 10.9 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.3 | 6.4 | 6.3 | 5.2 |
|  | 256QAM | 11.0 | 11.0 | 11.0 | 10.9 | 11.0 | 10.9 | 11.0 | 11.0 | 8.1 | 8.0 | 8.0 | 8.0 | 8.0 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3.5.1.2-1 Single CC A-MPR

Figure 6.1.3.5.1.2-2 Wide-band A-MPR

Note : NS_03S is equal to NS_53.

6.1.3.5.1.3	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.5.1.3-1: Simulation results for NS_53 Full and partial allocations.

| NS_53 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤7.5 | ≤7.6 | ≤7.0 | ≤7.3 | ≤6.7 | ≤7.4 | ≤6.4 | ≤7.4 |
| 16 QAM | ≤7.5 | ≤7.8 | ≤6.9 | ≤7.4 | ≤6.8 | ≤7.6 | ≤6.4 | ≤7.6 |
| 64 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.6 | ≤6.8 | ≤7.8 | ≤6.5 | ≤7.8 |
| 256 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.5 | ≤6.8 | ≤7.7 | ≤6.6 | ≤7.7 |

The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

6.1.3.5.2	A-MPR for S-SSB transmission

6.1.3.5.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_53, Table 6.1.3.5.2.1-1 and Figure 6.1.3.5.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.5.2.1-1: NS_53-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 11.16 | 15.09 | 9.13 | 12.21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 12.11 | 9.20 | 11.24 | 15.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 9.19 | 9.12 | 12.22 | 9.09 | 12.22 | 8.18 | 9.12 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.32 | 8.95 | 7.43 | 7.99 | 9.19 | 9.19 | 12.23 | 6.36 | 9.21 | 9.22 | 12.13 | 9.03 | 7.47 | 8.09 | 9.10 | 8.31 | 9.20 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 11.13 | 7.20 | 9.76 | 6.37 | 9.55 | 7.44 | 8.15 | 9.21 | 9.24 | 12.28 | 6.04 | 7.38 | 6.28 | 9.18 | 9.22 | 12.19 | 9.13 | 12.22 | 10.26 | 7.48 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.47 | 7.36 | 8.74 | 7.40 | 8.46 | 7.45 | 8.90 | 7.44 | 8.10 | 9.19 | 8.09 | 9.14 | 8.08 | 9.29 | 7.62 | 9.18 | 10.07 | 6.45 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.5.2.1-1: NS_53 S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.5.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.5.2.1-2 : NS_53 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 11.16 | 15.09 | 11.05 | 15.09 | 9.36 | 12.22 | 9.92 | 12.23 | 11.13 | 11.13 |
| Non-contiguous | - | - | - | - | 8.18 | 9.12 | 9.03 | 9.20 | 10.26 | 9.29 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.5.2.1-3 or Table 6.1.3.5.2.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.5.2.1-3: NS_53 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.5.2.1-4: NS_53 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

6.1.3.5.3	A-MPR for PSFCH transmission

6.1.3.5.3.1	LG Electronics’ simulation results (R4-2315542)

For NS_53, Table 6.1.3.5.3.1-1 and Figure 6.1.3.5.3.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.5.3.1-1: NS_53-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 10.33 | 10.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 7.29 | 10.32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 5.88 | 7.63 | 10.73 | 10.72 | 7.64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 4.66 | 5.88 | 7.63 | 10.65 | 7.63 | 10.63 | 5.88 | 7.63 | 7.64 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 3.70 | 4.59 | 5.87 | 7.63 | 10.73 | 5.85 | 7.60 | 10.69 | 10.54 | 4.60 | 5.85 | 5.86 | 5.85 | 5.85 | 7.63 | 7.63 | 7.64 | 7.60 | 4.64 |

Figure 6.1.3.5.3.1-1: NS_53-PSFCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.5.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.5.3.1-2 : NS_53 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 10.74 | 7.63 | 5.88 | 4.66 | 3.70 |
| Non-contiguous | N/A | 7.74 | 5.88 | 4.65 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.5.3.1-3 or Table 6.1.3.5.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.5.3.1-3 NS_53 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤12.5 | ≤9.5 | ≤8.0 | ≤6.5 | ≤5.5 |
| Non-contiguous | N/A | ≤9.5 | ≤8.0 | ≤6.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 6.1.3.5.3.1-4 NS_53 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤12.5 | ≤9.5 | ≤8.0 | ≤6.5 | ≤5.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

6.1.3.6	A-MPR for SL-U with NS_54

6.1.3.6.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.6.2	A-MPR for S-SSB transmission

6.1.3.6.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.7	A-MPR for SL-U with NS_58

6.1.3.7.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.7.1.1	LG Electronics’ simulation results (R4-2315542)

For NS_58, Table 6.1.3.7.1.1-1 and Figure 6.1.3.7.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.7.1.1-1: NS_58-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 2.48 | 2.83 | 2.15 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.83 | 2.15 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.83 | 3.18 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.48 | 5.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.82 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.82 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.82 | 2.81 | 2.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.48 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.47 | 2.47 | 2.47 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.16 | 2.48 | 3.16 | 2.48 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.07 | 5.88 | 5.06 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.15 | 2.47 | 2.48 | 2.47 | 2.48 | 2.46 | 0.10 | 2.13 | 0.00 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.15 | 2.47 | 2.47 | 2.47 | 2.48 | 2.46 | 0.36 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.16 | 2.15 | 2.81 | 2.48 | 3.16 | 2.48 | 3.16 | 2.12 | 3.16 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.07 | 5.46 | 5.06 | 5.47 | 5.06 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.48 | 2.47 | 2.48 | 2.47 | 2.47 | 2.13 | 0.10 | 2.46 | 0.00 | 2.47 | 1.20 | 2.13 | 0.10 |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 2.48 | 2.47 | 2.48 | 2.47 | 2.47 | 2.47 | 2.48 | 2.13 | 0.36 | 2.46 | 0.63 | 2.47 | 1.20 | 2.13 | 0.36 |
|  | '64QAM' | 2.82 | 2.48 | 3.17 | 2.48 | 3.16 | 2.47 | 3.16 | 2.47 | 2.81 | 2.48 | 3.15 | 2.12 | 3.16 | 2.46 | 3.16 | 2.47 | 3.16 | 2.12 |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.06 | 5.46 | 5.06 | 5.46 | 5.06 | 5.47 | 5.06 | 5.46 | 5.05 |

Figure 6.1.3.7.1.1-2: NS_58-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.7.1.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.7.1.1-2 : NS_58-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full(dB) | Partial (dB) |
| CP-OFDM | QPSK | 2.81 | 2.83 | 2.47 | 1.20 |
|  | 16 QAM | 2.81 | 2.83 | 2.47 | 1.20 |
|  | 64 QAM | 3.18 | 2.83 | 3.16 | 2.47 |
|  | 256 QAM | 5.48 | 5.09 | 5.88 | 5.06 |

The inner RB set configuration needs to considered when defining the SL-U A-MPR requirements.

Based on the results, Table 6.1.3.7.1.1-3 can be considered for NS_58 A-MPR for SL-U UE power class 5.

Table 6.1.3.7.1.1-3. NS_58 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |
|  |  | Full (dB)2 | Partial (dB)3 | Full (dB) 2 | Partial (dB) 3 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1.1-5 apply. |  |  |  |  |  |

6.1.3.7.1.2	OPPO’s simulation results (R4-2316119)

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.7.1.2-1 A-MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 5.4 | 5.6 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 6.7 | 8.0 | 3.5 | 4.7 | 3.2 | 3.3 | 3.2 |
| 16QAM | 5.4 | 5.7 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 6.8 | 8.0 | 3.5 | 4.8 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.3 | 5.6 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 6.8 | 8.0 | 4.1 | 4.7 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 8.0 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.7.1.2-2 A-MPR for wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 3.6 | 3.6 | 3.7 | 3.6 | 3.6 | 3.6 | 3.5 | 3.7 | 3.6 |
|  | 64QAM | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 4.8 | 4.7 | 4.9 | 4.7 | 4.8 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 7.4 | 7.5 | 8.5 | 7.4 | 7.5 | 7.4 | 7.4 | 8.5 | 7.7 | 7.3 | 8.5 | 7.3 | 7.7 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 | 3.1 | 3.3 | 3.4 | 2.7 |
|  | 16QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.1 | 5.0 | 5.0 | 5.0 | 5.1 | 3.2 | 3.5 | 3.3 | 2.8 |
|  | 64QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.1 | 5.1 | 5.1 | 5.1 | 5.1 | 4.1 | 4.1 | 4.1 | 4.0 |
|  | 256QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3.7.1.2-1 Single CC A-MPR

Figure 6.1.3.7.1.2-2 Wideband CC A-MPR

Note : NS_04S is equal to NS_58.

6.1.3.7.1.3	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.7.1.3-1: Simulation results for NS_58 Full and partial allocations.

| NS_58 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤5.2 | ≤5.4 | ≤5.9 | ≤6.0 | ≤5.6 | ≤5.8 | ≤5.4 | ≤5.7 |
| 16 QAM | ≤5.3 | ≤5.6 | ≤5.8 | ≤6.2 | ≤5.7 | ≤5.9 | ≤5.6 | ≤5.8 |
| 64 QAM | ≤5.3 | ≤5.8 | ≤5.8 | ≤6.2 | ≤5.6 | ≤5.9 | ≤5.6 | ≤5.8 |
| 256 QAM | ≤5.4 | ≤5.8 | ≤5.8 | ≤6.2 | ≤5.7 | ≤5.9 | ≤5.6 | ≤5.8 |

The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

6.1.3.7.2	A-MPR for S-SSB transmission

6.1.3.7.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_58, Table 6.1.3.7.2.1-1 and Figure 6.1.3.7.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.7.2.1-1: NS_58-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 7.90 | 6.82 | 6.81 | 6.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 7.45 | 9.20 | 8.17 | 7.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 6.46 | 6.69 | 7.21 | 4.76 | 7.20 | 8.18 | 6.79 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.41 | 8.95 | 6.78 | 7.97 | 6.48 | 6.89 | 7.19 | 5.98 | 4.31 | 4.61 | 7.12 | 9.04 | 6.62 | 7.85 | 6.61 | 7.79 | 6.47 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.91 | 7.19 | 9.59 | 6.61 | 9.54 | 6.67 | 7.99 | 6.53 | 6.80 | 7.20 | 6.12 | 4.95 | 5.66 | 4.09 | 4.70 | 7.28 | 4.56 | 7.16 | 9.97 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 8.85 | 6.62 | 9.14 | 6.54 | 8.62 | 6.08 | 8.80 | 5.83 | 8.04 | 6.48 | 7.86 | 6.46 | 8.13 | 6.49 | 7.49 | 6.35 | 10.15 | 6.31 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.7.2.1-1: NS_58-S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.7.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.7.2.1-2 : NS_58 S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | 10.91 | 6.86 | 6.11 | 3.98 |
| Non-contiguous sub-band RB sets | 9.94 | 7.22 | 6.93 | 4.82 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.7.2.1-3 or Table 6.1.3.7.2.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.7.2.1-3: NS_58 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 8.5 | ≤ 6.5 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

Table 6.1.3.7.2.1-4: NS_58 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

6.1.3.7.3	A-MPR for PSFCH transmission

6.1.3.7.3.1	LG Electronics’ simulation results (R4-2315542)

For NS_58, Table 6.1.3.7.3.1-1 and Figure 6.1.3.7.3.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.7.3.1-1: NS_58-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 1.73 | 1.16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 2.12 | 1.88 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 2.59 | 2.03 | 1.14 | 0.00 | 1.99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 2.24 | 2.12 | 2.01 | 1.09 | 0.36 | 0.00 | 2.41 | 2.04 | 2.05 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 1.85 | 2.31 | 2.58 | 2.01 | 1.10 | 0.63 | 0.47 | 0.00 | 0.00 | 2.37 | 2.42 | 2.49 | 2.31 | 2.07 | 1.99 | 1.98 | 1.98 | 1.81 | 2.60 |

Figure 6.1.3.7.3.1-1: NS_58-PSFCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.7.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.7.3.1-2 : NS_58 PSFCH A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous sub-band RB sets | 2.59 | 0.63 |
| Non-contiguous sub-band RB sets | 2.60 | 1.81 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.7.3.1-3 or Table 6.1.3.7.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.7.3.1-3 NS_58 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 4.5 | ≤ 2.5 |
| Non-contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

Table 6.1.3.7.3.1-4 NS_58 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

6.1.3.8	A-MPR for SL-U with NS_59

6.1.3.8.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.8.2	A-MPR for S-SSB transmission

6.1.3.8.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.9	A-MPR for SL-U with NS_60

6.1.3.9.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.9.1.1	LG Electronics’ simulation results (R4-2315542)

For NS_60, Table 6.1.3.9.1.1-1 and Figure 6.1.3.9.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.9.1.1-1: NS_60-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 5.47 | 7.62 | 5.89 | 7.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.47 | 8.07 | 5.89 | 8.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.47 | 8.06 | 5.89 | 7.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 8.06 | 5.89 | 8.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 4.67 | 5.47 | 7.62 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 4.67 | 5.47 | 8.06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 4.68 | 5.47 | 8.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 8.05 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.28 | 5.88 | 8.07 | 2.47 | 5.07 | 5.88 | 8.05 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 4.28 | 5.88 | 8.07 | 2.47 | 5.07 | 5.88 | 8.05 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.28 | 5.88 | 8.52 | 2.82 | 5.06 | 5.88 | 8.05 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.88 | 5.48 | 5.89 | 8.07 | 5.47 | 5.07 | 5.88 | 8.04 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.82 | 3.53 | 5.88 | 8.06 | 2.47 | 5.07 | 2.81 | 3.53 | 5.88 | 8.05 | 2.46 | 5.06 |  |  |  |  |  |  |
|  | '16QAM' | 2.82 | 3.53 | 5.88 | 8.06 | 2.47 | 5.07 | 2.81 | 3.53 | 5.88 | 8.49 | 2.46 | 5.06 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 3.53 | 5.89 | 8.06 | 2.81 | 5.07 | 3.16 | 3.53 | 5.88 | 8.05 | 3.16 | 5.06 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.88 | 8.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 8.51 | 5.46 | 5.06 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 3.17 | 3.90 | 5.89 | 8.07 | 2.47 | 5.07 | 2.47 | 3.16 | 2.47 | 3.17 | 5.88 | 8.04 | 2.46 | 5.05 | 2.47 | 3.16 | 5.88 | 8.05 |
|  | '16QAM' | 3.17 | 3.90 | 5.88 | 8.51 | 2.47 | 5.07 | 2.47 | 3.17 | 2.47 | 3.17 | 5.88 | 8.49 | 2.46 | 5.05 | 2.46 | 3.16 | 5.88 | 8.05 |
|  | '64QAM' | 3.17 | 3.90 | 5.88 | 8.51 | 2.81 | 5.06 | 3.16 | 3.16 | 2.81 | 3.17 | 5.47 | 8.05 | 3.16 | 5.05 | 3.16 | 3.16 | 5.88 | 8.05 |
|  | '256QAM' | 5.47 | 5.06 | 5.89 | 8.52 | 5.47 | 5.07 | 5.47 | 5.06 | 5.47 | 5.06 | 5.88 | 8.05 | 5.47 | 5.06 | 5.47 | 5.46 | 5.88 | 8.05 |

Figure 6.1.3.9.1.1-1: NS_60-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.9.1.1-2 shows the maximum value of simulation results considering Full/Partial RB allocation.

Table 6.1.3.9.1.1-2 : NS_60-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 5.89 | 8.07 | 2.48 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 16 QAM | 5.89 | 8.51 | 2.48 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 64 QAM | 5.89 | 8.52 | 3.16 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 256 QAM | 5.89 | 8.52 | 5.47 | 5.07 | 5.88 | 5.48 | 5.47 | 5.07 | 5.47 | 5.06 |

Based on the results, Table 6.1.3.9.1.1-3 can be considered for NS_60 A-MPR for SL-U UE power class 5.

Table 6.1.3.9.1.1-3. NS_60 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

6.1.3.9.1.2	OPPO’s simulation results (R4-2316119)

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.9.1.1-1 A-MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 6.4 | 6.6 | 3.4 | 3.5 | 3.5 | 3.5 | 3.5 | 7.8 | 8.9 | 4.5 | 5.8 | 4.0 | 3.6 | 3.5 |
| 16QAM | 6.4 | 6.7 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 7.8 | 9.0 | 4.5 | 5.8 | 4.0 | 3.6 | 3.5 |
| 64QAM | 6.3 | 6.6 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 7.8 | 9.0 | 4.6 | 5.8 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 9.0 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.9.1.1-2 A-MPR for wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 6.5 | 6.5 | 6.6 | 6.5 | 6.5 | 6.5 | 6.5 | 6.6 | 3.7 | 3.5 | 3.7 | 3.5 | 3.7 | 3.0 | 2.8 | 2.8 | 3.2 |
|  | 16QAM | 6.5 | 6.5 | 6.6 | 6.5 | 6.6 | 6.5 | 6.5 | 6.7 | 3.7 | 3.6 | 3.7 | 3.6 | 3.7 | 3.6 | 3.5 | 3.7 | 3.6 |
|  | 64QAM | 6.6 | 6.5 | 6.6 | 6.6 | 6.5 | 6.6 | 6.6 | 6.6 | 4.8 | 4.7 | 4.9 | 4.7 | 4.8 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 7.4 | 7.5 | 8.5 | 7.4 | 7.5 | 7.4 | 7.4 | 8.5 | 7.7 | 7.3 | 8.5 | 7.3 | 7.7 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 8.9 | 9.0 | 9.0 | 9.0 | 8.9 | 9.0 | 8.9 | 9.0 | 6.0 | 5.9 | 5.9 | 5.9 | 6.0 | 4.2 | 4.4 | 4.4 | 3.2 |
|  | 16QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.0 | 6.0 | 5.9 | 6.0 | 6.0 | 4.3 | 4.5 | 4.3 | 3.3 |
|  | 64QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.0 | 6.0 | 6.0 | 6.0 | 6.0 | 4.3 | 4.4 | 4.3 | 4.0 |
|  | 256QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 6.1.3.9.1.1-1 Single CC A-MPR

Figure 6.1.3.9.1.1-2 Wideband A-MPR

Note : NS_05S is equal to NS_60.

6.1.3.9.1.3	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.9.1.3-1: Simulation results for NS_60 Full and partial allocations.

| NS_53 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤7.5 | ≤7.6 | ≤7.0 | ≤7.3 | ≤6.7 | ≤7.4 | ≤6.4 | ≤7.4 |
| 16 QAM | ≤7.5 | ≤7.8 | ≤6.9 | ≤7.4 | ≤6.8 | ≤7.6 | ≤6.4 | ≤7.6 |
| 64 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.6 | ≤6.8 | ≤7.8 | ≤6.5 | ≤7.8 |
| 256 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.5 | ≤6.8 | ≤7.7 | ≤6.6 | ≤7.7 |

The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

6.1.3.9.2	A-MPR for S-SSB transmission

6.1.3.9.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_60, Table 6.1.3.9.2.1-1 and Figure 6.1.3.9.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.9.2.1-1: NS_60-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.24 | 8.17 | 12.15 | 6.16 | 9.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.31 | 8.63 | 9.09 | 9.02 | 8.17 | 12.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.29 | 7.05 | 7.87 | 6.15 | 6.20 | 9.21 | 6.20 | 9.21 | 7.67 | 6.25 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 10.34 | 6.86 | 8.81 | 5.70 | 7.67 | 6.08 | 6.19 | 9.19 | 6.11 | 6.15 | 6.23 | 9.09 | 8.92 | 6.11 | 7.97 | 6.13 | 7.51 | 6.23 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.92 | 6.67 | 9.86 | 5.49 | 9.44 | 5.56 | 7.59 | 6.17 | 6.22 | 9.19 | 5.89 | 4.50 | 5.39 | 6.18 | 6.21 | 9.20 | 6.14 | 9.18 | 10.54 | 7.21 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.60 | 6.16 | 8.72 | 6.14 | 8.26 | 5.82 | 9.16 | 5.30 | 7.74 | 6.16 | 7.76 | 6.05 | 7.72 | 6.12 | 7.20 | 6.22 | 9.94 | 6.23 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.9.2.1-1: NS_60 S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.9.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.9.2.1-2 : NS_60 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 9.24 | 12.15 | 11.31 | 12.15 | 9.29 | 9.21 | 10.34 | 9.19 | 10.92 | 10.92 |
| Non-contiguous | - | - | - | - | 7.67 | 6.25 | 8.92 | 6.23 | 10.54 | 7.21 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.9.2.1-3 or Table 6.1.3.9.2.1-4 based on the simulation results when considering implementation margin.

Table 2-66: NS_60 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤10.0 | ≤9.0 | ≤11.5 | ≤9.0 | ≤13.0 | ≤10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 2-67: NS_60 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

6.1.3.9.3	A-MPR for PSFCH transmission

6.1.3.9.3.1	LG Electronics’ simulation results (R4-2315542)

For NS_60, Table 6.1.3.9.3.1-1 and Figure 6.1.3.9.3.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.9.3.1-1: NS_60-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 7.27 | 7.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 4.26 | 7.26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 3.96 | 4.63 | 7.68 | 7.67 | 4.64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 2.79 | 2.93 | 4.64 | 7.63 | 4.63 | 7.64 | 3.27 | 4.64 | 4.65 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 3.15 | 2.66 | 2.89 | 4.63 | 7.68 | 2.88 | 4.60 | 7.61 | 7.53 | 3.11 | 2.88 | 3.40 | 3.22 | 3.02 | 4.63 | 4.63 | 4.64 | 4.61 | 3.53 |

Figure 6.1.3.9.3.1-1: NS_60-PSFCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.9.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.9.3.1-2 : NS_60 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 7.69 | 4.64 | 3.96 | 2.79 | 3.15 |
| Non-contiguous | N/A | 4.65 | 3.40 | 3.53 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.9.3.1-3 or Table 6.1.3.9.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.9.3.1-3 NS_60 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤9.5 | ≤6.5 | ≤6.0 | ≤5.5 | ≤5.5 |
| Non-contiguous | N/A | ≤6.5 | ≤6.0 | ≤5.0 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 6.1.3.9.3.1-4 NS_60 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤9.5 | ≤6.5 | ≤6.0 | ≤5.5 | ≤5.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

6.1.3.10	A-MPR for SL-U with NS_61

6.1.3.10.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.10.1.1	LG Electronics’ simulation results (R4-2315542)

For NS_61, Table 6.1.3.10.1.1-1 and Figure 6.1.3.10.1.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.10.1.1-1: NS_61-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 6.31 | 8.98 | 6.73 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 6.30 | 8.97 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 6.31 | 8.98 | 6.74 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 6.31 | 8.98 | 6.74 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 4.66 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 4.67 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 4.67 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.89 | 6.30 | 8.97 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.06 | 5.47 | 6.73 | 9.44 | 4.67 | 5.89 | 6.73 | 8.95 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.07 | 5.47 | 6.73 | 9.43 | 4.67 | 5.89 | 6.73 | 9.41 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.07 | 5.47 | 6.73 | 9.43 | 4.67 | 5.89 | 6.73 | 9.42 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.47 | 6.73 | 9.43 | 5.47 | 5.88 | 6.73 | 9.42 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 4.67 | 5.47 | 6.74 | 9.44 | 4.67 | 5.89 | 5.06 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '16QAM' | 4.67 | 5.47 | 6.73 | 9.43 | 4.66 | 5.89 | 5.06 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '64QAM' | 4.67 | 5.47 | 6.73 | 9.43 | 4.67 | 5.89 | 4.67 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.47 | 6.73 | 9.43 | 5.47 | 5.89 | 5.47 | 5.47 | 6.73 | 9.42 | 5.47 | 5.88 |  |  |  |  |  |  |
| '100MHz' | Scenario # | #6 | #12 | #22 | #39 | #23 | #40 | #24 | #41 | #25 | #42 | #26 | #43 | #27 | #44 | #28 | #45 | #29 | #46 |
|  | 'QPSK' | 4.67 | 5.47 | 6.73 | 9.43 | 4.67 | 5.89 | 4.66 | 5.47 | 4.66 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 | 2.81 | 3.89 | 6.73 | 8.96 |
|  | '16QAM' | 4.67 | 5.47 | 6.73 | 9.43 | 4.66 | 5.89 | 5.06 | 5.47 | 4.67 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 | 2.81 | 3.89 | 6.73 | 9.41 |
|  | '64QAM' | 4.67 | 5.47 | 6.74 | 9.44 | 4.67 | 6.30 | 4.66 | 5.47 | 4.66 | 5.47 | 6.73 | 9.42 | 3.52 | 5.88 | 3.16 | 3.89 | 6.73 | 9.42 |
|  | '256QAM' | 5.47 | 5.47 | 6.73 | 9.43 | 5.47 | 6.31 | 5.47 | 5.47 | 5.47 | 5.47 | 6.73 | 9.42 | 5.47 | 5.88 | 5.47 | 5.47 | 6.73 | 9.41 |

Figure 6.1.3.10.1.1-1: NS_61-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.10.1.1-2 shows the maximum value of simulation results considering Full/Partial RB allocation.

Table 6.1.3.10.1.1-2 : NS_61-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 6.74 | 9.44 | 4.67 | 5.89 | 5.06 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 16 QAM | 6.74 | 9.43 | 4.67 | 5.89 | 5.07 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 64 QAM | 6.74 | 9.44 | 4.67 | 6.30 | 5.07 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 256 QAM | 6.74 | 9.43 | 5.47 | 6.31 | 5.47 | 5.47 | 5.47 | 5.47 | 5.47 | 5.47 |

For NS_61, the maximum output power is limited with 14dBm. Therefore, basically, A-MPR should be met with the following equation considering power class 5, 20dBm.

- NS_61 A-MPR = max{Table 6.1.3.10.1.1-2 + implementation margin, 6 dB}.

Based on the results, Table 6.1.3.10.1.1-3 can be considered for NS_61 A-MPR for SL-U UE power class 5.

Table 6.1.3.10.1.1-3. NS_61 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

6.1.3.10.1.2	OPPO’s simulation results (R4-2316119)

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.1.3.10.1.1-1 A-MPR for single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 3.2 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.4 | 4.3 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6.1.3.10.1.1-2 A-MPR for wideband

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Figure 6.1.3.10.1.1-1 Single CC A-MPR

Figure 6.1.3.10.1.1-2  Wideband A-MPR

Note : NS_06S is equal to NS_61.

6.1.3.10.2	A-MPR for S-SSB transmission

6.1.3.10.2.1	LG Electronics’ simulation results (R4-2315542)

For NS_61, Table 6.1.3.10.2.1-1 and Figure 6.1.3.10.2.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.10.2.1-1: NS_61-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.24 | 9.14 | 13.08 | 7.15 | 10.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.18 | 9.58 | 10.10 | 9.02 | 9.23 | 13.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.84 | 7.13 | 8.68 | 7.13 | 7.13 | 10.21 | 7.12 | 10.20 | 8.88 | 7.69 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 10.20 | 8.27 | 9.35 | 6.64 | 8.49 | 7.23 | 7.20 | 10.22 | 5.69 | 7.14 | 7.21 | 10.21 | 9.98 | 7.65 | 9.34 | 7.52 | 9.37 | 7.37 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 11.00 | 8.15 | 10.53 | 7.12 | 9.82 | 6.71 | 8.24 | 7.20 | 7.14 | 10.13 | 5.89 | 5.43 | 5.38 | 7.07 | 7.21 | 10.23 | 7.22 | 10.11 | 10.29 | 8.04 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.71 | 7.67 | 9.11 | 7.74 | 9.93 | 7.32 | 9.09 | 6.12 | 9.01 | 7.48 | 8.77 | 7.30 | 9.14 | 7.23 | 8.29 | 7.18 | 10.66 | 7.41 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 6.1.3.10.2.1-1: NS_61 S-SSB A-MPR simulation results for SL-U power class 5

Table 6.1.3.10.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.10.2.1-2 : NS_61 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 9.24 | 13.08 | 11.18 | 13.09 | 9.84 | 10.21 | 10.20 | 10.22 | 11.00 | 11.00 |
| Non-contiguous | - | - | - | - | 8.88 | 7.69 | 9.98 | 7.65 | 10.66 | 8.04 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.10.2.1-3 or Table 6.1.3.10.2.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.10.2.1-3: NS_61 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤11.5 | ≤11.0 | ≤12.5 | ≤11.0 | ≤13.0 | ≤11.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.10.2.1-4: NS_61 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

6.1.3.10.3	A-MPR for PSFCH transmission

6.1.3.10.3.1	LG Electronics’ simulation results (R4-2315542)

For NS_61, Table 6.1.3.10.3.1-1 and Figure 6.1.3.10.3.1-1 show the A-MPR simulation results for the scenarios.

Table 6.1.3.10.3.1-1: NS_61-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 8.30 | 8.73 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 5.26 | 8.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 4.57 | 5.63 | 8.69 | 8.68 | 5.64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 4.39 | 4.30 | 5.63 | 8.64 | 5.63 | 8.65 | 4.67 | 5.63 | 5.64 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 4.19 | 4.47 | 4.53 | 5.63 | 8.69 | 3.87 | 5.59 | 8.65 | 8.53 | 4.39 | 4.64 | 4.60 | 4.74 | 3.87 | 5.63 | 5.63 | 5.63 | 5.60 | 4.57 |

Figure 6.1.3.10.3.1-1: NS_61-PSFCH A-MPR simulation results for SL-U power class 5

Table 6.1.3.10.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 6.1.3.10.3.1-2 : NS_61 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 8.73 | 5.63 | 4.57 | 4.47 | 4.19 |
| Non-contiguous | N/A | 5.64 | 4.74 | 4.57 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 6.1.3.10.3.1-3 or Table 6.1.3.10.3.1-4 based on the simulation results when considering implementation margin.

Table 6.1.3.10.3.1-3 NS_61 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤10.5 | ≤7.5 | ≤6.5 | ≤6.5 | ≤6.0 |
| Non-contiguous | N/A | ≤7.5 | ≤6.5 | ≤6.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 6.1.3.10.3.1-4 NS_61 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤10.5 | ≤7.5 | ≤6.5 | ≤6.5 | ≤6.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

<<<<<<<<<<< End of changes in section 6 >>>>>>>>>>

- References

[1] R4-2315542, On Tx requirements for NR sidelink evolution, LG Electronics

[2] TR 38.786 v1.0.0